| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-014736 |
| Permit Reference Number: | EPR-FB3302MM |
| Permit Holder: | RL Waste Consultancy |
| Site Address: | Fferem Gorniog, LL75 8yl |
| Grid Reference: | Farm location: SH 52166 77604 Fields: 1 PETERSYKE SH5258078070 2 BIG HAY SH5262077790 3 OVER ROAD L SH5242077530 4 BELOW BARN SH5213077250 5 BOTTOM LEFT SH5207077630 6 MIDDLE B LEFT SH5216077780 7 PANTHER SH5223077960 Storage: SH5212077550 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk Environment team: TimAmgylcheddMonacArfon@cyfoethnaturiolcymru.gov.uk NLO: Sean.Wannop@environment-agency.gov.uk |
| Date due: | 03/08/2021 |
| Are fields on Anglesey? If yes, then send application. | Yes: ajxht@anglesey.gov.uk pem@anglesey.gov.uk |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | EA permit, still live |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | Y | Amount: £798 (paid in same transaction as PAN-014735) Date cleared on MyNRW: 15/07/2021 Transaction number: 2091228 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | N | Not required until November 2021 |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | N |  |
| ## Has the permit holder name been provided? | Y | Application states “RL Waste Consultancy”, permit reads “RL Waste Consultancy Limited”. Confirm with applicant. |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| ## Has the address and grid reference of the main farm address been given? | Y | Just farm name and postcode. Grid reference also given. |
| ## Does the farm address match the grid reference? | Y | Map names farm as “Fferam Gorniog” on MyMap for permitting rather than “Fferen Gorniog”, but spelt the same as applicant states on regular MyMap. Confirm spelling with applicant. |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | N | No holding number given. MyMap suggests 53/125/0007 and 53/012/0140, neither of which link to the named landowner or farm name. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 38.9ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | N | No phone number, no email address. Confirm its just one land owner when checking the CPH number. |
| ## Has consent been given to carry out the activity? | Y | Consent give by a Mr R Davies |
| Has all of the previous land treatment table been completed? | n/a | Not previously spread on within last 12 months. |
| ## Which risk band does the activity fall within? | High risk waste, low risk area. Band medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | n/a |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N | Not required until November 2021 and not submitted with this application |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | N |  |
| Has all of the waste types table been completed? | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y | Only 19 02 04 |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Google |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y | 136 tonnes |
| Has checklist been completed? | Y |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y | Small pond in “Petersyke” entitled “Coed Tu’n-ol-I’r-ardd” on applicant’s map – requires a buffer. Small pond just outside south east corner of “Bottom Left” – requires a buffer. Small pond to south east of “Middle B Left” – requires buffer. |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | Y | Spring between “Panther” and “Petersyke” – buffered. Issue between “Big Hay” and “Petersyke” – buffered. Spring and issue together in south east of “Over Road L” and issue at north east side – all buffered. Spring near south east edge of “Below Barn” – buffered. Start of a water course towards mid-south side of “Panther”. Stream buffered but issue is not. Requires a buffer. |
| location of all storage sites for the waste to be spread | Y |  |
| any spring(or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | N | Path through western side of “Below Barn” and along western edge of “Over Road L”. |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | N | Ancient woodland bordering fields “Over Road L” and “Below Barn”. Western corners of fields “Below Barn” and “Bottom Left”, along with northern corners of “Panther” and “Petersyke” noted for threatened bryophytes. Coed Plas Gwyn SINC bordering eastern side of “Petersyke”, “Big Hay” and “Over Road L”. AONB for Anglesey approx 100m from nearest field – no concern. |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields “Below Barn”, majority of “Over Road L” Red and yellow patches throughout Petersyke, Panther, Middle B Left and Bottom Left 12 degrees plus (red)- fields Eastern side of “Over Road L”. south east corner and north west edge of “Big Hay”. Sections of fields marked in yellow are not considered to be at risk from this application as the material being spread is solid and being spread at 3.5t/ha. |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y | 90.9% |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Liming effect (neutralising value of the waste) |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | N |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y | Spreading in line with COGAP |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y | After crop cutting in early summer and late summer |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | N | Liming not mentioned |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | Y | Several fields have Mg indices of 3 or above. No concern. |
| - Included the nutrient level in their environmental risk assessment | Y |  |
| - Stated the source of information that they used to set the nutrient level. | Y |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y | No significant negative impacts as long as following guidance, car to be taken near watercourses, will follow COGAP |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | N | Cannot assess until yield confirmed |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Comments from WFD officer. Fields “Over Road L” and “Below Barn” are within the Ceint water body catchment which is failing to meet “Good Water Body” status. Providing COGAP is followed correctly and buffer zones are adhered to, this should not be a concern. All fields are within a WFD groundwater body “Ynys Mon Secondary”. This should not be a concern as above. “Panther” has a drain and pond in the centre of the field. Proposed buffer in the areas that rain to ditches should be increased. RFI to protect the potential pond/water source in south centre of field. General concerns regarding the spreading of cement dust and that it should not be spread in windy conditions and extra care should be taken around water courses. COGAP will be followed, no concern.

MyMap and location assessment: Several new buffers required, as noted above in the map section.

Protection of habitats and ecosystems Ancient woodlands need to be marked on the map.

Agricultural benefit calculations: Queries regarding yield and total nutrient calculations as vary slightly from ours.

Request for information: Request for Information 1 – 18/08/2021 - Provide full farm address. Provided. - Provide correct spelling of farm name. Fferam Gorniog. - Confirm operator name. RL Waste Consultancy Ltd. “RL Waste Consultancy Limited” will be used on the deployment as this matches the permit. - Confirm permit number due to potential spelling error. FB3209LE. - Fields are under two CPH numbers, neither belong to the farm or the named farmer. Confirm the correct CPH number and farmer for the area. Mr R Davies is the tenant farmer on all the fields. His consent was provided in the initial LPD1 submission. - Provide contact information for the landowner. Provided. - Confirm “Bellow Farm” is a typo on the waste analysis and that it should be “Below Farm”. - Mark and/or buffer: - All paths through fields - Three small ponds - A spring - Ancient woodland - SINC - AONB All buffered and/or marked. AONB acknowledged by operator. - Red and yellow areas of slope need to be justified and/or buffered. Justified as there will be no run-off due to the waste being solid and being spread at a very low application rate. Spreading will be in accordance with landspreading best practice and COGAP. - Estimated yield not provided. 38t/ha/yr. - Grass type needs confirming. Silage at 25% DM. - Include negative impacts of liming in the ABS. Not provided. Later satisfied by the pH being addressed within the benefit statement. Request for Information 2 – 27/08/2021 - Total nutrients not clearly stated and conflict between tables 2 and 3 based on the proposed available nutrients. Total amounts stated in table 2 are ok. - Levels of K different to ours. Agrees with our calculations. - Levels of sulphur different to ours. Applicant has rounded up to worse case scenario – acceptable.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date |  |
| ## Effective date |  |
| ## Permitting Officer Date | Rhiannon Smith 03/09/21 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 03/09/2021 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 30 |
| Determination | 645 |
| Sign off | 5 |
| Issuing | 45 |
| Total | 725 |