[image: image2.png]Cyfoeth
Naturiol
Cymru

Natural
Resources
Wales





Notice of variation and consolidation with introductory note
The Environmental Permitting (England & Wales) Regulations 2016
Hafren Dyfrdwy Cyfyngedig
Llwyn Onn Water Treatment Works 

Cefn Road
Abenbury
Wrexham
LL13 0NY
Variation application number

PAN-014239
Permit number
CM0066701
Llwyn Onn Water Treatment Works 

Permit number CM0066701
Introductory note

This introductory note does not form a part of the notice.
The following notice gives notice of the variation and consolidation of an environmental permit.
This permit has been varied to include Operator Self Monitoring (OSM) conditions.

The schedules specify the changes made to the permit.
The status log of a permit sets out the permitting history, including any changes to the permit reference number.  It is not backdated before 6 April 2010. 
	Status log of the permit

	Description
	Date
	Comments

	Permit determined
CM0066701
	13/07/2000
	Permit issued to Dee Valley Water Limited

	Application PAN-004016 received
	04/12/2018
	Notification of change of company name from Dee Valley Water Limited to Hafren Dyfrdwy Cyfyngedig

	Variation determined 

CM0066701/V002 
	04/01/2019
	Varied permit issued to Hafren Dyfrdwy Cyfyngedig

	Application PAN-014239 received
	Duly made

09/06/2021
	Application to vary and update the permit to modern conditions.

	Variation determined CM0066701/V003 
	08/09/2021
	Varied and consolidated permit issued in modern condition format to Hafren Dyfrdwy Cynfyngedig


End of introductory note

	Notice of variation and consolidation 


The Environmental Permitting (England and Wales) Regulations 2016
The Natural Resources Body for Wales (“Natural Resources Wales”) in exercise of its powers under regulation 20 of the Environmental Permitting (England and Wales) Regulations 2016 varies and consolidates
permit number

CM0066701
issued to:

Hafren Dyfrdwy Cynfyngedig (“the operator”)

whose registered office is
Packsaddle
Wrexham

Rhostyllen

Wrexham
Clwyd
LL14 4EH
company registration number 03527628 
to operate a regulated facility at
Llwyn Onn Water Treatment Works
Cefn Road
Abenbury
Wrexham
LL13 0NY
to the extent set out in the schedules.
The notice shall take effect from 08/09/2021
	Name
	Date

	Melissa Broad


	08/09/2021


Authorised on behalf of Natural Resources Wales
Schedule 1 

The following conditions were varied as a result of an Natural Resources Wales initiated variation:-
· Conditions 3.3.3, 3.3.4, 3.3.5, 4.3.3 and 3.3.5 were added

· Tables S3.1, S3.2 and S3.3

· Table 3B was added

· Tables S4.1 and S4.2

· Schedule 7
All other conditions were varied as a result of a Natural Resources Wales initiated variation.
Schedule 2 – consolidated permit

Consolidated permit issued as a separate document

	Permit


The Environmental Permitting (England and Wales) Regulations 2016
Permit number 

CM0066701
This is the consolidated permit referred to in the variation and consolidation notice for application PAN-014239 authorising, 
Hafren Dyfrdwy Cyfyngedig (“the operator”),

whose registered office is

Packsaddle

Wrexham Road

Rhostyllen

Wrexham

Clwyd

LL14 4EH

company registration number 03527628
to operate a water discharge activity at

Llwyn Onn Water Treatment Works
Cefn Road
Avenbury
Wrexham
LL13 0NY
to the extent authorised by and subject to the conditions of this permit.

	Name
	Date

	Melissa Broad
	08/09/2021


Authorised on behalf of Natural Resources Wales
Conditions

1 Management

1.1 General management

1.1.1 The operator shall manage and operate the

(a) in accordance with a written management system that identifies and minimises risks of pollution so far as is reasonably practicable, including those risks arising from operations, maintenance, accidents, incidents, non-conformances and those drawn to the attention of the operator as a result of complaints; and

(b) using sufficient competent persons and resources.

1.1.2 Records demonstrating compliance with condition 1.1.1 shall be maintained. 

1.1.3 Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of the permit.

2 Operations

2.1 Permitted activities

2.1.1 The only activity authorised by the permit is the activity specified in schedule 1 table S1.1.

2.2 The site 

2.2.1 The discharge activity shall take place at the discharge point marked on the site plan at schedule 7 to this permit, and as listed in table S3.2.
3 Emissions and monitoring

3.1 Emissions to water

3.1.1 The limits given in schedule 3 table S3.1 shall not be exceeded.

3.2 Emissions of substances not controlled by emission limits

3.2.1 The operator shall take appropriate measures to minimise so far as reasonably practicable the polluting effects of the emissions of substances in the discharge not controlled by emission limits (excluding odour).

3.3 Monitoring 

3.3.1 The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables in schedule 3 to this permit:

(a) point source emissions specified in tables S3.1 and S3.3.

3.3.2 The operator shall maintain records of all monitoring required by this permit.
3 Monitoring equipment, techniques, personnel and organisations employed for the emissions monitoring programme and the environmental or other monitoring specified in condition 3.3.1 shall have either MCERTS certification or MCERTS accreditation (as appropriate), where available, unless otherwise agreed in writing by Natural Resources Wales.

3.3.4 An Accessible monitoring point shall be provided and maintained to enable the emissions monitoring programme [and the environmental or other monitoring]   to be carried out at the monitoring point specified in table S3.3 of schedule 3 and shown marked on the site plan in schedule 7. 

3.3.5 The monitoring programme for the parameters subject to schedule 3B shall be:

(a) pre-scheduled to cover a calendar year and the programme recorded before the start of a calendar year sample period; and

(b) spot samples collected at approximately equal intervals during the year, including samples from different days of the week and different times.  Approximately 10% of samples should be outside the normal sampling window which is 9am-3pm, Monday to Friday.

3.3.6 After becoming aware, or following a notification that a sample has not been taken on the schedule 3B Monitoring Programme pre-scheduled date, or is lost, or a result for that sample cannot be reported, the operator shall record the details and reschedule the sample.

4 Information

4.1 Records

4.1.1 All records required to be made by schedule 3, 4 and 5 to this permit shall:

(a) be legible;

(b) be made as soon as reasonably practicable;

(c) if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

(d) be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made. 

4.1.2 The operator shall maintain convenient access, in either electronic or hard copy, to the records, plan and management system required to be maintained by this permit. 

4.2 Reporting

4.2.1 The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

4.2.2 Within the time period after the end of the reporting period specified in schedule 4 table S4.1 the operator shall, unless otherwise agreed in writing by Natural Resources Wales, submit reports of the monitoring and assessment carried out in accordance with the conditions of this permit, as follows:

(a) in respect of the parameters and monitoring points specified in schedule 4 table S4.1;

(b) [for the reporting periods specified in schedule 4 table S4.1 and] using the forms specified in schedule 4 table S4.2; and

(c) giving the information from such results and assessments as may be required by the forms specified in those tables.

4.3 Notifications

4.3.1 Natural Resources Wales shall be notified as soon as reasonably practicable following detection, within the site of the regulated facility of:

(a) any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution; and

(b) any breach of a limit specified in schedule 3 table S3.1.
(c) any other significant adverse environmental effects, which may have been caused by the activity, shall also be notified to Natural Resources Wales as soon as reasonably practicable following detection.
4.3.2 The information provided under condition 4.3.1 shall be supported by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

4.3.3 Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling specified in schedule [3B/3C], the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

4.3.4 Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

(a) any change in the operator’s trading name, registered name or registered office address; and

(b) any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

(c) any change in the operator’s name or address; and

(d) any steps taken with a view to the dissolution of the operator.

4.3.5 Where the operator proposes to make a change in the nature of the activity by increasing the concentration of, or the addition of, or allowing the introduction of, a substance to the activity to an extent that the operator considers could have a significant adverse environmental effect on the receiving waters, and the change is not permitted by emission limits specified within schedule 3 table S3.1 or the subject of an application for approval under the EP Regulations or under the terms of this permit:

(a) Natural Resources Wales shall be notified in writing at least 14 days before the increase or addition or allowing the introduction; and

(b) the notification shall contain a description of the proposed change.

4.4 Interpretation

4.4.1 In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

4.4.2 In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “as soon as reasonably practicable”, in which case it may be provided by telephone.

Schedule 1 – Operations 

	Table S1.1 Activities

	Description of activity
	Limits of specified activity

	Discharge of trade effluent consisting of Supernatant and backwash waters via Trade Effluent Outfall
	N/A


Schedule 2 – Waste types, raw materials and fuels

Schedule 2 not in use.

Schedule 3 – Emissions and monitoring

	Table S3.1 Point Source emissions to water (other than sewer) – emission limits and monitoring requirements

	Effluent and discharge point
	Parameter  
	Limit (including unit)
	Reference Period
	Limit of effective range
	Monitoring frequency
	Compliance Statistic

	Trade effluent consisting of Supernatant and backwash waters via Trade Effluent Outfall
	Maximum daily discharge volume
	1100 m3/day

5 
	Total daily volume
	N/A                                                                
	Continuous
	Maximum 

	6 
	Maximum rate of discharge
	28  litres per second
	Instantaneous (spot sample)
	N/A
	Continuous
	Maximum

	7 
	15-minute instantaneous or averaged flow
	No limit set. Record as l/s
	15 minute
	N/A
	Continuous

8 
	N/A

9 

	10 
	ATU-BOD as O2
	20 mg/l
	Instantaneous (spot sample)
	N/A
	As specified in schedule 3B
	Maximum

11  

	12 
	Suspended solids (measured after drying at 105oC)
	50 mg/l
	Instantaneous (spot sample)
	N/A
	As specified in schedule 3B
	Maximum

13  

	14 
	Total available chlorine
	50 µg/l
	Instantaneous (spot sample)
	N/A
	As specified in schedule 3B
	Maximum

15  

	16 
	pH
	6 to 9
	Instantaneous (spot sample)
	N/A
	As specified in schedule 3B
	Minimum and maximum

	17 
	Total Aluminium
	10,000 µg/l
	Instantaneous (spot sample)
	N/A
	As specified in schedule 3B
	Maximum


	Table S3.2 Discharge points

	Effluent Name
	Discharge Point
	Discharge point NGR
	Receiving water

	Trade effluent consisting of Supernatant and backwash waters via Trade Effluent Outfall 
	Trade Effluent Outfall
	SJ 35823 48549
	River Clywedog


	Table S3.3 Monitoring points

	Effluent and discharge point
	Monitoring type
	Monitoring point NGR
	Monitoring point reference

	Trade effluent consisting of Supernatant and backwash via Trade Effluent Outfall 
	Effluent sampling
	SJ 35874 48754
	OSM Sampling Point

	18 
	Flow monitoring 
	SJ 35871 48760
	Flow Monitoring Point

	19 
	OSM Sampling
	SJ 35874 48754
	OSM Sampling Point


Schedule 3B - OSM tier 2 sampling frequency

	Tier 2 Category
	Parameter
	Normal frequency` of  samples per year 

20 
	Reduced Sampling frequency after 12 consecutive months of numeric permit compliance, samples per year or pro rata over the remainder of a year
	On numeric limit failure return to normal frequency as soon as reasonably practicable, samples per 12 months
	Out of hours samples

	5m3/d (but not in tier 3) with numeric limits for ammoniacal nitrogen (expressed as N), nutrients, hazardous pollutants, UWWTR self monitoring
	Sanitary 

Non sanitary
	12

6
	6

6
	12

6
	For 12 samples 1 out of hours sample per annum, for 6, 1 every 2 years on average

21 

	>20 m3/d (but not in tier3) with numeric limits for one or more of biochemical oxygen demand or suspended solids or pH or temperature only.
	Sanitary
	12
	4
	12
	For 12 samples 1 out of hours sample per annum, for 4, 1 every 3 years on average

22 


Schedule 4 – Reporting

Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

	Table S4.1 Reporting of monitoring data

	Parameter
	Monitoring point reference
	Reporting period
	Period begins

	Total daily volume

23 
	Flow monitoring point
	Annually

Report to be submitted within 2 months of the end of the calendar year
	1 January

	15-minute flow

24 
	Flow monitoring points table
	Reports to be provided to Natural Resources Wales upon request 

Report to be submitted within 28 days unless otherwise specified in writing by Natural Resources Wales
	Upon request by Natural Resources Wales

	Operator Self Monitoring - ATU-BOD as O2, ammoniacal nitrogen (expressed as N), suspended solids (measured after drying at 105oC), total chlorine as Cl, pH, total Aluminium as Al
	OSM Sampling Point
	Quarterly

Report to be submitted within 28 days
	1st of month

	Operator Self Monitoring summary report 
	OSM Sampling Point
	Annually

Report to be submitted within 2 months of the end of the calendar year
	1 January


	Table S4.2 Reporting forms

	Parameter
	Reporting format

	Total daily volume

25 
	WISKI electronic format specified by Natural Resources Wales

	15-minute flow

26 
	WISKI electronic format specified by Natural Resources Wales

	OSM - ATU-BOD as O2, suspended solids (measured after drying at 105oC), total available Chlorine as Cl, pH, total Aluminium as Al
	Quarterly - Electronic format specified by Natural Resources Wales

	Operator Self Monitoring summary report
	Annually - Summary report of compliance with the monitoring programme specified in Table 3.1 and Schedule 3B in a format specified by Natural Resources Wales


Schedule 5 – Notification 

These pages outline the information that the operator must provide. 

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

Part A

	Permit Number
	CM0066701

	Name of operator
	27 

	Location of Facility
	Llwyn Onn Water Treatment Works, Cefn Road, Abenbury, Wrexham, LL13 0NY

	Time and date of the detection 
	28 


	(a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques,  accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution

	To be notified within 7 days of detection unless otherwise agreed in writing by Natural Resources Wales

	Date and time of the event
	29 

	Reference or description of the location of the event 
	30 

	Description of where any release into the environment took place
	31 

	Substances(s) potentially released/type or nature of sewage released
	32 

	Best estimate of the quantity or rate of release of substances and/or duration of discharge
	33 

	Best estimate of the environmental impact of the discharge
	34 

	Measures taken, or intended to be taken, to stop any emission
	35 

	Description of the failure or accident.
	36 


	(b) Notification requirements for the breach of a limit specified in schedule 3 table S3.1 

	The information specified below is to be notified to Natural Resources Wales as soon as reasonably practicable following detection.

	Monitoring point reference/ source
	37 

	Self monitoring regime (where relevant)
	e.g. OSM

38 

	Type of failure 
	e.g. maximum/other



	Date of sample/event
	39 

	Parameter
	40 

	Result and units
	41 

	Limit and units
	42 


Part B – to be submitted as soon as reasonably practicable unless otherwise agreed in writing by Natural Resources Wales

	Any more accurate information on the matters for notification under Part A.
	43 

	Measures taken, or intended to be taken, to prevent a recurrence of the incident/breach/exceedance
	44 

	Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission
	45 


	Name*
	46 

	Post
	47 

	Signature
	48 

	Date
	49 


* authorised to sign on behalf of the operator
Schedule 6 – Interpretation 

“accident” means an accident that may result in pollution.

"annually" means once every year.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“appropriate measures” for the purposes of the emission of substances not controlled by emission limits condition (condition 3.2.1) do not require the operator to undertake treatment to a level beyond that specified in schedule 1 table S1.1, or to carry out routine monitoring for substances not controlled by emission limits.

“ATU-BOD as O2” means the biochemical oxygen demand (measured after 5 days at 20oC with nitrification suppressed by the addition of allylthiourea).

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the permitted activities, which are not controlled by an emission limit.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.
“MCERTS” means Natural Resources Wales’ Monitoring Certification Scheme.
 “Natural Resources Wales” means the Natural Resources Body for Wales established by article 3 of the Natural Resources Body for Wales (Establishment) Order 2012. The Natural Resources Body for Wales (Functions) Order 2013 transferred the relevant functions of the Countryside Council for Wales, and functions of the Environment Agency and the Forestry Commission in Wales to the Natural Resources Body for Wales.

"quarter" means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“significant pollution” means a high level impact incident as indicated by NRW incident classification system.
“year” means calendar year ending 31 December.

Schedule 7 – Site plan

[image: image1.emf]
© Crown Copyright and database right 2021. Ordnance Survey licence number 100019741.

END OF PERMIT

	Variation and consolidation application number PAN-014239  CM0066701
	Page i
-  -
- i -
- i -i
	


PAGE  
	Variation and consolidation application number PAN-014239  CM0066701
	Page 3
-  -
- 3 -
- 3 -3
	



