# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for 1 -10 Channel View operated by Vale of Glamorgan Council.

The application number is PAN-011865

The permit number is EPR/CB3294FU

The permit regulates a new discharge of secondary treated sewage effluent from a residential estate comprised of ten 3-bedroommed properties. A maximum of 6 m3/day of sewage effluent will be treated in a British Standard certified package treatment plant connected to a phosphate dosing chamber prior to its discharge into Marcross Brook via a surface water drain. Sewage effluent at the site was previously being discharge to a failing cesspit and the new activity will therefore offer an improvement to the current system. The permit has numeric limits in place for ammonia and phosphate.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 web publicising responses

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to consultation and web publicising | The web publicising and consultation responses (Annex 2) were taken into account in the decision. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the site has been carried out as part of the permitting process. We consider that the application will affect the features of the site. Emission limits have been placed on the permit to mitigate against these effects. We have not formally consulted on the application. The decision was taken in accordance with our guidance. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. This has been ascertained at a screening distance of 1000m from the discharge point. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. The proposed treatment efficiencies for sanitary determinands were assessed using RQP software to establish potential deterioration in the receiving waterbody as a result of the discharge. This identified potential deterioration and shift in the waterbodies’ WFD status class for ammonia and phosphate (British Standard treatment efficiencies). For the waterbody to remain within its current status class we set new limits of 1.3 mg/l phosphate and 4.6 mg/l ammonia. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |

# Annex 2: web publicising responses

Response received from

Public Representee

Brief summary of issues raised

Concerns raised over limited flow in the receiving watercourse (Marcross brook) and impact of discharge to downstream SSSI.

Summary of actions taken or show how this has been covered

Flow estimations using QUBE suggested an adequate mean flow of 1434 m3/day and Q95 flow of 346 m3/day. Due to lowered confidence in the flow estimates, NRW’s Hydrometry team went out to take flow gauging’s of Marcross Brook on 4 separate occasions between April and July 2021. This data showed a substantial reduction from the estimated flows with a mean flow of 130 m3/day and a Q95 flow of 17 m3/day. Consequently, sanitary parameters were re-modelled in RQP software to assess potential deterioration with the reduced flow rates. The results from modelling showed potential deterioration and shift in the waterbodies’ WFD status class at the operators proposed limits for ammonia and phosphate (British Standard treatment efficiencies). For the waterbody to remain within its current status class we have proposed new limits to the operator of 1.3 mg/l phosphate and 4.6 mg/l ammonia. The operator accepted these limits during review of the draft permit and intends to install a suitable package treatment plant equipped with a phosphate dosing chamber in order to achieve the agreed standards.