# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Coal Terminal operated by South West Wood Products Limited.

The permit number is EPR/CB3295FG/A001

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the consultation, and web publicising responses

# Annex 1: decision checklist

This document should be read in conjunction with the Duly Making checklist, the application and supporting information and permit/ notice.

| ### Aspect considered |  | ### Justification / Detail | Criteria met |  |
| --- | --- | --- | --- | --- |
|  |  |  | Yes |  |
| Consultation |  |  |  |  |
| Scope of consultation |  | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. |  |  |
| ### Responses to consultation and web publicising |  | The web publicising and consultation responses (Annex 2) were taken into account in the decision. As part of the determination we consulted with the following organisations: - Associated British Ports - Newport City Council – Planning - Aneurin Bevan University Health Board - Newport City Council – Environmental Health - South Wales Fire and Rescue Service (SWFRS) A summary of the consultation responses from organisations which responded to our consultation request are provided in Appendix A. The decision was taken in accordance with our guidance. |  |  |
| Operator |  |  |  |  |
| Control of the facility |  | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. |  |  |
| Sustainable Management of Natural Resources (SMNR) |  |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources.” |  |  |
| The site |  |  |  |  |
| Extent of the site of the facility |  | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. |  |  |
| Site condition report |  | The operator has provided a description of the condition of the site. We consider this description is satisfactory. The decision was taken in accordance with our guidance on site condition reports – guidance and templates (H5). |  |  |
| Biodiversity, Heritage, Landscape and Nature Conservation |  | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the Severn Estuary Special Area of Conservation (SAC), Special Protection Area (SPA), Ramsar and the River Usk SAC and the Gwent Levels St Brides SSSI, Newport Wetlands SSSI, River Usk (lower Usk) SSSI, and Severn Estuary SSSI has been carried out as part of the permitting process. The sealed, impermeable nature of the site removes the pathways as identifed and the remaining controls are inbuilt into standard permitting (where protected sites are not a constraint) and therefore this appears consistent with People over Wind ruling. We consider that the application will not affect the features of the protected sites OGN 200/Form 1 assessment Assessment of Likely Significant Effect: The project has been screened for likelihood of significant effects and, taking account of the advice received from protected sites advisors, is considered not likely to have a significant effect on any Natura 2000/Ramsar site (as documented in section 3.2 of OGN 200 form 1, or section 5 if applicable) Form 1 (and Form 2) – saved on DMS as “to be assessed” for public register purposes. |  |  |
| Environmental Risk Assessment and operating techniques |  |  |  |  |
| Environmental risk |  | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all none point source emissions may be categorised as environmentally insignificant. |  |  |
| Operating techniques |  | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The operator has provided a Dust Management Plan that details the measures used to prevent or minimise the dust emissions from the facility. These measures have been assessed and considered suitable to control dust emissions from the facility in line with ‘How to comply with your environmental permit’. The operator has provided a Fire Prevention and Mitigation Plan that details the measures used to prevent or minimise the risk of fires on site. These measures have been assessed and considered suitable in line with Guidance Note 16: Fire Prevention & Mitigation Plan Guidance – Waste Management. The operator has provided an Environmental Management Section document which outlines a number of measures to ensure that the site does not cause pollution during operation including waste pre-acceptance and waste acceptance criteria, refuelling plant and machinery, control of mud and litter, spillage. These measures have been assessed and considered suitable and in line with ‘How to comply with your environmental permit’ and ‘Technical Guidance Note WM3: Waste Classification’ |  |  |
| The permit conditions |  |  |  |  |
| Waste types |  | We have specified the permitted waste types, descriptions and quantities, which can be accepted at the regulated facility. We are satisfied that the operator can accept these wastes for the following reasons as they are consistent with the type of facility and the on-site pollution controls and infrastructure (e.g. impermeable surface with sealed drainage system to containment with no discharges to the environment). We made these decisions with respect to waste types in accordance with WM3 Waste Classification: Guidance on the classification and assessment of waste and how to comply with your environmental permit. |  |  |
| Incorporating the application |  | We have specified that the applicant must operate the permit in accordance with descriptions in the application, including all additional information received as part of the determination process. These descriptions are specified in the Operating Techniques table in the permit. |  |  |
| Operator Competence |  |  |  |  |
| Environment management system |  | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. |  |  |
| Technical competence |  | Technical competency is required for activities permitted. The operator is a member of an agreed scheme. The operator has confirmed that the TCM is Mr John Boland who holds: - Level 4 in Waste Management Operations – Managing Treatment Hazardous Waste (4TMH) - Level 4 in Waste Management Operations – Managing Transfer Hazardous Waste (4TSH) The operator has also confirmed that the technical competent manager has the necessary continuing competence certificates in place until 2023. The operator satisfies the criteria in RGN 5 on Operator Competence. |  |  |
| Relevant convictions |  | Our Enforcement Database has been checked to ensure that all relevant convictions have been declared. No relevant convictions were found. |  |  |
| ### Financial competence |  | There is no known reason to consider that the operator will not be financially able to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. |  |  |

# Annex 2: Consultation, and web publicising responses

Summary of responses to consultation, and web publication and the way in which we have taken these into account in the determination process.

Response received from

Aneurin Bevan University Health Board

Brief summary of issues raised

The main points raised by Aneruin Bevan UHB were: - Subject to assured application of Best Available Techniques (BAT) to control emissions and fire risk, then we would have no public health concerns. - Our main concern with sites that store and manage waste such as wood is the risk of fire.

Summary of actions taken or show how this has been covered

Throughout the determination of the application we have assessed the Operators Fire Prevention and Mitigation Plan against our Guidance Note 16: Fire Prevention & Mitigation Plan Guidance – Waste Management. This also included consultation with the South Wales FRS. The Fire Prevention and Mitigation Plan has been added to the Table 1.2 of the Operating Techniques table in the permit. In regards to emissions as part of the permit determination we have assessed the operators Dust Management Plan. During this assessment we have considered our how to comply with your environmental permit guidance. The Dust Management Plan has been added to the Table 1.2 of the Operating Techniques table in the permit.

Response received from

Newport City Council – Environmental Health

Brief summary of issues raised

No comments were raised.

Summary of actions taken or show how this has been covered

Not applicable.

Response received from

South Wales Fire and Rescue Service (SWFRS)

Brief summary of issues raised

The main points raised by SWFRS were: - The Fire Prevention and Mitigation Plan (FPMP) mentions that the site operatives will be trained in practical use of fire extinguishers and other methods of extinguishing fires, and this will be provided by the Fire and Resuce Service (FRS). The FRS do not carry out the training mentioned in the FPMP. - The operator needs to outline how they will inform nearby sensitive receptor of a major fire.

Summary of actions taken or show how this has been covered

The above points have been addressed by the final version of the FPMP The reference to staff being trained by the FRS has been removed. In addition the Operator has also confirmed how they will communicate with nearby receptors in the event of a fire.