SCR evaluation form
	Applicant
	South West Wood Products Limited

	Permit reference
	PAN 013672
EPR/CB3295FG

	Address
	Coal Terminal, South Docks, East Way

Road, Newport Docks, Newport

NP20 2WE

	Grid reference
	ST 3169 8429

	Activity Type
	A16 Physical Treatment of Non Hazardous Waste. 
The site operations are for the treatment of the permitted wastes by manual sorting, bulking up, cutting, shredding, chipping and screening with storage of unprocessed and processed materials. 

Waste types limited to ‘wood’ type wastes as outlined in the Supplementary Information – Application for Environmental Permit provided 


	SCR document reference
	2190/SCR

	SCR date and version number
	March 2021


	1.0 Site details 
(Source)

	Has the applicant provided the following information as required by the application SCR template?
	Response 



	Site plans showing site layout, drainage, surfacing, receptors, sources of emissions/releases and monitoring points


	Permitting Officer Comments 

Permit Plan: LMM 047 01 Revision A. (Updated from original application) available in the Environmental Management Plan. 

The Operator’s Site Condition Report confirms that the site surface area is impermeable surfacing with drains into holding pits. 



	2.0 Condition of the land at permit issue
(Receptor)

	Has the applicant provided the following information as required by the application SCR template?
	Response 



	a) Environmental setting including geology, hydrogeology and surface waters

b) Pollution history including:

· pollution incidents that may have affected land

· historical land-uses and associated contaminants

· visual/olfactory evidence of existing contamination

· evidence of damage to existing pollution prevention measures

c) Evidence of historic contamination (i.e. historical site investigation, assessment, remediation and verification reports (where available)

d) Has the applicant chosen to collect baseline reference data?
	Permitting Officer Comments
The Operators Site Condition Report confirms the following: 

· Geology: Bedrock - Mercia Mudstone Group Superficial – Tidal Flat Deposits and Glaciofluvial Deposits

· Hydrogeology: not within a groundwater protection zone. Newport docks are to north and west. The River Usk is over 185m to the south of

the site. Other than drainage associated with the docks there are no surface waters across the site.

· No known pollution incidents

· The site is made up ground which as an impermeable surfacing with drains into a holding pit then discharges into the River Usk. 

· The ground originally farmland then the ‘South Dock’ was built in 1893. The area has been used for and also for railway sidings. There are no buildings on the site.

· No evidence of contamination or damage to pollution measures

In addition to the above its worth noting that the operator has confirmed in their EMS that: 

· The site currently drains into a holding pit with the waters then discharging to the River Usk however for the duration of this waste operation the outlets of the holding pit will be sealed off to ensure there is no water emission point for the discharge of site waters. Water in the holding pit will be pumped into one of two water storage tanks to retain the site waters and allow their reuse on site in dust suppression.


	3.0 Permitted activities 
(Source)

	Has the applicant provided the following information

as required by the application SCR template?
	Response  

	a) Permitted activities

b) Non-permitted activities undertaken at the site


	The operator has proposed to undertake recycling of waste wood.
The permitted activities include R3 & R13. 

Treatment operations are limited to manual sorting, separation, bulking, screening, chipping and shredding.

All treatment is for the purpose of recovery only. Treatment may be by either manual and/or mechanical methods.




	3.0(a) Environmental Risk Assessment 

(Source)

	The H1 environmental risk assessment should identify elements that could impact on land and waters, cross- referenced back to documents and plans provided as part of the wider permit application.


	The operators Risk Assessment (Coal Terminal Risk Assessment Doc Ref RA/2190 – 25/03/2021) demonstrates an assessment of the major risks from the site (eg. contaminated discharges, dust, noise, odour, fire)

The site also has a Dust and Emissions Management Plan (2190/DEMP – V1.3) and a Fire Prevention and Mitigation Plan (2190/FPMP – V1.4). 

All measures within these documents appear to be consistent with the type of application and activity proposed. 






	3.0(b) Will the pollution prevention measures protect land and groundwater?

(Conceptual model)

	Are the activities likely to result in pollution of land? 
	No. 
Site Surface Water Run Off

This permit does not allow any point source emission into surface waters or groundwater. All run-off from site surfaces are directed on to the on-site holding pits where they are then pumped into one of the two water storage tanks on site ready for reuse on site for dust suppression. Fire water containment is also outlined within the sites FPMP (2190/FPMP – V1.4)

The site surface is 100% impermeable. 
Fugitive Dust Emissions 

The site also has a Dust and Emissions Management Plan (2190/DEMP – V1.3) which outlines suitable controls on emissions of fugitive dusts. This will prevent large quantities of dusts being deposited to land within or outside of the permit boundary.

	For dangerous and/or hazardous substances only, are the pollution prevention measures for the relevant activities to a standard that is likely to prevent pollution of land?
	N/A - no dangerous or hazardous wastes permitted.


	Application SCR decision summary 
	Complete all that apply

	Sufficient information has been supplied to describe the condition of the site at permit issue.
	Yes

	Information is missing- the following information must be obtained from the applicant.
	N/A

	Pollution of land and water is unlikely- pollution prevention measures just need to be reviewed during operation of the site
	Yes

	Pollution of land and water is likely
	No. The site surface is 100% impermeable.  All run-off from site surfaces are directed on to the on-site holding pits where they are then pumped into one of the two water storage tanks on site ready for reuse on site for dust suppression.

	Historical contamination is present- advise operator that collection of background data may be appropriate 
	No background data provided.

	Date and name of reviewer
	Matthew Fryer
Waste Permitting Officer

08/09/2021


	
	
	
	


	Doc No 233_06
	Version 6
	Last printed 29/09/2021
	Page 2 of 6


[image: image1.png]




