| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-015033 |
| Permit Reference Number: | EPR/AB3891CX |
| Permit Holder: | Mr Daniel & Mrs Carys James |
| Site Address: | Hafod Farm, Ferwig, Cardigan, Ceredigion, SA43 1PU & Pwllglas, Sarnau, Llandysul, Ceredigion, SA44 6QR-removed 07/10 |
| Grid Reference: | Farm location: Hafod Farm SN 18094 50342 & Blaencwmpridd SN 31513 51310 Fields: Hafod Farm - land at Crug Farm 8295 SN 16822 50937 1594 SN 17135 50953 2312 SN 17239 51116 3498 SN 17351 50971 4526 SN 17453 51260 5412 SN 17542 51112 5994 SN 17607 50931 7025 SN 17697 51249 8232 SN 17839 51280 to be removed-see e-mail 07/10 Blaencwmpridd - land at Pwll Glas- to be removed, see e-mail of 07/10 0454 SN 32017 51519 2934 SN 32292 51327 Storage: SN 18153 50376 contingency only-confirmed 07/10 SN 32351 51206 SN 18100 50729 nurse tank added 07/10 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: CeredigionEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: ffion.quan@cyfoethnaturiolcymru.gov.uk |
| Date due: | 02/09/2021 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- DMS-no variations |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | N Checked on 03/09 LB 24/09 JG 27/09 JG -contacted banking-unable to find, confirmed 27/09 Y | Amount: £1018.00 Date cleared on MyNRW: 05/10/2021 Transaction number: 2094075 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | N | Not supplied or required at time of application |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | EPR/AB3891CX |
| ## Has the permit holder name been provided? | Y | Mr Daniel & Mrs Carys James |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y | Mr David Powell |
| ## Has the technically competent manager details been provided? | Y | Mr David Powell |
| ## Has the nominated competent person details been provided? | N Y | e-mail of 07/10 confirmed as DJ Powell |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y Mr Morris Davies-Hafod Farm Mr James Cowan-Pwllglas | 55/226/0027 *See e-mail 07/10 55/211/0053 |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 50ha Amended 07/10, 34.6ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | Mr Morris Davies-Hafod Farm-differs to MyMap Confirmation of status and consent given in e-mail of 07/10 |
| ## Has consent been given to carry out the activity? | Y | Tbc in RFI Updated information given 07/10 , confirmed |
| Has all of the previous land treatment table been completed? | Y | PAN-010105 Expired 20/05/21 |
| ## Which risk band does the activity fall within? | High | List B wastes, High Risk location |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | Y |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | n/a |  |
| Has all of the waste types table been completed? | Y | 02 05 02,02 07 02 |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Google |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| Has checklist been completed? | Y |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a | Concerns re storage lagoon contaminating stored waste-see RFI Lagoon for contingency only, will be empty if needed-replaced by nurse tank, map amended accordingly-see e-mail 07/10 |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y | 10m non-spreading zone in place |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | Y |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring(or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | Cardigan Bay and West Wales Marine SACs very close to fields at Crug; Aberarth - Carreg Wylan & Caeau Crug Bychan, Ty Gwyn a Llwyn Ysgaw adjacent to 4526,7025 & 8232 |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y Some not all | PH- Lowland dry acid grassland and Lowland calcareous grassland adjacent to 8232,2312,4526,1594,0454,2934 close to 7025, 8232; Purple moor grass and rush pastures in 5994, adjacent to 3498; SAM- Crug-Bychan Round Barrow close to 7025, 8232; AW-Restred ancient Woodland & Ancient Semi Natural Woodland within 500m of 0454 , 2934 but not close; |
| What is the slope of the fields? 8232 only real concern, justification & mitigation in place for other fields -see SSRA and map “red” areas included in buffer zones already in place on spreading map | 0-7 degrees (transparent)- fields #8295,2312,3498, 5412,5994(Crug) 7 -12 degrees (yellow)- fields #0454, 2934(Pwllglas), small area to north of 1594, area straddling 7025 and 8232, larger area to north of 8232, small strip at NW of 4526 12 degrees plus (red)- northern edge of field #0454 (Pwllglas), small area toward north end of 1594 (Crug) |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y | July 21 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | N |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y | March 2020 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | Y |  |
| - Stated the source of information that they used to set the nutrient level. | Y |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. Although there is potential for impact of the proposed activities, due to the condtions of the SR2010 No.4 under which this waste is being deployed i.e non-spreading buffer zones of 10m around any watercourse and 50m surrounding any well, spring or borehole used for the supply of water consumption including from a private water supply or supplies, the potential impact pathway is reduced. The following measures the applicant intends to put in place will further minimise risks. These are : Ensuring that appropriate application rates are adhered to in accordance with the Agricultural Benefit Statement provided and nutrient calculations based on RB209 ; Non spreading areas to be observed as listed in site-specific risk assessment –20m no spread areas on the border of fields closest to the SAC; Spread only when weather conditions are suitable i.e not during periods of heavy rainfall, icy conditions etc. ; Spreading done by sub surface injection system for grassland, thereby minimising risk of odour and optimising nutrient availability to the medium soil type of the fields ; On arable fields waste will be applied and incorporated into the soil with cultivations or surface applied with a dribble bar into the standing crop ; Wastes will be applied in accordance with CoGAP and EMS ; Operations will not cause any additional effects on surrounding roads due to the existing agricultural nature of the area and current normal agricultural practice occurring in the surroundings ; Ensure that the correct spreading equipment is used with professional drivers , who are to be made aware of any site specific guidelines ; Wastes have a low potential to produce airborne dust and particulate matter, good practice operating systems to be followed further minimises airborne risks. |  |

Comments

Consultation responses: - concerns re slurry remaining in lagoon and mixing with controlled waste-see RFI - area team recently become aware that a hedge that provided a natural barrier between 8232 and SSSI no longer exists, implications of SSSI agreement not adhered to means risk increased significantly, request 8232 removed from application. -see RFI

MyMap and location assessment: Saved as projects under PAN-015033, also copies saved to DMS

Protection of habitats and ecosystems No evidence supplied or required at time of application

Agricultural benefit calculations: Closely align with ours, slight differences due to rounding, within acceptable margins, all additions well within limits and less than offtake.

Request for information: Draft prepared and sent for review 29/09 RFI sent 30/09 re points highlighted above-deadline for response 14/10 Response received 07/10 addressing all points -updated information given for landowner/occupier to be accepted since may well have changed since MyMap last updated-see below. Fields 0454 & 2934 at Pwllglas and Hafod 8232 are to be removed from the application. Payment made on 05/10 , confirmed 08/10

Other information to note: MyMap notes the following : Fields C/P/H Owner 1594 55/226/0030 LJ Jenkins and Partners 4526 55/226/0030 LJ Jenkins and Partners 8232 55/226/0030 LJ Jenkins and Partners 5412 55/226/0030 LJ Jenkins and Partners 2312 55/226/0030 LJ Jenkins and Partners 8295 55/226/0030 LJ Jenkins and Partners Response of 07/10 states : Mr Davies of Hafod farm has a number of holding numbers as he owns and rents various farms. This is the list of the various farms and their holding numbers. Hafod 55-226-0027 Penfeidr/Bolafron 55-226-0026 Nantycrio 55-226-0005 Tyriet 55-226-0028 Towyn 55-226-0128 Crug/Cyn yr Ynys 55-226-4527

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 08/10/2021 |
| ## Effective date | 08/10/2021 |
| ## Permitting Officer Date | Judith Green 08/10/2021 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 08/10/2021 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1660 |
| Sign off | 10 |
| Issuing | 30 |
| Total | 1760 |