Red Wharf Bay Sea Defence Wall

Water Framework

Directive

Assessment

CPF6991

Document Control Sheet

| Document Title: | Water Framework Directive Assessment |
| --- | --- |
| Document Author(s): | Elin Mirain Jones |
| Project Ref / Title: | CPF6991 – Red Wharf Bay Sea Defence Wall |
| Project Manager: | Jason Humphreys |

Revision History

| Date | Version No. | Summary of Changes |
| --- | --- | --- |
| 04/08/2021 | 0.01 | Initial draft version |
| 04/08/2021 | 0.02 | Review |

Reviews

| Name | Title | Date | Version |
| --- | --- | --- | --- |
| Eifion Glyn Davies | Senior Environment Officer | 04/08/2021 | 0.02 |

Approvals

| Name | Title | Date | Version |
| --- | --- | --- | --- |
| Owain Griffith | Principal Engineer | 19/08/2021 | 0.02 |

Distribution

| Name | Title | Date | Version |
| --- | --- | --- | --- |
| Jason Humphreys | Assistant Engineer | 20/08/2021 | 0.02 |
| Marine Licensing | Designated Officer | 20/08/2021 | 0.02 |

| © 2020 Gwynedd Council / YGC. All Rights Reserved. Copyright in any or all of this documentation belongs to Gwynedd Council / YGC of Council Offices, Shirehall Street, Caernarfon, Gwynedd, LL55 1SH (the 'Owner') and may not be used, sold, transferred, copied or reproduced in whole or in part, in any manner of form or on any media to any person other than in accordance with the terms of the Owner's agreement or otherwise without the prior written consent of the Owner. |  |
| --- | --- |

Content

| Content |  | 3 |
| --- | --- | --- |
| Background |  | 4 |
| Introduction |  | 4 |
| Legislative Background |  | 5 |
| WFD Assessment in Stages |  | 6 |
| The Proposal |  | 6 |
| WFD Assessment |  | 8 |
| WFD Classification | 8 |  |
| WFD Assessment - Screening: | 9 |  |
| WFD Assessment – Scoping Exercise: | 9 |  |
| Hydromorphology: | 9 |  |
| Biology - habitats: | 10 |  |
| Biology – fish | 11 |  |
| Water Quality: | 11 |  |
| WFD Protected Areas | 13 |  |
| Invasive Non-Native Species (INNS) | 13 |  |
| Summary of Scoping: | 14 |  |
| Assessment of impacts on waterbody quality elements | 15 |  |
| WFD Compliance | 15 |  |
| Biology: habitats | 16 |  |
| Water Quality | 16 |  |
| Protected Areas | 16 |  |
| Invasive Non Native Species | 17 |  |
| Conclusion |  | 17 |
| Appendices |  | 18 |

Background

YGC have been commissioned by Anglesey County Council to carry out Environmental Assessments, including a Water Framework Directive Assessment, in support of a Marine Licence application, for the construction of an increased level of protection to the existing sea defence at Red Wharf Bay, located between the small town of Benllech and the village of Pentraeth on the eastern side of Anglesey. (Grid Reference. X: 253006 Y: 381085).

Introduction

Red Wharf Bay, located between the small town of Benllech and the village of Pentraeth on the eastern side of Anglesey is at increased risk of flooding and coastal erosion. Currently, coastal defence at Red Wharf Bay is provided by a series of mostly concrete and masonry walls of varying profile. From overtopping assessments made of the current defences, there is significant risk of overtopping at present time from extreme tide levels alone. This scheme is proposed to provide an increased protection level to the existing sea defence.

The development has been split into six sections, the first two sections will involve taking down the existing sea defence gabion walls and replace by installing a new precast concrete sections and glass panels to the desired protection level. The remaining four sections will involve pressure pointing, grouting and replacing the existing wall/barrier with a new masonry wall to the desired protection level.

Works are expected to commence in March 2022 for a period 6 to 12 months.

The proposal is subject to a Marine Licence by Natural Resource Wales, and must also follow the guidelines set out by the WFD legislation and not cause negative effects to the water environment, water quality, ecosystems or biodiversity. This report therefore highlights any impacts the work could potentially have on the waterbody, along with mitigation and an assessment (if required) of whether the proposals complies with the Water Framework Directive.

A full method statement can be found as a supporting document, however the main works to be carried out include the following:

1: Establish site compound

2: General site clearance

The works is split into six sections:

Section 1 and 2 – cast in situ proposed new concrete base to provide fixing for the precast concrete sections. Masonry cladding and backfill.

Section 1 – no works to be carried out from the beach

Section 2 – removal of existing gabions and excavation on the beach front. Works to be carried out during spring tides to allow sufficient opportunities between high and low water.

Section 3 – 6 – Pressure pointing and grouting of voids

- Temporary working platform to be erected daily during the course of the works

- Contractor to assess and consider tide times, heights and ground stability

Legislative Background

The Water Framework Directive (WFD) is a European Directive which sets out a strategic planning process for the purposes of managing, protecting and improving the water environment. The WFD introduces new environmental requirements which aim to meet good status in all water bodies. For surface waters, good status is made up of ‘good ecological status (GES)’ (or good ecological potential (GEP) where artificial or heavily modified) and ‘good chemical status’. Ecological status and potential are made up of a number of biological, hydromorphological and physio-chemical quality elements. Chemical status is recorded as either good or failing. For groundwater to be in overall ‘good’ status, both quantitative and chemical status must be ‘good’. The WFD also requires prevention of deterioration in water body status including deterioration of any of the individual quality elements.

The main objectives of the WFD are to:

- Prevent deterioration in the status of aquatic ecosystems, protect them and improve the ecological condition of waters;

- Aim to achieve at least ‘Good Status’ for all waters by 2015 (2021 or 2027 where fully justified within an extended deadline under Article 4.4);

- Promote sustainable use of water;

- Conserve habitats and species that depend directly on water;

- Progressively reduce or phase out the release of individual pollutants or groups of pollutants that present a significant threat to the aquatic environment;

- Progressively reduce the pollution of groundwater and prevent or limit the entry of pollutants; and

- Help reduce the effects of floods and droughts.

New activities and schemes that affect the water environment may adversely impact biological, hydromorphological, physico-chemical and/or chemical quality elements (WFD quality elements), leading to deterioration in water body status. They may also render proposed improvement measures ineffective, leading to the water body failing to meet its WFD objectives for GES/GEP. Under the WFD, activities must not cause deterioration in water body status or prevent a water body from meeting GES/GEP by invalidating improvement measures.

Tudalen 5 o 18

WFD Assessment in Stages

The Water Framework Directive Assessment can have up to three stages, which are

- Screening Stage: excludes any activities that do not need to go through the scoping or impact assessment stages

- Scoping Stage: identifies the receptors that are potentially at risk from your activity and need impact assessment

- Impact Assessment Stage: considers the potential impacts of your activity, identifies ways to avoid or minimise impacts, and shows if your activity may cause deterioration or jeopardise the water body achieving good status

The Proposal

| Your activity | Description, notes or more information |
| --- | --- |
| Name of activity | Red Wharf Bay Sea Defence Wall |
| Brief description of activity | Construction of a new coastal flood defence at Red Wharf Bay. The development has been split into six sections, the first two sections will involve taking down the existing sea defence gabion walls and replace by installing a new precast concrete sections and glass panels to the desired protection level. The remaining four sections will involve pressure pointing, grouting and replacing the existing wall/barrier with a new masonry wall to the desired protection level. |
| Location of activity (central point XY coordinates or national grid reference) | Grid Reference – X:253006, Y: 381085 |
| Footprint of activity (ha) | 600m section of wall. Proposal footprint totals approximately 0.5ha. |
| Timings of activity (including start and finish dates) | Start date: March 2022 End date: August 2022 – March 2023* *Exact timings to be confirmed on contractor appointment. |
| Extent of activity (for example size, scale frequency, expected volumes of output or discharge | The activities will be confined to the areas shown on the plan included as Appendix A. Specific sections of the work, in particular sections 3-6 will be dependent on the tides. Section 2 will be carried out during spring tides and section 1 will not be carried out from the beach/marine environment. |

Tudalen 6 o 18

| Use or release of chemicals (state which ones) | None |
| --- | --- |

Tudalen 7 o 18

WFD Assessment 
WFD Classification

The WFD classification for a defined water body is produced by assessment of a wide variety of different ‘elements’ which includes:

- ‘biological elements’ such as fish, invertebrates, phytobenthos (which includes plants, macro-algae and phytoplankton);

- ‘supporting elements’ that include chemical measurements such as ammonia, dissolved oxygen, pH, phosphate, copper, zinc and temperature; and

- ‘supporting conditions’ (sometimes referred to as hydromorphology), that assess the physical attributes of the water body such as ‘quantity and dynamics of flow’ and ‘morphology’.

The assessment given for each element is also accompanied by a measure of certainty in the result. The status classification is published in the River Basin Management Plan (RBMP)1 and provides a baseline condition against which compliance and future improvements can be measured.

The activity / proposal involves the Anglesey North waterbody, and a summary of key elements is provided below.

| Water body1 | Description, notes or more information |
| --- | --- |
| WFD water body name | Anglesey North |
| Water body ID | GB641010620000 |
| River basin district name | Western Wales River Basin |
| Water body type (estuarine or coastal) | Coastal |
| Water body total area (ha) | 12600ha |
| Overall water body status (2018) | Moderate |
| Ecological status | Good |
| Chemical status | Fail |
| Target water body status and deadline | Not Designated |
| Hydromorphology status of water body | Not Designated |
| Heavily modified water body and for what use | No |
| Higher sensitivity habitats present | Mussel beds habitat within 500m of the proposed work (at 26m east of the works at closest point) |

1

https://www.gov.uk/government/collections/river-basin-management-plans-2015#north-west-river-basin-district-rbmp:-2015

Tudalen 8 o 18

| Lower sensitivity habitats present | Rocky shore, gravel and cobbles, intertidal soft sediment (mudflats and sandflats) |
| --- | --- |
| Phytoplankton status | Not Designated |
| History of harmful algae | None |
| WFD protected areas within 2km | Trwyn Dwlban SSSI, Morwenoliaid Ynys Mon SPA, Bae Lerpwl SPA, Menai Strait and Conwy Bay SAC |

WFD Assessment - Screening:

The WFD is also required in support of a Band 2 Marine Licence Application, by NRW.

WFD Assessment – Scoping Exercise:

The scoping exercise is carried out to identify all the potential risks to each receptor element that are used to determine the status of the waterbody. The receptors / elements are:

- Hydromorphology

- biology – habitats

- biology – fish

- water quality

- protected areas

These receptors are based on the water body’s quality elements (supporting elements, biological elements, and chemical elements).

The scoping exercise has been undertaken following the template provided on the gov.uk2 website.

Hydromorphology:

| Consider if your activity: | Yes | No | Hydromorphology risk issue(s) / observations |
| --- | --- | --- | --- |
| Could impact on the hydro morphology (for example morphology or tidal patterns) of a water body at high status | Requires impact assessment | Impact assessment not required | The proposal seeks to maintain and improve the existing structure. The improvements will be on the same footprint as existing wall. |
| Could significantly impact the hydro morphology of any water body | Requires impact assessment | Impact assessment not required | As above. |

2 https://www.gov.uk/guidance/water-framework-directive-assessment-estuarine-and-coastal-waters#contents

Tudalen 9 o 18

| Is in a water body that is | Requires impact assessment | Impact | The proposed activity is not |
| --- | --- | --- | --- |
| heavily modified for the same |  | assessment | within a waterbody classed as |
| use as your activity |  | not required | heavily modified. |

Biology - habitats:

Consider if habitats are at risk from your activity.

| Higher sensitivity habitats 3 | Lower sensitivity habitats 4 |
| --- | --- |
| chalk reef | cobbles, gravel and shingle |
| clam, cockle and oyster beds | intertidal soft sediments like sand and mud |
| intertidal seagrass | rocky shore |
| maerl | subtidal boulder fields |
| mussel beds, including blue and horse mussel | subtidal rocky reef |
| polychaete reef | subtidal soft sediments like sand and mud |
| saltmarsh |  |
| subtidal kelp beds |  |
| subtidal seagrass |  |

| Consider if the footprint5 of your activity is: | Yes | No | Biology habitats risk issue(s) / observations |
| --- | --- | --- | --- |
| 0.5km2 or larger | Yes to one or more – requires impact assessment | No to all impact assessment not required | No – the total area of the proposed activity is 0.5ha, which equates to 0.005km2. |
| 1% or more of the water body’s area |  |  | No - the total area of water body is 12600 ha, and area of proposed activity does not exceed 0.5ha, which represents less than 0.0040% of the waterbody area. |
| Within 500m of any higher sensitivity habitat |  |  | Yes – The proposed work is within 500m and located 26m west of a mussel bed habitat. Due to the proximity of the higher sensitivity habitat and the tidal processes between both sites, an impact |

3 Higher sensitivity habitats have a low resistance to, and recovery rate, from human pressures.

4 Lower sensitivity habitats have a medium to high resistance to, and recovery rate from, human pressures.

5 Note that a footprint may also be a temperature or sediment plume. For dredging activity, a footprint is 1.5 times the dredge area.

Tudalen 10 o 18

|  |  |  | assessment is required, as it cannot be ruled out at the scoping stage. |
| --- | --- | --- | --- |
| 1% or more of any lower sensitivity habitat |  |  | No – There are lower sensitivity habitats within the area of the works. However, it is not believed that the footprint of the scheme would exceed a total area of more than 1% of the habitat area. |

Biology – fish

Consider if fish are at risk from your activity, but only if your activity is in an estuary or could affect fish in or entering an estuary.

| Consider if your activity: | Yes | No | Biology fish risk issue(s) / observations |
| --- | --- | --- | --- |
| Is in an estuary and could affect fish in the estuary, outside the estuary but could delay or prevent fish entering it or could affect fish migrating through the estuary | Continue with questions | Go to next section | Whilst the work is located on the lower reaches of Afon Nodwydd, the work does not involve creating any physical barriers within the watercourse that would impact fish movement. No other works such as piling is scheduled either that would result in noise that would deter fish from migrating up the river. |
| Could impact on normal fish behaviour like movement, migration or spawning (for example creating a physical barrier, noise, chemical change or a change in depth or flow) | Requires impact assessment | Impact assessment not required | Not applicable |
| Could cause entrainment or impingement of fish | Requires impact assessment | Impact assessment not required | Not applicable |

Water Quality:

Consider if water quality is at risk from your activity.

Tudalen 11 o 18

| Consider if your activity: | Yes | No | Water quality risk issue(s) / observations |
| --- | --- | --- | --- |
| Could affect water clarity, temperature, salinity, oxygen levels, nutrients or microbial patterns continuously for longer than a spring neap tidal cycle (about 14 days) | Requires impact assessment | Impact assessment not required | No – the proposed work would predominantly be carried out above tide level, and therefore would not directly impact water clarity, temperature, oxygen levels, nutrients or microbial patters of the water body, especially not for continuous periods longer than 14 days. There may be some discolouration of water and sediment resuspension, however, this would clear within 1 tidal cycle i.e. high water – low water – high water |
| Is in a water body with a phytoplankton status of moderate, poor or bad | Requires impact assessment | Impact assessment not required | No – From available information, the waterbody does not have a phytoplankton status of moderate, poor or bad. |
| Is in a water body with a history of harmful algae | Requires impact assessment | Impact assessment not required | No – As far as we are aware, the waterbody does not have a history of harmful algae. |

Consider if water quality is at risk from your activity through the use, release or disturbance of chemicals.

| If your activity uses or releases chemicals (for example through sediment disturbance or building works) consider if: | Yes | No | Water quality risk issue(s) / observations |
| --- | --- | --- | --- |
| The chemicals are on the | Requires | Impact assessment not required | Plant machinery will be required to undertake the proposed activity, and therefore there is an inherent risk of releasing oils and fuels into the environment, albeit, the risk is considered to be low. |
| Environmental Quality | impact |  |  |
| Standards Directive (EQSD) list | assessment |  |  |

Tudalen 12 o 18

| It disturbs sediment with contaminants above Cefas Action Level 1 | Requires impact assessment | Impact assessment not required | No – we have no reason to believe that the activity would disturb sediments with contaminants above Cefas Action Level 1. |
| --- | --- | --- | --- |

| If your activity has a mixing zone (like a discharge pipeline or outfall) consider if: | Yes | No | Water quality risk issue(s) / observations |
| --- | --- | --- | --- |
| The chemicals released are on the Environmental Quality Standards Directive (EQSD) list | Requires impact assessment5 | Impact assessment not required | No – the proposal does not use a chemical mixing zone. |

WFD Protected Areas

Consider if WFD protected areas are at risk from your activity. These include:

- Special Areas of Conservation (SAC)

- Special Protection Areas (SPA)

- Shellfish waters

- Bathing Waters

- Nutrient Sensitive Areas

| Consider if your activity is: | Yes | No | Protected areas risk issue(s) / observations |
| --- | --- | --- | --- |
| Within 2km of any WFD protected area6 | Requires impact assessment | Impact assessment not required | The proposed work is within and / or within 2km of Trwyn Dwlban SSSI, Morwenoliaid Ynys Mon SPA, Bae Lerpwl SPA, Menai Strait and Conwy Bay SAC. |

Invasive Non-Native Species (INNS)

Consider if there is a risk your activity could introduce or spread INNS. 
Risks of introducing or spreading INNS include:

- materials or equipment that have come from, had use in or travelled through other water bodies

- activities that help spread existing INNS, either within the immediate water body or other water bodies

Tudalen 13 o 18

| Consider if your activity could: | Yes | No | INNS risk issue(s) / observations |
| --- | --- | --- | --- |
| Introduce | Requires | Impact | The NRW coastal risk assessment shows that Anglesey North is at risk from the following INNS: - American Oyster Drill - Leathery Sea Squirt And is at an overall INNS risk of deterioration by 2050. There are no equipment scheduled to be placed directly in the water. Plant machinery will be the primary equipment used to carry out the works. The risk of introduction of marine INNS is low in this instance, however risk should be managed by ensuring the plant are cleaned before leaving the existing / preceding site, and are clean on arrival onto this site. |
| or spread | impact | assessment |  |
| INNS | assessment | not required |  |

Summary of Scoping:

| Receptor | Potential risk to receptor? | Note the risk issue(s) for impact assessment |
| --- | --- | --- |
| Hydromorphology | No | It is concluded that it is unlikely that the proposed work would have any impact on hydromorphology, due to the maintenance and minor improvements nature of the works on the footprint of the existing wall, therefore this receptor has been scoped out from further assessment. |
| Biology: habitats | Yes | There is a Higher Sensitivity Habitat located 26m away from the proposed work. Due to the close proximity the impact could not be ruled out at the scoping stage. |

Tudalen 14 o 18

| Biology: fish | No | Whilst the work is located on the lower reaches of Afon Nodwydd, the work does not involve creating any physical barriers within the watercourse that would impact fish movement. No other works such as piling is scheduled either that would result in noise that would deter fish from migrating up the river. |
| --- | --- | --- |
| Water quality | Yes | Plant and machinery will be required to undertake the proposed activity, and therefore there is an inherent risk of releasing oils and fuels (PAHs) into the environment, albeit, the risk is considered to be low. |
| Protected areas | Yes | The proposed activity is being undertaken on the boundary of a protected area, therefore, potential impact to protected sites cannot be ruled out at scoping stage. Risk / issues for impact assessment include physical change, disturbance and pollution. |
| Invasive non-native species | Yes | Plant and machinery will be used but will not be submerged in water at any given point during the course of the works. However at this stage it cannot be ruled out completely that the plant would be free from INNS, as it is unknown where the plant would be used immediately prior to mobilisation to this site. Risk of introducing INNS to the waterbody cannot be ruled out at scoping stage. |

Assessment of impacts on waterbody quality elements

WFD Compliance

There are three key objectives against which the impacts of proposed works on a water body need to be assessed to determine compliance with the overarching objectives of the WFD:

- Objective 1: The Scheme will not cause deterioration in any element of water body classification.

- Objective 2: The Scheme will not prevent the WFD status objectives from being reached within the water body or other downstream water bodies.

- Objective 3: The Scheme will contribute to the delivery of the relevant WFD objectives. In this case it will be what contribution the Scheme can make towards the water body reaching its objective Good Ecological Potential (GEP) through planned RBMP mitigation measures.

The first two objections must be met to avoid infraction of the WFD. The delivery of the third objective is central to the implementation of the WFD, where it can be supported through its operational activities. If it is considered that the scheme is likely to cause deterioration in water body status or prevent a water body from meeting its ecological objectives then an assessment would be

Tudalen 15 o 18

made against the conditions listed in Article 4.7 of the WFD. Article 4.7 can be invoked if; ‘new modifications’ are of overriding public interest and/or the environmental and social benefits of achieving the WFD objectives are outweighed by the benefits of the new modifications to human health, safety and sustainable development; there are no significantly better environmental options that are technically feasible or not disproportionately costly; and all practicable steps for mitigation have been taken.

Biology: habitats

The scoping exercise identified that the proposed activity is within 26m of a Mussel bed that is a Habitat of Higher Sensitivity. The work is being undertaken on a coastal defence structure, measuring 600m in length and the tidal patterns at Red Wharf Bay provides a potential impact pathway between the works and the Mussel bed. Whilst accepting that there is an impact pathway, the proposed work would not take any land from the Mussel bed or directly disturb it in any way. The proposal would not affect any factors such as inundation of the sea bed (changes in hydromorphology) or the characteristic of the water inundating the Mussel bed, therefore, it is considered unlikely that the proposed activity would contravene Objective 1 for this particular element.

Water Quality

Plant and machinery will be required to undertake the proposed activity, and therefore there is an inherent risk of releasing oils and fuels into the environment, although the risk is considered to be low due to no in water works, the impact on the waterbody could not be completely ruled out at the scoping stage. The following Reasonable Avoidance and Mitigation Measures will be implemented during the course of the proposed work:

- Vehicles to use biodegradable oils.

- Use of spill kits and personnel trained to use the kit will be available on site on case of oil leak.

- Good working practices will be taken, such as adhering to GPP5 Works and maintenance in or near water.

- The contractor has a designated company, which they use to deal will accidental spillages this forms part of their emergency plan.

- The staff will be trained to contain any oil/diesel and clean up.

- NRW emergency help line (0300 065 3000) will be contacted in the case of an emergency spill.

Since the plant and machinery would not directly enter the waterbody, and assuming that machinery used are well maintained, it is considered unlikely that the proposed activity would result in a contravention of Objective 1. However, accidents do happen – and by having the above measures in place it will reduce any impact should an accident occur.

Protected Areas

The proposed work is located within and within 2km of WFD Protected Areas, namely Trwyn Dwlban SSSI, Morwenoliaid Ynys Mon SPA, Bae Lerpwl SPA, Menai Strait and Conwy Bay SAC. Since there are engineering works within 2km of these protected areas, and a potential

Tudalen 16 o 18

impact pathways in the form of tidal action, impact upon these receptors could not be ruled out at scoping stage.

A Habitat Regulations Assessment (CPF6991 Red Wharf Bay Flood Alleviation Scheme Habitat Regulations Assessment– available as a separate document), has been produced for the works. Whilst Likely Significant Effect could not be ruled out in the screening stage (Stage 1 – Test of Likely Significant Effect), due to potential pollution and potential impact on the mobile features of the Morwenoliaid Ynys Môn SPA. A conclusion of No likely Significant Effect was reached at Stage 2 with the implementation of Reasonable Avoidance Measures. Therefore it is considered that the proposed activity will not result in the contravention of Objective 1.

Invasive Non Native Species

The NRW coastal risk assessment shows that Anglesey North is at risk from the following INNS:

- American Oyster Drill

- Leathery Sea Squirt

And is at an overall INNS risk of deterioration by 2050.

There are no equipment scheduled to be placed directly in the water to carry out the works. Plant machinery will be the primary equipment used to carry out the works. The risk of introduction of marine INNS is low in this instance, however risk should be managed by ensuring the plant are cleaned before leaving the existing / preceding site, and are clean on arrival onto this site.

By implementing the simple protocol of Check, Clean and Dry, it is considered that the proposed activity would not result in a contravention of Objective 1.

Conclusion

It is established through the assessment above that by implementing good working practices and reasonable avoidance measures that the proposed works would not lead to deterioration of any quality element of the waterbody (objective 1) nor would prevent the overall objectives for the waterbody being met (Objective 2).

Tudalen 17 o 18

Appendices

Red Wharf Bay General Arrangement (as separate document)

Red Wharf Bay Flood Alleviation Scheme – Method Statement (as separate document)

Tudalen 18 o 18

Appendix A