GD ENVIRONMENTAL SERVICES LTD

Unit 11, East Bank Road, Felnex Industrial Estate, Newport, NP19 4PP

Waste Transfer Station Facility

Site Management System & Working Plan

Permit: EPR/RP3295FS

CONTENTS:

Page No.

1.0 GENERAL CONSIDERATIONS

1.1 Site operator/permit holder 							4

1.2 Site history and assessment 							4

1.3 Planning and waste licensing status 						4

1.4 Permit area/waste management operations					4

1.5 Hours of operation								5

1.6 Waste types and quantities 							5

1.7 Staffing and management 							6

1.8 Health and safety policy 							6

1.9 Fit and proper persons 							6

1.10 Exempt activities 								7

2.0 SITE ENGINEERING AND INFRASTRUCTURE

2.1 Access, parking and site roads 						8

2.2 Notice board and signs 							8

2.3 Site security 								8

2.4 Site office and service buildings 						8

2.5 Weighbridge 								9

2.6 Wheel cleaning facilities 							9

2.7 Fuel/chemical storage 							9

2.8 Waste transfer building 							9

2.9 Storage areas 								10

2.10 Drainage 									10

2.11 Vehicles, plant and equipment 						10

2.12 Sampling and monitoring facilities 						10

3.0 SITE OPERATIONS

3.1 Preliminary procedures 							12

3.2 Checking in & inspection of loads 						12

3.3 Waste deposit 								13

3.4 Waste treatment operations 						13

3.5 Waste product/storage 							13

3.6 Waste/product removal & export 						14

3.7 Sampling and analysis 							14

3.8 Record keeping 								14

3.9 Site diary, manual and visitors book 						15

4.0 ENVIRONMENTAL CONTROL, MONITORING AND REPORTING

4.1 Breakdowns and spillages 							16

4.2 Site inspections and maintenance 						16

4.3 Control of mud and debris 							17

4.4 Dust monitoring and control 						17

4.5 Odour monitoring and control 						18

4.6 Litter monitoring and control 						18

4.7 Pest and vermin control 							18

4.8 Noise monitoring and control 						18

4.9 Environmental monitoring and reporting 					18

5.0 EMERGENCY PROCEDURES

5.1 General 									19

5.2 Fire 										19

5.3 Spillages 									19

5.4 Drums 									19

5.5 Adverse reactions 								20

5.6 High winds 									20

5.7 Poor visibility 								20

5.8 Operational failure 								20

5.9 Overturned vehicle 								20

5.10 Bomb scare 								20

Appendices:

Appendix I - Drawings: Felnex WTS- Yard Layout 001 & Felnex-WTS-EPRRP3295FS_Site Lay out Drawing 002

Document History

| Document Version | Issue Date | Revision Status | Prepared By |
| --- | --- | --- | --- |
| Felnex/WTS/EMS/1.0 | 08/12/2018 | Prepared in line with new ISO & BS Standards. Superseded the previous EMS [Version 2.0] | L. Meredith |
| Felnex/WTS/EMS/2.0 | 13/09/2021 | Added appendices from original permit EAWML30387 into 4.0 ENVIRONMENTAL CONTROL, MONITORING AND REPORTING In line with variation and consolidation of permit | C. Higgins |

1.0 GENERAL CONSIDERATIONS

  - Site operator/permit holder

1.1.1 Environmental permit No. EPR/RP3295FS permits the keeping and treatment of waste at the site which is the subject of this management system. A licence was issued to the site operator, GD Environmental Services Ltd, on 19/09/2006. The site mainly receives waste from a wide range of regular customers from South East Wales and the surrounding area. GD Environmental vehicles also tip off at the site.

1.1.2 Developments in legislation such as the introduction of the landfill tax have increased

the effectiveness and scope of operations for waste transfer and recycling centres, permitting greater recovery rates for recyclable waste. This management system has been drafted to reflect the site operations which have been implemented to reduce the volume of waste which is sent to landfill by the operator. The recycling operations include sorting household, commercial and industrial waste to produce feedstock for operations producing building materials, topsoil, hardcore, aggregate replacements and other products which reduce the need to use virgin products.

1.1.3 The site office address for GD Environmental Services Limited is as follows:

Unit 11 East Bank Road,
Felnex Industrial Estate,
Newport
NP19 4PP

  - Site history and assessment.

1.2.1 The surrounding land use is primarily industrial.

  - Planning & Waste licensing status

1.3.1 The site has the benefit of valid planning permission issued by Newport City Council which was obtained following a successful planning appeal. The conditions have been discharged and the planning permission implemented.

1.4 Permit area/waste management operations.

1.4.1 The area which is the subject of the environmental permit was originally outlined in green on Drawing No. 001. All references to ‘the site’ in this management system shall mean this area, the recycling centre and the associated infrastructure, plant and equipment.

1.4.2 The environmental permit is required for the storage (keeping) prior to removal, and treatment (all types of handling/processing) of waste. Waste treatment processes to be carried out on site include the following:

- Compacting (by loading shovel or grab)

- Sorting (with loading shovel, 360 Excavator with grab/ bucket or by hand)

- Waste sorting machine to segregate various waste types

1.4.3 Specified waste management operations will include waste disposal and waste recovery

operations as detailed in the Environmental Permitting (England and Wales) Regulations 2016. They are listed in summary below:

R2	Recycling or reclamation of organic substances which are not used as solvents.

R3	Recycling or reclamation of metals or metal compounds.

R4	Recycling or reclamation of other inorganic materials.

R13	Storage of waste consisting of materials intended for submission, on this site to any of the category ‘R’ operations.

D9	Physico-chemical treatment of waste not listed elsewhere in this table which results in final compounds or mixtures which are disposed of on this site by means of any of the category ‘D’ operations authorised under this column, or elsewhere than on this site, by means of any of the operations listed in Part III of Schedule 4 of the 1994 Regulations.

D14	Repackaging of waste prior to waste being submitted on this site to any of the category ‘D’ operations authorised under this column, or elsewhere than on this site, by means of any of the operations listed in Part III of Schedule 4 of the 1994 Regulations.

D15	Storage pending category D operations.

The D9 code refers to the sorting activities which result in a residue of waste with no recycling or reuse potential and which must be disposed of. The D15 code refers to the storage of such waste pending its disposal.

1.5.1 The site is operated during the following hours for the receipt of waste and all other

operations:

Monday to Friday 07:00 - 17:00

Saturday 08:00 - 12:00

Sunday Closed

Bank/Public holidays Closed

1.5.2 Floodlights will be used if necessary. Floodlights will be made available if additional lighting is required.

1.6 Waste types and quantities

1.6.1 The waste types accepted at the site are predominantly solid, dry, non-hazardous household, commercial and industrial wastes. The waste delivered to the site is collected from household, commercial and industrial premises. The maximum quantities of all wastes received, stored and processed with the total tonnage accepted per annum totalling a maximum of 74,999 tonnes.

1.6.2 A list of the waste types, consisting of the relevant codes and descriptions from the EWC

Waste Classification is contained within the permit section in the HSEQ Management Folder.

1.6.3 Other wastes which have not been listed but which fall within the category described in 1.6.1 above will be accepted with the written agreement of NRW. No clinical or liquid wastes will be accepted.

1.6.4 If the maximum storage capacity of the site is reached then no further waste will be accepted until waste can be removed from the site and taken to a suitably licensed or exempt site.

1.6.5 Strict requirements will apply to certain types of wastes i.e. Empty used containers which have contained chemicals or special waste must be cleaned or certified as clean before they will be accepted. Paints tins or other containers will be accepted if they have residues in as long as those residues are solid and non-special.

1.6.6 Licensed activities under section 2.1 of the permit

| Description of Activities | Limits of Activities |
| --- | --- |
| D15 R13 D14 | -All bulking of non-hazardous waste must be carried out in areas identified on a plan to be agreed with the agency -Non-hazardous waste must be kept on impermeable pavement with sealed drainage -Inert waste must be kept on hard standing or on impermeable pavement with sealed drainage |
| D9 R2 R3 R4 | -Treatment consisting only of manual sorting, separation, screening, or crushing of waste into different components for disposal [no more than 50 tonnes per day] or recovery. -Non-hazardous waste must be treated on impermeable pavement with sealed drainage -Inert wastes must be treated on hard standing or on impermeable pavement with sealed drainage. |

1.7 Staffing and management

1.7.1 The site will open for the deposit of waste or for other essential operations during the hours listed in Section 1.4 above. The list below details the staff structure of the site when it is operating at full capacity. The minimum staffing requirements when the site is operational are shown in bold italic font in brackets below. Attendance by the site manager is specified by NRW guidance as a minimum of 20% of the site’s operational hours.

Position 			No. 			Responsibilities

Site Manager			 1 			Overall site management/ COTC holder

Machine/plant operator 	5 (1) 			Operating loading plant/site supervision

Office manager/administrator 	1 (1)			Managing site administration

General operatives 		14 (1) 			Sorting waste, maintenance, litter picking

Additional temporary staff will be employed on site during busy periods to carry out site

maintenance works, plant maintenance, administration and record keeping.

1.7.2 Relevant staff will be trained to take samples of the waste for waste stream analysis, to be familiar with operational conditions and to observe potential breaches of permit conditions and understand the action to be taken.

1.8 Health and safety policy

1.8.1 All operations on site will be carried out in accordance with the relevant requirements of the Health and Safety at Work Act 1974 and company SHEQ Management Systems. Conditions of site use for visitors and contractors are briefed by the Site Manager upon entering the site. A document has been prepared for visitors & Contractors.

1.8.2 Fire extinguishers will be kept on site to deal with fires as stated in the emergency procedures in Section 5. A first aid kit will also be kept on site.

1.8.3 Copies of the company’s QHSE policies are posted within the site office.

1.9 Fit and proper persons

1.9.1 Oliver Hazell is the WAMITAB COTC holder (Level 4 High Risk Operator Competence for Managing Physical & Chemical Treatment of Hazardous Waste). The level of COTC (Certificate of Technical Competence) which would be applicable to any incoming mangers at the site is Managing Treatment Operations Level 4 (TSB-4). A manager with TSB-4 would be able to meet the technical competence requirements of permit.

1.9.2 The required managerial cover is 20% of the site’s operational hours. Any changes to the site management will be notified to NRW within 5 working days of the decision to hire the new manager or person providing the cover.

1.9.3 Relevant convictions - at the date of application neither Oliver Hazell nor employees of GD Environmental Ltd have been convicted or formally cautioned in relation to a relevant offence.

1.10 Exempt activities

1.10.1 Activities which are outside the scope of the environmental permit for the site (listed in

Schedule 3 of the Environmental Permitting Regulations 2016) may be carried out at the recycling centre and the relevant details will be registered with the NRW prior to commencement. Where the exemption requires registration with a body other than the Agency a copy of the exemption registration details will be sent to the Agency for information.

2.0 SITE ENGINEERING AND INFRASTRUCTURE

2.1 Access, parking and site roads

2.1.1 Access to the site is gained from the road adjacent to the site. The transfer station compound is fenced off from general public and all gates are locked at the end of the working day.

2.1.2 Parking - sufficient parking space is available on site for 7 vehicles (visitors and office staff).

2.2 Notice board and signs

2.2.1 A notice board displaying the following information is positioned at the entrance to the site:

- The permit holder/ site operator's name and address

- The NRW’s contact details and emergency telephone number

- The site permit number issued by NRW

- Opening hours

- Emergency telephone number for the operator

2.2.2 Additional signs are displayed around the site for operational/health and safety purposes.

2.3 Site security

2.3.1 The recycling centre is fenced off from the other nearby units and has its own lockable gates.

2.4 Site office and service buildings

2.4.1 The site office is located as shown on drawing No. 002

2.4.2 The table below details the relevant site documentation which is kept in the site office.

Documents to be retained in site office

Waste Management Licence

The Working Plan

Site Diary

Weighbridge Tickets

All in-house inspection sheets/recording forms

Duty of care transfer notes [for 2 years minimum]

Hazardous Waste Consignment Notes - Consignor Copies- for 3 years minimum- for waste produced on site & rejected waste - Carrier Copies- for 3 years minimum for loads taken to other sites & rejected waste - Consignee Copies- for incoming waste- retained for the duration of the waste management licence.

Weighbridge tickets

Waste delivery tickets

Accident book

NOTE: Copies of weighbridge tickets, duty of care notes, Hazardous waste consignment notes will be kept at GD head office in Nash, Newport.

2.4.3 The GD head office will be used to store any field testing/sampling equipment which may be

used on site to verify the composition of incoming loads i.e. Sample jars and chain of custody forms.

2.5 Weighbridge

2.5.1 A weighbridge is used to weigh all incoming loads. Outgoing loads will leave via the weigh bridge on the opposite side of the yard [near the weighbridge office].

2.6 Wheel cleaning facilities

2.6.1 The waste types handled and the fact that the main running surfaces are all concreted

reduces the likelihood of mud and debris being carried onto the adjacent road. Additional procedures, which include the use of a mechanical/ vacuum sweeper, will however be put in place if there is a problem to ensure that mud and debris are not tracked onto the industrial estate road and the public highway.

2.7 Fuel/chemical storage

2.7.1 Any fuel or chemical storage tanks and containers which are stored on site will be surrounded by a bund which is capable of containing a minimum of 110% of the volume of liquid stored in the tank. All pipework and associated infrastructure will be enclosed within the bund. A lock will be fitted to tank valves to prevent unauthorised operation. All valves and gauges on the bund will be constructed to prevent damage caused by frost. The tanks and containers will be clearly marked showing the product within and also their capacity.

2.7.2 Any chemicals used on the site are securely stored in the workshop.

2.8 Waste transfer area

2.8.1 The location of the waste transfer, recycling and storage operations are shown on Drawing No. 002.

2.8.2 The waste transfer area consists of several designated areas according to the specific waste streams as detailed on Drawing No. 002.

• Waste which is to be removed off site for further processing is stored segregated

from the unsorted mixed waste imports.

• Waste which is destined for landfill is stored separately from the mixed reception and the sorted waste which is awaiting further processing off site.

2.8.3 The floor of the transfer yard is surfaced with mesh reinforced concrete of sufficient strength for the tipping of loads, storage of waste and running loading plant. Concrete specification: Mix: C35 (minimum 35% cement) Depth: minimum 150mm, fibre reinforced where required.

2.9 Storage areas

2.9.1 Drawing No. 002 shows the following storage areas:

i Bulk waste skips storage areas

ii Plant & equipment storage overnight parking area

iii Product/inert storage bay.

iv Transfer area waste reception and sorting bay.

2.10 Drainage

2.10.1 Surface water from the site passes through a silt trap and interceptor prior to discharging

into to the existing mains surface water sewer.

2.11 Vehicles, plant and equipment

2.11.1 The waste inputs are estimated to be 50 vehicles delivering or removing waste per day.

The estimated peak hourly rate is 10 HGV’s with an average of 5 per day.

2.11.2 Waste will be handled using a 360 Excavator (grab) or wheeled loading shovel equipped with a bucket. The list below also includes other plant/equipment which may be in use at the site. The minimum requirements when the site is operational are shown in bold italic print.

Type of plant/equipment 		No. 		Function

Loading shovel 				1(1) 		Loading/unloading/movement/sorting

300 excavator (grab/bucket) 		1 		loading/unloading/movement/sorting

Water bowser 				1(1) 		dust suppression

2.11.3 All plant will be stored on site and will only be operated by trained personnel.

2.12 Sampling and monitoring facilities

2.12.1 A sealed plastic container will be used for storage of the samples which will be stored in the workshop or site office. Empty containers will be stored in the site office as mentioned in Paragraph 2.4.3 above.

3.0 SITE OPERATIONS

3.1 Preliminary procedures

3.1.1 Guidance is given by the site management to all employees, sub-contractors, other waste carriers and customers regarding the waste types which are acceptable at the site, which is contained within the permit.

3.1.2 Where waste is brought in under sub-contract, or is delivered by other hauliers, the carrier registration details will be taken (for all new haulage operators bringing waste to the site) and details will be periodically checked with the Environment Agency/ NRW to ensure that they are still registered. The procedures below are followed prior to the receipt of waste on site.

3.1.3 When a GD employed driver arrives at a customer site to collect a consignment of waste, (s)he will inspect the load for conformity with relevant regulations and safety procedures.

i. If the load is satisfactory the driver will sign the relevant paperwork and remove the load from the customer’s premises.

ii. If the waste does not meet the description stated on the controlled waste transfer note the customer will be advised to check the note and give a more detailed description of the waste.

iii. If the more detailed description of the waste reveals that the waste is not permitted at the transfer site then the customer will be advised to contact the NRW/Environment Agency to find an alternative site.

iv. The driver may also report back to the site manager for instructions.

v. Where it is suspected that the details given on the transfer note are incorrect the

Environment Agency/NRW may be contacted for advice.

vi. Where the load contains soil from an industrial site the Environment Agency/NRW may be contacted for advice to ensure that the load to be removed does not contain contaminated soil.

3.2 Checking in & inspection of loads

3.2.1 All incoming vehicles are required to report to site supervisor in the weighbridge office

or loading plant operator upon arrival at the site. The details of the load will be recorded and the duty of care note/company documentation will be further checked by the supervisor/operator to ensure that the load is acceptable at the recycling centre. Any deviation from the procedures or problems with any loads will be reported to the site manager.

3.2.2 Once a load has been accepted by the supervisor/operator the driver will be asked to unsheet

the vehicle (if it is sheeted) and a visual inspection of the contents will be carried out to ensure that the waste types comply with the environmental permit. The nature of mixed household, commercial and industrial waste makes full inspection difficult until the load is deposited. If unsuitable waste is discovered before deposit the load will not be tipped and will be rejected by the operator and returned to the producer. In cases where the unauthorised waste is likely to lead to a breach of permit conditions or where the rejected waste is special the Environment Agency/NRW will be contacted.

3.2.3 If the load is acceptable the driver will be instructed to deposit the load in the appropriate

bay/ designated storage container. If the load is unacceptable after deposit it will be loaded back onto the delivery vehicle and returned to the producer.

3.3 Waste deposit

3.3.1 Once a load has been accepted by the supervisor/operator the contents of the delivery vehicles will be discharged into the mixed waste bay for sorting or into the inert waste

storage bay, as shown on Drawing No. 002 and in accordance with the procedures outlined below.

3.4 Waste treatment operations

3.4.1 All mixed loads will be tipped in the waste reception area (mixed waste bay) and crudely

sorted using the 360 (grab) or loading shovel which will then deposit inert waste in the inert waste storage area, non-inert waste but which is suitable for further recycling/reuse in the relevant recycling compound/ container, and finally the waste not suitable for reuse/recycling into the waste for non-recyclable waste area.

3.4.2 Operatives will sort by hand recyclable waste such as cardboard, plastics, wood, building products and metals for recycling. Such waste is dropped into bulk containers or skips to await removal off site.

3.4.3 Any resulting waste which cannot be recycled and therefore destined for landfill will be

stored on the impermeable hardstanding within the reception bay.

3.5 Waste/product storage

3.5.1 The operational outputs and residues produced by the site and the disposal or recovery

routes are detailed as follows:

i Wood - forwarded to chipboard/mdf manufacturer via a third party.

ii Paper and cardboard - delivered to a local paper mill or other relevant receiver.

iii Brick/rubble - to be taken off site for re-use.

iv Fines - exported for use as landfill cover or as soil conditioning material for site restoration works.

v Soils - exported for use as landfill cover or as soil conditioning material for site restoration works.

vi Light waste (non-recyclable) - taken to lined landfill site which is licensed to take all waste arisings from the recycling centre.

3.5.2 The names of the sites and hauliers used have not been listed because of commercial

confidentiality but their details are available to NRW & Environment Agency staff on request.

3.6 Waste/product removal & export

3.6.1 Rejected wastes will be deposited in the quarantine skip. This will occur when non-conforming waste is discovered after the deposit of a load and the producer of the load cannot be contacted (or identified) or where the removal off-site of the waste may cause further problems. NRW will then be contacted to agree a course of action where necessary. The contents of the skip will be recorded in the site diary and no wastes will be placed in the skip which are likely to react with other wastes.

3.6.2 When a collection vehicle arrives at the site the driver will be instructed to report to the

site office or the loading shovel operator. All relevant documentation will be completed, and the vehicle will be passed to pick up the load and take it to the designated recycler/disposal site. The product or waste will be loaded using the loading shovel or 360 machine.

3.6.3 The inert storage area will be used to store recovered materials until removal off site for re-use. Materials will be stored in the designated containers/stockpiles as shown on Drawing No. 002.

3.7 Sampling and analysis

3.7.1 Incoming wastes may be randomly sampled and submitted for analysis at the discretion

of the site manager to verify the composition of a load. Contaminated soils are not permitted on site and the sampling procedure has been implemented to carry out spot checks to comply with the Duty of Care.

3.7.2 Analysis of soil samples will be carried out by an accredited Laboratory.

3.8 Record keeping

3.8.1 The details below will be recorded on a combination of the record keeping forms such as, invoices, weighbridge tickets, the site diary and controlled waste transfer notes (where required). The records will be kept in paper format & managed on the Waste Logistics System.

3.8.2 The following details will be recorded for every load deposited at the site:

(i) The date and time of delivery.

(ii) The name and address of the waste producer.

(iii) The type (including EWC codes) and quantity of waste (in tonnes or

cubic metres).

(iv) The carrier's name and address

(v) Driver’s name, signature and vehicle registration No.

(vi) Signature or initials of person accepting/ inspecting the waste

(vii) Additional handling details/notes made by the driver after inspection of

the load

3.8.3 The following details will be recorded for all deposits of non-conforming waste at the

site and will be forwarded to NRW, where required:

(i) Date and time of deposit.

(ii) A description of the waste (including EWC codes).

(iii) The quantity of waste (in tonnes or cubic metres).

(iv) Name, address and telephone No. of waste producer.

(v) The carrier's name, registration number and vehicle registration.

(vi) Reason for the rejection of waste and action taken.

3.8.4 The following details will be recorded for every load of waste leaving the site:

(i) The date and time of removal.

(ii) The type (including EWC codes) and quantity of waste (in tonnes).

(iii) The destination waste management site or exempt facility.

(iv) The name and registration No. of the carrier or employee removing the

waste (if applicable) and vehicle registration No.

3.8.5 A summary of waste types and quantities deposited at and removed from the site will be

forwarded to NRW at intervals specified in the environmental permit for the site.

3.8.6 The outcome of all inspections of waste types, hardstanding areas, push walls, drainage

channels etc. will be recorded onsite inspection form CF 027 and detailed comments will be entered into the site diary (including action taken or proposed).

3.8.7 Visitors to the site will be requested to sign the visitor’s book upon arrival and exit stating

the purpose of their visit and who they represent.

3.9 Site diary, manual and visitors’ book

3.9.1 The site diary forms only part of the ‘site manual’, which is defined as the documentation system for the recycling centre operation i.e. all documents required by the permit. The site manual includes the following documentation, all of which are stored in the weighbridge office:

Document type

i Site diary/site inspection forms

ii Controlled waste transfer notes

iii Hazardous waste consignment notes

iv Waste input and output records

v NRW inspection forms

vi Completed in-house inspection reports

vii Rejected waste forms

viii Environmental permit

ix Internal Management System

x Planning consent

xi Health and safety documentation

xii Accident records

4.0 ENVIRONMENTAL CONTROL, MONITORING AND REPORTING

4.1 Breakdowns and spillages

4.1.1 In the event of breakdown of the loading plant an alternative loading shovel/ 360 machine will be brought on site until it is repaired. If an alternative machine cannot be used, then no further waste will be accepted until suitable plant is obtained. Minor repairs on plant and machinery will be carried out on site with absorbents used to clear oil or fuel spillages. All other operations on site will continue as normal.

4.1.2 All site surfaces will be inspected daily when the site is in operation. Debris will be swept as required and placed in a skip.

4.1.3. Any spillages of fuel will be cleared immediately by depositing sand or absorbents on the affected area. The sand or absorbents will be placed in a skip to be taken to a suitably licensed site for disposal. All spillages of waste and windblown litter will be cleared by the end of the working day in which they occur. Spillage clearance procedures are detailed in Section 5- Emergency Procedures.

4.1.4 All wastes liable to give rise to contamination will be removed from the site if the site

is not secure or if operations cease or are temporarily suspended.

4.1.5 A drain seal/plug will be used to prevent spillages or fire fighting runoff entering the

drainage system. This will ensure that contaminated drainage does not enter the foul sewer.

4.2 Site inspections and maintenance

4.2.1 The inspection frequencies for maintenance/housekeeping are listed on record form CF 027. The inspection form will be completed by the site manager who is familiar with the requirements of the management system and environmental permit for the site. All details of defects, problems and repairs carried out will be recorded on the form on the day that each event occurs. Detailed comments may also be recorded in the site diary. All repairs will be carried out within 5 working days.

4.2.2 All repairs to site security fencing will be made within 5 working days of the discovery of the damage and the site will be made secure until the repair has been effected.

4.2.3 Any major defects found during the daily site inspection which are likely to lead to a

breach of permit conditions will be repaired by the end of the working day in which they are found where possible. If a repair is not possible by the end of the working day NRW will be contacted to agree a suitable timescale for repair.

4.2.4 All defects and problems likely to give rise to pollution will be recorded on the form CF 027 with repairs/solutions being carried out immediately.

4.2.5 Essential spares for plant maintenance will be kept in the store. If there is a risk that dust will be emitted following a malfunction or breakdown the plant will be shut down for repairs and the waste/stockpiles handled in accordance with the procedures outlined in Section 4.4 below.

4.2.6 All drainage gullies and silt traps will be inspected daily (visual inspection) to ensure that they are functioning effectively. Sediment levels in silt traps will inspected every week and residues will be emptied as required.

4.3 Control of mud and debris

4.3.1 Vehicles will be visually inspected before exit to check that loads are safe and that no mud is carried out on the wheels or body of the vehicle. Visual inspections of the site surface are carried out daily (CF 027) and staff will report any problems with mud on the site surface immediately to the site manager.

4.3.2 The deposit of material on to the highway will be treated as an emergency and will be cleared before the end of the working day by a vacuum/road sweeper. Silt will not be washed into roadside drains or gullies. A sweeper will also be hired in as necessary to ensure that hard surfaced areas of the site are free from mud and debris.

4.3.3 A water hose is available to spray loads/stockpiles/site surfaces if excessive dust is being

produced. All loads contained potentially loose material will be sheeted or sprayed with water prior to leaving the site to ensure that they comply with the requirements of the Duty of Care.

4.4 Dust monitoring and control

4.4.1 Water Supply - A constant supply of mains water will be available for dust suppression in all climatic conditions. All external water pipes will be lagged to prevent frost damage during Winter months.

4.4.2 Water hoses for use in the control of dust emissions are available on site.

4.4.3 All site operations will be carried out to minimise the creation of dust. Formal inspections for the appearance of dust at the site boundary will take place twice daily and the findings and any actions taken will be recorded on record form CF 027.

4.4.4 Operatives will constantly monitor all plant for the presence of visible dust emissions.

If a problem arises the operator will shut down the vehicle, isolate the source of the emission and repair the vehicle if required.

4.4.5 Sheeting of vehicles - Vehicles carrying potentially dusty loads off site will be securely sheeted before leaving the site. Loads will be sprayed with water prior to sheeting if necessary.

| Standards for monitoring and control of aerial emissions of dusts, fibres and particulates |  |
| --- | --- |
| - Monitoring of aerial emissions | Site staff supervising individual waste handling operations shall, during the carrying out of those operations, undertake visual monitoring of aerial emissions. |
| - Storage requirement | Solid wastes which when handled or stored are likely to generate significant quantities of dusts, fibres or particulates only permitted if they are handled and stored either in: - Buildings or containers providing containment of aerial emissions of dusts, particulates; or - Bays or roofed areas provided with a permanent water supply and water spraying or misting equipment and with an impermeable pavement and a sealed drainage system; and the water spraying or misting equipment is used at all times when significant quantities of dusts, fibres or particulates are likely to be, or are being generated. |
| - Remedial action | - On detection or notification of visible aerial emissions that are likely to be transported beyond the site boundary, immediate action shall be taken to stop the waste handling operations giving rise to the emission and to suppress the aerial emission from the waste. - The incident and the remedial action shall be recorded in the site diary. |

4.5 Odour monitoring and control

4.5.1 The containment of rejected waste within an enclosed skip and the range of waste types accepted at the site present a very low risk of odour nuisance. If malodorous waste is deposited on site it will be consigned to the quarantine skip and removed from the site immediately.

| Standards for monitoring and control of emissions of odours |  |
| --- | --- |
| - Monitoring of odorous emissions | Olfactory monitoring of aerial emissions from the site shall be carried out: - By the site manager or supervisor, at least twice per day, t the site boundary situated downwind of the waste operations and shall be recorded in the site diary; and - By site staff supervising individual waste handling operations, during the carrying out of those operations. |
| - Storage requirement | - Odorous wastes, including wastes which are likely to be odour producing during storage only permitted if: - received in sealed containers and stored in sealed containers and in areas provided with impermeable pavement and sealed drainage; or - stored in covered buildings providing containment of aerial emissions; or - stored in bays provided with an impermeable pavement and sealed drainage. - These wastes shall not be stored for longer than 48 hours, unless otherwise agreed in writing with Natural Resources Wales. |
| - Odorous emissions action plan | - On detection or notification of aerial emissions of odour that are or are likely to be transported beyond the site boundary at such levels that they are likely to cause pollution of the environment or harm to human health or serious detriment to the amenity of the locality, immediate action shall be taken to stop the waste handling operations giving rise to the emission and to suppress the aerial emission from the waste. - The incident and the remedial action shall be recorded in the site diary. |

4.6 Litter monitoring and control

4.6.1 The most likely sources of litter on site are from the tipping of light loads. The bays and

the building significantly reduce the effects of wind. Daily inspections will be carried out for the presence of windblown litter and operatives will be instructed to collect the litter, place it in a skip for disposal/recovery before the end of the working day.

4.7 Pest and vermin monitoring and control

4.7.1 It is unlikely that vermin will present a problem because of the waste types handled at the site. The site will be inspected daily for the presence of vermin and the results of the inspection noted in the site diary or site inspection form. Should the presence of vermin be detected then arrangements will be made with a suitable pest control contractor to provide appropriate remedies.

| Standards for monitoring and control of pest infestations |  |
| --- | --- |
| - Monitoring of pest infestations | An inspection of stored wastes for pest infestations shall be carried out at least at weekly intervals by the site supervisor and shall be recorded in the site diary. |
| - Storage requirement | Wastes which are likely to attract pests shall not be stored for longer than 48 hours, unless otherwise agreed in writing with Natural Resources Wales. |
| - Pest infestation action plan | - On detection or notification of pest infestations, immediate action shall be taken to secure the attendance of a professional pest control contractor, to eliminate the pest infestation. - The incident and remedial action shall be recorded in the site diary. |

| Standards for monitoring and control of scavenging birds and other scavengers |  |
| --- | --- |
| - Monitoring of scavengers | Stored wastes which are likely to attract scavengers shall be routinely monitored for the presence of scavenging animals or flocks of scavenging birds, throughout the working day by the site supervisor. |
| - Storage requirement | Wastes which are likely to attract scavengers are only permitted if: - Stored in closed or secure containers - Stored in covered buildings providing security against scavengers, or - Stored in bays provided with netting or fencing providing security against scavengers. |
| - Scavengers action plan | - On detection or notification of scavenging animals or flocks of scavenging birds, immediate action shall be taken to: - Remove or deter them from site, and - Isolate and secure the wastes attracting the scavengers against further scavenging - The incident and the remedial action shall be recorded in the site diary. |

4.8 Noise monitoring and control

4.8.1 It is not anticipated that site operations will be cause a noise nuisance or vibration problem because of the scale, location and plant type used on site of the operation. The Best Practicable Means (BPM) will be employed on site at all times to ensure that all plant and equipment does not produce excessive noise beyond the site boundary.

4.9 Environmental monitoring and reporting

4.9.1 Surface water - no surface water monitoring is required at present.

4.9.2 Ground water - no groundwater monitoring is required at present.

4.9.3 Waste - loads containing soils may be sampled as described in Section 3.7 above. A copy of any analysis results will be kept in the site office and will be available for inspection by NRW staff.

4.9.4 Complaints - all complaints received, and action taken in respect of those complaints will

be recorded in the site diary and in line with the Company’s complaints procedure.

5.0 EMERGENCY PROCEDURES

5.1 General

5.1.1 In addition to obligations imposed by RIDDOR (Reporting of Injuries, Diseases and Dangerous Occurrences Regulations) the operator will notify the NRW of any serious injuries to employees of GD Environmental Limited, other site users or members of the public arising as a result of operations on site. Minor injuries such as cuts and grazes will be recorded in the accident book on site. Separate procedures will be used for different types of emergency. An emergency at the site is defined by the site management as follows: “Any incident likely to result in harm to human health or pollution of the environment or serious breach of permit conditions and serious detriment to the amenities of the locality.”

5.1.2 For all emergency situations the deposit of any further waste will be suspended where necessary to allow action to be taken safely. If necessary staff and other users of the site will be evacuated to an area which is a safe distance away from the hazards. Staff handling the emergency will be provided with and trained to use the necessary PPE (personal protective equipment) unless the manager instructs them that the hazard is too severe and outside help is needed from the emergency services or specialist waste contractors.

5.2 Fire

5.2.1 No waste will be burnt on site. In the event of a fire occurring on site the operator will exercise his judgement and extinguish the fire with a suitable fire extinguisher and/or call the fire service for assistance. Any fires will be reported to NRW on the working day that they occur and will be confirmed in writing by letter within 3 working days. All persons will be evacuated from the site if necessary. Smoking is not permitted on site. Firefighting residues will be disposed of to a licensed waste management facility.

5.3 Spillages

5.3.1 All fuel and chemical stores on site are bunded to contain any fuel leaks. If any spills occur a spill containment kit (absorbent pads, booms or granules) will be used to prevent further spillage and the contaminated absorbents placed in a skip for disposal to a suitably licensed landfill. Any hardstanding areas contaminated by a spillage will have the contaminated ground removed to a suitable licensed facility and replaced by clean uncontaminated materials.

5.4 Drums

The deposit of drummed waste is not permitted at the site. If a drum is concealed within a skip

and is not observed until the skip is emptied on the hardstanding area then the following procedure will apply:

i. The staff member will visually check the condition of the drum from a safe distance, noting any labels referring to the possible contents or hazards.

ii. The site manager will be contacted to verify the observations and to decide on further action.

iii. The producer of the waste and the NRW will be contacted for advice and further information if necessary and both will be informed that a breach of the Duty of Care and permit conditions has occurred as the result of the unauthorised deposit.

iv. No further waste will be deposited until the emergency has been dealt with.

v. All spillages will be cleared using a spill containment kit and all contaminated absorbents placed in a skip for disposal to a suitably licensed waste management site.

vi. If the deposit results in serious reactions with other waste or harmful emissions or the drum contents cannot be identified then the emergency services and/or specialist waste contractors brought in to assist. If necessary staff will be evacuated from the site or to a safe area within the site.

5.5 Adverse reactions

5.5.1 No wastes are accepted which will react to present such a hazard. If unauthorised waste is found in a skip and does present such a hazard the same procedures as for the deposit of drums (above) shall apply.

5.6 High winds

5.6.1 The transfer building provides sufficient containment to prevent light wastes from blowing off site in high winds. Vehicles leaving the site will be sheeted to comply with the requirements of the Duty of Care legislation.

5.7 Poor visibility

5.7.1 The site will not operate in conditions of poor visibility such as dense fog to reduce the risk of vehicle collision or other accidents.

5.8 Operational failure

5.8.1 The manager will be contacted in the event of any operational failure such as the breakdown of plant, systems or equipment and will decide whether operations are to continue or be suspended prior to corrective action being taken. Serious operational failures which result in the closure of the site will be recorded in the site diary.

5.9 Overturned vehicle

5.9.1 If a vehicle is overturned on the site or near the site entrance then no further waste will be accepted until the vehicle has been righted and any spillages have been cleared as described in Section 5.3.

5.10 Bomb Scare

5.10.1 In the unlikely event of a bomb scare the site will be evacuated and the police contacted.

The police will then assume control of the site until the threat has been verified or the device defused and removed.