| Details |  |  |
| --- | --- | --- |
| Deployment Reference Number: | PAN-015124 |  |
| Permit Reference Number: | EPR-GP3792SK |  |
| Permit Holder: | Byproduct Recovery Limited |  |
| Site Address: | Cassandra Farm, Caerleon, NP18 1LR |  |
| Grid Reference: | Farm location: ST 34327 92967 |  |
|  | Fields: 1 ST3526191855 2 ST3562791831 3 ST3550991756 4 ST3532191674 5 ST3516691675 6 ST3501591765 7 ST3510391921 8 ST3476593095 9 ST3449592897 10 ST3457393104 11 ST3413392969 13 ST3497591581 15 ST3458991431 16 ST3447491560 20 ST3456392561 22 ST3472492149 |  |
|  | Storage (in original application) 1 (A) ST3521791950 2 (B) ST3542291845 3 (C) ST3516091895 4 (D) ST3508991775 5 (E) ST3497791865 6 (F) ST3502591898- grid reference/map inconsistency 7 (G) ST3462591751 8 (H) ST3446191335 9 (I) ST3474092214 10 (J) ST3468692222 11 (K) ST3457992668 12 (L) ST3433292930- not on place of spreading 13 (M) ST3400292992 14 (N) ST3439592968 15 (O) ST3484793159 | After RFI: A 335217,191820 B 335442,191853 C 335156,191938 D 335069,191675 E 334926,191829 F 335090,191875 G 334537,191739 H 334435,191328 I 334765,192093 J 334670,192194 K 334585,192556 L 334376,192950 M 334784,193065 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: SE.waste.queries@cyfoethnaturiolcymru.gov.uk Environment team: ET.NewportCaerphilly&BlaenauGwent@cyfoethnaturiolcymru.gov.uk NLO: Ewan.Marshall-Harries@environment-agency.gov.uk |
| Date due: | 16/09/2021 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | English- EA public register |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | Y | Amount: £1,018 Date cleared on MyNRW: 06/08/2021 Transaction number: 2092352 BS/060821/13 BYPRODUCT RECOVERT LIMITED PSCAPPBYPRO00986 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y | Some out of date |
| EIA Agri Regulations evidence | N/A |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | Y | Previous deployment: PAN-011455 |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y | Emily Marr who has now left company. Contact is Chris Ash/Paul Whyatt |
| ## Has the technically competent manager details been provided? | Y | Mr Richard Evans |
| ## Has the nominated competent person details been provided? | Y | Mr Richard Evans |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | N | On LPD1 form: 60/034/0021 1-11 and 13 match. No CPH on our systems for field 15. Different CPH number for fields 16, 20 and 22. CPH number log at the end of this document. See RFI RESOLVED 13/10/21 |
| ## Has all of parcels of land table been completed? | Y | On sperate document: 1.1 LPD1 Supplement. |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 49.2 |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the landowner or occupier of the land you want to spread on/treat? | N | Given Mr James Mayho of Cassandra Farm for all fields. 1-11 and 13 match. No landowner or occupier on our systems for field 15. Different landowner or occupier number for fields 16, 20 and 22. Landowner or occupier log at the end of this document. See RFI RESOLVED 13/10/21 |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has consent been given to carry out the activity? | Y |  |
| Has all of the previous land treatment table been completed? | Y | On separate document: 1.1 LPD1 Supplement Previouslandtreatment. PAN-011455 valid until 06/10/2021 |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | Y |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N/A | Not required until Nov 2021 |
| If yes has the screening opinion provided by Welsh Government been provided? | N/A |  |
| If no have details of the assessment evidence to support this conclusion been provided? | N/A |  |
| Has all of the waste types table been completed? | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| Has all of waste storage table been completed? | Y | See 1.1 LPD1 Supplement waste storage and ABS storage sections. |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y | Each field heap is limited to 3000 t and confirmed that “No more than 3,000t to be stored at any one time across all storage areas”. Liquid waste is not to be stored under this deployment. |
| Has checklist been completed? | Y |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y | Said “no” but should be yes. Email received 09/07/21 covers Dr Chris Ash, Dr Emily Marr, Paul Whyatt, Dr Becky Wheeler |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | Y | Getting declaration reconfirmed. See RFI in RFI RESOLVED 13/10/21 |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | N | See comments. RESOLVED 13/10/21 |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | Same | All field heaps are identical to PAN-011455, therefore they have to be moved. See RFI RESOLVED 13/10/21 |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | N | 2 water courses in Field 8 with no buffers. All others are buffered RESOLVED 13/10/21 |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | n/a | None found |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | N | Spring at SW corner of field 16 No wells, boreholes or abstractions found within distance RESOLVED 13/10/21 |
| location of all storage sites for the waste to be spread | Y | Location 6 doesn’t match map; however, all need to be relocated RESOLVED 13/10/21 |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | N | Spring at SW corner of field 16 No wells, boreholes or abstractions found within distance RESOLVED 13/10/21 |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | N | Spring at SW corner of field 16 No wells, boreholes or abstractions found within distance RESOLVED 13/10/21 |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | River Usk SAC and SSSI. No SPA or Ramsar within distance |
| location of any public rights of way (shown on map) | Y | Dotted line of map |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y N n/a | Lowland dry acid grassland and Lowland calcareous grassland- fields 3, 13. No action required. Eel, Silver Eel, Yellow Eel, Grisle, Springer in water courses alongside fields 8, 9, 10, 13, 16, 20, 22. No spread zones in place. Marine Laprey, Lampern, May Fish in water courses alongside fields 13, 16 and in the River Usk. No spread zones in place. |
| What is the slope of the fields? Slope Map tool document | 0-7 degrees (transparent)- all fields 7 -12 degrees (yellow)- fields 1, 5, 7, 6, 10, 15, 16, 20, 22 12 degrees plus (red)- fields 6, 11, 15, 16 See comment section for assessment |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 190902- Iron and Aluminium 190606- pathogens |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | N | See RFI RESOLVED 13/10/21 |
| Does the analysis include the following? |  |  |
| - pH | N | See RFI RESOLVED 13/10/21 |
| - Potassium (K) | N |  |
| - Phosphorus (P) | N |  |
| - Magnesium (Mg) | N |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | N | See RFI |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | Mainly P and K. Mg levels are at optimum |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | N | DCWW Pontsticill- cake- arsenic levels too high at max app rate RESOLVED 13/10/21 |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. No likely significant effect due to the following standard practices: - Spreading will only take place subject to ground conditions and following the Code of Good Agricultural Practice (Defra, 2011), NVZ regulations and the permit holder’s Environmental Management System (EMS). - Targeted periods of spreading on grass fields include spring (late Feb – Apr), and after cutting of silage through summer and autumn. - Spring Barley – the waste will be spread onto the seedbed prior to drilling in the spring (between March-April). - No more than 50t/ha of liquid sludge will be spread on a field in any 3-week period in accordance with CoGAP, and no more than 250t/ha will be spread within any 12-month period. - The stockpile locations are accessible by delivering vehicle, near field entrances so the potential damage to fields by delivering vehicles is minimal. - The stockpiles are not within 10m of any ditch, watercourse, or footpath, not within a SPZ1, and are at least 50m from any well spring or borehole. They are also a safe distance from overhead powerlines. - The cakes will be spread using conventional rear discharge spreaders as this equipment is readily available to the farmer/contractor and the most appropriate for the material and rates it is to be applied at. - Liquid sludges will be surface spread by tractor and tanker using a trailing hose/shoe or dribble bar. - The wastes may be applied separately or in combination. If the wastes are applied in combination the total combined amount applied will not exceed 250 t/ha, the total nitrogen loading will be less than 250 kg/ha, and the amount of available nitrogen and total or available phosphate and potash (whichever is appropriate) will not exceed the fertiliser recommendation or the amount removed in crop offtake, whichever is the greater. - Apply according to PQA, RB209 and Soil Code. Comply with NVZ, Cross Compliance, Sludge Regs and EPR. - No spreading areas to be observed as per attached plans. - Apply only in suitable conditions. - WTW sludge has low concentrations of PTEs. Some WTW sludges contain aluminium but solubility low at observed soil pHs. Alum sludge will not be spread on fields with soil pH <6 - Delivery and spreading to be undertaken when ground conditions are suitable - The WTW sludge has minimal odour. Odour management plan available in EMS in accordance with SR2010No4 permit - Waste streams have low potential to produce airborne dust and particulate matter. Waste will be applied in accordance with CoGAP and EMS - Waste does not contain litter as it derives from a controlled manufacturing processes. Waste will be applied according to Codes of Good Agricultural Practice and SR2010No4 EMS. - The WTW sludge is highly unlikely to attract scavenging animals. - Sludge has low potential to attract flies. All waste will be stored, transported and spread in accordance with conditions set in SR2010No4 permit and CoGAP. Wastes are unlikely to attract pests as WTW sludge is not food based - Professional contractors employed instructed to take care around trees. Leave a 2m minimum buffer zone adjacent to trees and hedgerows - Proximity of protected site. SAC borders parts of fields 2 and 13 - Particularly sensitive nature of site (fish and plant species) - Waste streams have low potential to produce airborne dust and particulate matter - Agricultural machinery in agricultural areas - Delivery to storage area is >250m away from designated area Apply according to PQA - 20m non-spreading buffer zone applied to where SSSI borders fields and to watercourses that run directly into SSSI - Avoid sensitive spreading periods e.g. breeding season - No activity to take place within designated areas |  |

Comments

Consultation responses: - info on CPH numbers (see RFI) - identified River Usk SSSI (See Appendix 4 assessment and Habitat Regulation Assessment sections of this document)

MyMap and location assessment: See location section and RFI

Protection of habitats and ecosystems n/a

Agricultural benefit calculations: See nutrient calculator and RFI

Request for information sent 28/09/21 with response deadline of 13/10/21. email saved to DMS: - CPH number - consent - previous deployment - declaration - waste storage - no spread areas - field slopes - soil analysis - PTE - ABS- contingency

RFI Response 12/10/21 - resolved point 4- declaration

RFI Response 13/10/21 - point 1: Relevant info provided. - point 2 Signed consent provided for fields 15, 16, 20 and 22 - point 3 Confirmed, no activity will take place on the deployment PAN-015124 until the deployment has been agreed with NRW and issued. - point 5 storage locations moved. - point 6 one of the two watercourses in field 8 have been buffered. Spring in field 16 has been buffered. On review with aerial images. second watercourse on field 8 does not require buffer. - point 7 field 11 removed entirely, sections of fields 15/16 removed - point 8 analysis provided for fields 20/22. Waiting for PTEs - point 9 app rate reduced to 80 t/ha - point10 requested confirmation provided

RFI Response 15/10/21 Point 8 (PTEs) resolved

Other information to note: Andigestion is a high RAN waste, however storage restrictions are not relevant as this waste is not being stored under the deployment. Original field storage grid references: not included in RFI as all locations were requested to be revised: - grid ref for 6 (F) ST3502591898 doesn’t match the map - storage 12 (L) ST3433292930 is not within a spreading area and is actually located within a no spreading zone. Please remove this from your maps and the list. Slope assessment: 0-7 degrees (transparent)- all fields 7 -12 degrees (yellow)- fields 1, 5, 7, 6, 10, 15, 16, 20, 22 12 degrees plus (red)- fields 6, 11, 15, 16 Risk factor assessment: - Spreading will only take place subject to ground conditions and following the Code of Good Agricultural Practice (Defra, 2011), NVZ regulations and the permit holder’s Environmental Management System (EMS). - Targeted periods of spreading on grass fields include spring (late Feb – Apr), and after cutting of silage through summer and autumn. - Spring Barley – the waste will be spread onto the seedbed prior to drilling in the spring (between March-April). - No more than 50t/ha of liquid sludge will be spread on a field in any 3-week period in accordance with CoGAP, and no more than 250t/ha will be spread within any 12-month period. - only stackable waste stored in field heaps - The cakes will be spread using conventional rear discharge spreaders as this equipment is readily available to the farmer/contractor and the most appropriate for the material and rates it is to be applied at. - Liquid sludges will be surface spread by tractor and tanker using a trailing hose/shoe or dribble bar - Medium soils - Spreading will only be undertaken when weather conditions are suitable within restrictions outlined in CoGAP and any relevant closed periods. - Spreading will not be carried out in any areas of a field that will be sub-soiled. - Machinery operations will take account of soil conditions, slopes etc. - Liquid spreading machinery will be turned off and lifted away from soil prior to turning at the end of each run. - Machinery will be checked daily when in use, regularly serviced and spreading equipment calibrated. Umbilical hoses will be regularly checked for damage to prevent leaks. - Machinery turns will not be executed in the buffer strips. - Waste deliveries to field/stores will be supervised. - All spillages will be reported immediately to NRW. Requested red areas in fields 11, 15 and 16 are removed. CPH and consent checks: 1 60/034/0021 MAYHO FARMS CASSANDRA FARM 2 60/034/0021 MAYHO FARMS CASSANDRA FARM 3 60/034/0021 MAYHO FARMS CASSANDRA FARM 4 60/034/0021 MAYHO FARMS CASSANDRA FARM 5 60/034/0021 MAYHO FARMS CASSANDRA FARM 6 60/034/0021 MAYHO FARMS CASSANDRA FARM 7 60/034/0021 MAYHO FARMS CASSANDRA FARM 8 60/034/0021 MAYHO FARMS CASSANDRA FARM 9 60/034/0021 MAYHO FARMS CASSANDRA FARM 10 60/034/0021 MAYHO FARMS CASSANDRA FARM 11 60/034/0021 MAYHO FARMS CASSANDRA FARM 13 60/034/0021 MAYHO FARMS CASSANDRA FARM 15 No data 16 60/144/0005- S L JONES- TREVELLA FARM 20 60/034/0014- W AND M MCDONALD PENCARN FARM LTD- SHAFTSBURY FARM 22 60/034/0014 W AND M MCDONALD PENCARN FARM LTD- SHAFTSBURY FARM Previous deployments on the same fields: 1 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 2 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 3 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 4 PAN-000100, PAN-001928, PAN-003070, PAN-011455 5 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 6 PAN-000100, PAN-001928, PAN-003070, PAN-011455 7 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 8 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 9 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 10 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 11 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 13 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 15 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 16 PAN-000100, PAN-001928, PAN-003070, PAN-006264, PAN-011455 20 PAN-000547, PAN-001928, PAN-003070, PAN-006264, PAN-011455 22 PAN-000547, PAN-001928, PAN-003070, PAN-006264, PAN-011455

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 18/10/2021 |
| ## Effective date | 18/10/2021 |
| ## Permitting Officer Date | Louise Bailey 18/10/2021 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 18/10/2021 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 640 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 720 minutes |