| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-015154 |
| Permit Reference Number: | EPR/AB3891CX |
| Permit Holder: | Mr Daniel & Mrs Carys James |
| Site Address: | Hafod Farm, Ferwig, Cardigan, Ceredigion, SA43 1PU |
| Grid Reference: | Farm location: SN 18094 50342 Fields: 5505 SN 16551 51051 4885 SN 16480 50858 Daries 2056 SN 16203 50569 Tawyn 0834 SN 16088 50345 Tawyn 5928 SN 16598 50289 Tawyn 3370 SN 16338 49708 Dun 2 SN 16983 50759 Heolgwyddil 1 SN 17767 49425 Heolgwyddil 2 SN 17531 49496 Tawyn 3234 SN 16327 50327 Storage: SN 17956 50061 above ground storage tank-contingency SN 17052 50301 field nurse tank SN 17791 49535 field nurse tank |

Field nurse tank

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: CeredigionEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: ffion.quan@cyfoethnaturiolcymru.gov.uk |
| Date due: | 17/09/2021 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- DMS/PaLS No variations |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | Y | Amount: £1018.00 Date cleared on MyNRW: 07/10/2021 Transaction number: 2094186 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | N | Not supplied or required at time of application |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | EPR/AB3891CX |
| ## Has the permit holder name been provided? | Y | Mr Daniel & Mrs Carys James |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | N Y | Tbc Information rec’d 27/10 |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? MyMap notes the following: (2020) Dun 2 CPH 55/226/0030 L J Jenkins and Partners T3379 55/226/0027 MRS J C DAVIES AND SON D2056 55/226/0030 L J Jenkins and Partners H2 55/226/0027 MRS J C DAVIES AND SON | Y | 55/226/0027 Updated information rec’d 27/10, accepted as most recent and accurate -see e-mail in DMS |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 49.7ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | Mr Morris Davies Updated information rec’d 27/10, accepted as most recent and accurate -see e-mail in DMS |
| ## Has consent been given to carry out the activity? | Y | Tbc-see RFI Updated information rec’d 27/10, accepted as most recent and accurate -see e-mail in DMS |
| Has all of the previous land treatment table been completed? | Y | PAN-011209 Expiry date 20/09/2021 |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y | High risk location and list B wastes |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N | Evidence not supplied or required at time of application |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | N | Evidence not supplied or required at time of application |
| Has all of the waste types table been completed? | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Google |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| Has checklist been completed? | Y |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y | 10m Non-spreading buffer zones in place |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring(or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | 50m non-spreading buffer zone in place |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | Cardigan Bay SAC adjacent to 5505, D2056 & T0834; West Wales Marine SAC close to T3370, adjacent to 5505, D2056 & T0834. Aberarth-Carreg Wylan & River Teifi SSSI’s in close proximity |
| location of any public rights of way (shown on map) | Y | Dissecting Heolgwyddil 1 |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y some not all | PS- Threatened Lichen close to T0834; T0834 close to migratory route for European eel and Atlantic salmon PH- Lowland dry acid grassland and Lowland calcareous grassland adjacent to 4885, close to D 2056; Purple moor grass and rush pastures between 4885 & Dun 2, 72m from D2056, adjacent to T 5928 ; Reedbeds 72m from D2056 at closest point; SAM- Craig y Gwbert Defended Enclosure 23m from T0834 at closest point |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields #5505, 4885, Daries 2056,Tawyn 0834, Tawyn 5928,Dun 2, Heolgwyddil 1, Heolgwyddil 2, 7 -12 degrees (yellow)- fields #T 3234-eastern border, areas to North and South of T 5928, east and south of T3370 Non-spreading buffer zones/further justification to be requested by RFI to mitigate run-off risk Updated information rec’d 27/10, further justification and mitigation measures in place to address potential risk assessed and considered satisfactory. |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y | Sept 2020 & July 21 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | N |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y | April 2021 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | Y |  |
| - Stated the source of information that they used to set the nutrient level. | Y |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | n/a | Statement given confirming wastes will not be mixed |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N | App 4 Saved for information only |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. Although there is potential for impact of the proposed activities, due to the conditions of the SR2010 No.4 under which this waste is being deployed i.e non-spreading buffer zones of 10m around any watercourse and 50m surrounding any well, spring or borehole used for the supply of water consumption including from a private water supply or supplies, the potential impact pathway is reduced. The following measures the applicant intends to put in place will further minimise risks. These are : Ensuring that appropriate application rates are adhered to in accordance with the Agricultural Benefit Statement provided and nutrient calculations based on RB209 ; Non spreading areas to be observed as listed in site-specific risk assessment and location plans ; Spread only when weather conditions are suitable i.e not during periods of heavy rainfall, icy conditions etc.; Spreading done by sub surface injection system for grassland, thereby minimising risk of odour and optimising nutrient availability to the medium soil type of the fields ; On arable fields waste will be applied and incorporated into the soil with cultivations or surface applied with a dribble bar into the standing crop ; Wastes will be applied in accordance with CoGAP and EMS ; Operations will not cause any additional effects on surrounding roads due to the existing agricultural nature of the area and current normal agricultural practice occurring in the surroundings ; Ensure that the correct spreading equipment is used with professional drivers , who are to be made aware of any site specific guidelines ; Wastes have a low potential to produce airborne dust and particulate matter, good practice operating systems to be followed further minimises airborne risks. |  |

Comments

Consultation responses: - waste reg officer concerned that slurry stores might be used for storage when not empty, thereby contaminating waste-clarification provided for related deployments, tbc in RFI Updated information rec’d 27/10, confirmation given that storage locations will not contain slurry and no mixing of wastes will occur

MyMap and location assessment: Saved under projects PAN-015154, copies saved to DMS

Protection of habitats and ecosystems No evidence supplied or required at time of application

Agricultural benefit calculations: Closely align with our own, slight differences for rounding but within acceptable margins of error/difference

Request for information: RFI sent 15/10 re points highlighted above, deadline for response 29/10 -see DMS Response addressing all points received 27/10 -RFI discharged. E-mail saved to DMS

Other information to note: MyMap notes the following: (2020) Dun 2 CPH 55/226/0030 L J Jenkins and Partners T3379 55/226/0027 MRS J C DAVIES AND SON D2056 55/226/0030 L J Jenkins and Partners H2 55/226/0027 MRS J C DAVIES AND SON Updated information received 27/10 confirming ownership and tenancy agreement in place for fields within deployment, addressing CPH and consents-see e-mail in DMS

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 01/11/2021 |
| ## Effective date | 01/11/2021 |
| ## Permitting Officer Date | Judith Green 29/10/2021 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 01/11/2021 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 975 to 13.30 29/10 |
| Sign off | 10 |
| Issuing | 60 |
| Total | 1105 |