Please note – this document is purely to assist you in making your decision and may not necessarily cover all information you require.

| Deployment Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-015198 |
| Permit Reference Number: | EPR/QP3293FY/V004 |
| Permit Holder: | Vertase FLI Limited |
| Site Address: | Phase 4a and 5, Glan Llyn Former Coking Works, Llanwern, Newport, NP19 4QZ |
| Grid Reference: | 336187,187042 |

| Administrative Checks- By admin officer |  |  |
| --- | --- | --- |
|  | Comments |  |
| Consultation | Waste reg team: SE.waste.queries@cyfoethnaturiolcymru.gov.uk Environment team: ET.NewportCaerphilly&BlaenauGwent@cyfoethnaturiolcymru.gov.uk NLO: Wessex.waste@environment-agency.gov.uk Land/groundwater remediation only: Geoscience: geoscience@cyfoethnaturiolcymru.gov.uk Local Authority Contaminated Land: environmental.health@blaenau-gwent.gov.uk; info@newport.gov.uk |  |
| Date due: | 21/09/2021 |  |
| Permit compliance rating and multiplier |  | Rating A- 100% |

| Administrative Checks- By determining officer |  |  |
| --- | --- | --- |
|  | Action completed or document provided? (Yes, No or n/a) | Comments |
| Correct fee? | Y | See permit compliance rating and multiplier |
| Original fee received and cleared | Y | Amount: £2,541.00 Date cleared on MyNRW: 02/09/2021 Transaction number:2093389 Reference: BS02092021/33 BR VERTASE F L I LTD |
| Additional fee received and cleared | n/a |  |
| MyMap report completed? | Y |  |
| Location map | Y | D4583_06A Site Layout.pdf D1910_13B Location of Reen's & Celtic Treatment Area.pdf D4583_09 Designated Areas.pdf Remediation Working Plan Llanwern August 2021 (Final with Apps).pdf |
| Site Specific Risk Assessment | Y | Remediation Working Plan Llanwern August 2021 (Final with Apps).pdf |

Word versions of online forms - NRW View (sharepoint.com)

| # About your permit | Y N n/a | Comments |
| --- | --- | --- |
| # What permit is being deployed under? | # Bespoke permit for treatment using mobile plant |  |
| # Has the permit number been given? | # Y | # EPR/QP3293FY/V004 |
| # Is the base permit still live? | # Y | # EA public register checked |
| # Has there been any variations | # Y | # All copies saved onto DMS. V004 was an updated and consolidated permit. |
| # Has name and address of permit holder been provided? | # Y |  |
| # Has contact information been provided? | # Y |  |

| # Deployment details | Y N n/a | Comments |
| --- | --- | --- |
| # Have we been consulted on, and agreed to, the Conceptual Site Model/Risk Assessment/Remediation Strategy? | # Y |  |
| # If yes, have references and contact information been provided? | # Y | # EA Officer Haf # Peskett # Ref: Glan Lynn Coking Works, Update to Remediation Strategy. # See consultation section |
| # If yes, has there been any changes? | # N |  |
| # Document uploaded? | # N |  |
| # Does the applicant want this application to be assessed without having remediation targets pre-agreed by us? | # n/a | # See consultation section |
| # Has there been any other pre-application advice? | # N | # Refused deployment PAN-014130 |
| # If yes, have references and contact information been provided? | # n/a |  |

| # About the site | Y N n/a | Comments |
| --- | --- | --- |
| # The site name, address, NGR given? | # Y |  |
| # Does the site address and 12 digit grid reference match? | # Y |  |
| # Is the site located within the boundary of another Environmental Permit? | # N |  |
| # If yes, please provide details below: | # n/a |  |
| # Site plan provided? | # Y | - D4583_06A Site Layout.pdf - D1910_13B Location of Reen's & Celtic Treatment Area.pdf - D4583_09 Designated Areas.pdf - Remediation Working Plan Llanwern August 2021 (Final with Apps).pdf |
| # Has there been an approved deployment in the same area within the last 12 months? | # N | # Celtic deployment nearby. Deployment for the same site refused- PAN-014130 |
| # Does it include the following? - Operating boundary - Security and access arrangements - Areas of waste soils and contaminated material, substances or products for remediation by the mobile plant - Location/siting of principal plant and equipment - Process, treatment, storage, and quarantine areas - Drainage systems - Location of boundary monitoring points and pollution control units - Potential receptors (i.e. housing, watercourses etc.) - Protected sites (if applicable i.e. SAC, SPA, Ramsar or SSSI within 1km) | # 1. Y # 2. Y # 3. Y # 4. N # 5. Y # 6. Y # 7. N # 8. Y # 9. Y | # 1. Operating boundary isn’t clear on the 3 listed site plans. There is a boundary in the Remediation Working Plan, but it doesn’t include the stockpile, treatment/quarantine area, nor the office and compound areas. # 2. access shown on D4583_06A Site Layout. Security info in 2.1.1 of the Remediation Working Plan # 3. phases 1 and 2 shown on D4583_06A Site Layout. more specifically shown on plan D4583_007 in the Remediation Working Plan # 4. presumed to be “treatment/quarantine area”. Not clear need to clarify # 5. but outside of site boundary # 6. D1910_13B Location of Reen's & Celtic Treatment Area # 7. Monitoring points on D4583_06A Site Layout. Not clear where pollution control units are located. # 8. D4583_06A Site Layout, D4583_09 Designated Areas and D1910_13B Location of Reen's & Celtic Treatment Area # D4583_09 Designated Areas |

| # Waste types and quantities | Y N n/a | Comments |
| --- | --- | --- |
| # Has the waste types, quantities etc and treatment types been provided? | # Y | # Codes and descriptions provided match those within WM3. # Contaminants and treatment technologies provided. |
| # Are the waste types and quantities from specified in the operator’s permit? | # n/a | # No waste types specified within the permit |
| # Has the document been uploaded? | # n/a |  |

| # Specified activities to be carried out on site | Y N n/a | Comments |
| --- | --- | --- |
| # Has details on suitability of treatment methods been provided? | # Y | # Detailed within section 2.3 of Remediation Working Plan |
| # Has the operator told us which treatment technology they will use? | # Y |  |
| # Are all of the proposed treatment technologies listed in the operator’s permit? | # Y |  |
| # Is the treatment technology fit for purpose? | # Y | # Reviewed against Remediation Position Statements 3A, 5, 12 and 14 |
| # Has details on residual materials/waste been provided? | # Y | # Detailed within section 2.3 and 2.6 of Remediation Working Plan |
| # Are we satisfied that these will be handled appropriately? | # N | # See query re quarantine facility |
| # Has details on quarantine facilities been provided? | # Y | # As above, this is outside of the red site boundary. # Details in Section 2.6 Remediation Working Plan. |
| # Are we satisfied the maximum capacities of the quarantine facilities are suitable to prevent or minimise pollution? | # N | # “quarantine area sufficient for at least 500m3” # No further info on how this area will be managed, nor the procedure for liquid wastes. |
| # Have the locations of the quarantine facilities been included on the site plan? | # Y | # But outside of red boundary line |
| # Has the document been uploaded? | # Y | # Remediation Working Plan Llanwern August 2021 (Final with Apps).pdf |

| # Activities involving the import of waste | Y N n/a | Comments |
| --- | --- | --- |
| # Will the activity involve the import of wastes? | # N |  |
| # Does the site form part of a Cluster project? * | # N |  |
| # Has waste acceptance information been provided? | # n/a |  |
| # Has the document been uploaded? | # n/a |  |

| # Duration of this deployment | Y N n/a | Comments |
| --- | --- | --- |
| # Is the duration of the deployment 12 months or less? | # Y | # “The works associated with this deployment are expected to last for 12 months.” |

| # Management supervision | Y N n/a | Comments |
| --- | --- | --- |
| # Has the technically competent manager details been provided? | # Y | # Miss Melissa Charters |
| # Has the nominated competent person details been provided? | # Y | # Mr Daniel Mills |
| # Has the site supervision plan been provided? | # Y | # Details in Section 2.5 of the Remediation Working Plan |
| # Is the site supervision plan appropriate? | # Y |  |
| # Has the document been uploaded? | # n/a |  |

| # Conceptual site model and risk assessment | Y N n/a | Comments |
| --- | --- | --- |
| # Has the conceptual site model (CSM)/risk assessment been provided? | # Y | # Details in Appendix E of the Remediation Working Plan. # Basic but covers made points. |
| # Has the document been uploaded? | # n/a |  |

| # Pollution control | Y N n/a | Comments |
| --- | --- | --- |
| # Have the pollution control measures been provided? | # Y | # Section 2.8 of the Remediation Working Plan (also section 2.1.2 and Appendix E) |
| # Has the document been uploaded? | # n/a |  |

| # Emission monitoring plans | Y N n/a | Comments |
| --- | --- | --- |
| # Has the site specific monitoring plan been provided? # The following may be applicable: - Groundwater - Surface water - Soil gases - Air emissions - Noise - Vibrations - Odour - Volatile Organic Compounds | # Y N | # Surface water # Soil gases # Air emissions # Noise # Odour # Volatile Organic Compounds # Sections 2.1, 2.3 and 2.8, Appendix C, Appendix D, Appendix F, Appendix H of Remediation Working Plan # No groundwater or vibration monitoring- why? |
| # Have the indicator parameters been provided? | # Y | # Table 2.2 |
| # Has the document been uploaded? | # n/a |  |

| # Record Keeping - Commissioning, operating and maintenance | Y N n/a | Comments |
| --- | --- | --- |
| # Details of record keeping given? | # Y | # Appendix G of Remediation Working Plan |
| # Document uploaded? | # n/a |  |

| # Supporting documents | Y N n/a | Comments |
| --- | --- | --- |
| Have relevant documents been ticked and supplied? - Site plan showing all required features - Conceptual Site Model and Risk Assessment - Pollution Control - Monitoring Plans - Record Keeping details | # 1. Y # 2. Y # 3. Y # 4. Y # 5. Y | # See RFI for missing details in the sections |

| # Confidentiality and national security | Y N n/a | Comments |
| --- | --- | --- |
| # Has confidentially been claimed? | # N |  |
| # Has national security been claimed? | # N |  |

| # Declaration | Y N n/a | Comments |
| --- | --- | --- |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | # Y | # Letter Of Authority MK |
| Has the operator confirmed they will comply with the conditions of the rule set? | # Y |  |
| # Has a relevant person or approved person signed the declaration? | # Y |  |

| Site specific risk assessment | Y N n/a | Comments |
| --- | --- | --- |
| If there are other potential sensitive receptors has a site specific risk assessment been submitted? | Y |  |
| Does the site specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| SSSI/ Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 1 km of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a “likely significant effect” been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as “information only” | N |  |

| Natura 2000 or Ramsar site/ Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 1 km of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | No |

Comments

Consultation responses: Geoscience 20/09/21: - majority of previous concerns relating to this application have been addressed however I’ve been unable to access the baseline soil and surface water results for emissions monitoring. - It also remains unclear where the surface water monitoring locations are in relation to the remediation works taking place. They are included within the site plan provided. Appendix H obtained and forwarded to Geoscience 11/10/21 Geoscience comments dated 21/10/21 following submission of baseline data (received via email): The remediation working plan states a background study will be undertaken to define the baseline to which results obtained during the works would be compared. We have reviewed the data provided and confirm sufficient baseline has been collected. Could the application please clarify what will be used as the baseline for which results during works can be compared to. For example with the average or maximum values for determinants be used from this data set for comparison? This isn’t set out within the remediation working plan as far as I can see. I have also noticed that that in the monitoring plan table of Appendix D, the surface water is said to be tested in line with the discharge consent determinants rather than the baseline. Could you please clarify what will be used to demonstrate ‘no deterioration’. The remediation working plan sets out that in order to demonstrate that there is no worsening of reen/ditch water quality during the remediation works, surface water monitoring (at positions WS01, WS02, WS03, WS04) will be undertaken and up to 4no monthly samples will be taken and analysed for an appropriate suite of contaminants (suite tbc depending on the results of the baseline analysis). Similarly to the above query, can this suite now be provided following completion of the baseline analysis? In addition, can the application clarify how sampling locations WS01-WS04 are appropriate to demonstrate ‘no deterioration’ taking into consideration that it looks as though positions WS01-WS04 appear to be located up stream of the water treatment site & discharge. This was added into the RFI. See below.

MyMap and location assessment: See MyMap Searches, Risk Screening Report and Appendix 2 of this document

Request for information sent 21/10/2021. Deadline of 08/11/2021 - References is made within the Remediation Working Plan to "phase 4a and 5", however this terminology isn’t used anywhere on any site plans. Is phase 4a the same as phase 1, and phase 5 the same as phase 2? Please confirm the terminology used. Please amend the site plans to include the following: - you are required to show the deployments operating boundary. The site boundary shown in red on plan D4583_02 does not include to the west the stockpile area, treatment/quarantine area, or the WTP or to the east the office & compound. All of this should be included within the operating boundary. - you are required to show the location/siting of principal plant and equipment. As above the locations of the treatment/quarantine area, the WTP is outside of the site boundary. It is also not clear from any site plans where any other plant or equipment (if any is present) is stored. - you are required to show the process, treatment, storage, and quarantine areas. As above these are outside of the site boundary. - you are required to show the location of boundary monitoring points and pollution control units. Boundary monitoring points have been provided but it is not clear where the pollution control units (if any present) are located. - the discharge point under permit EPR/BS3905IP/V004 needs to be shown on a site plan as well as the drainage route from the Celtic deployment PAN-011383 to the discharge point at the Severn Estuary. - Remediation Working Plan section 2.1.2 Ex-Situ Treatment of Encountered Groundwater states: “In order to facilitate excavation works in grossly impacted areas, LNAPL impacted water is to be removed from the dig area and subject to ex-situ treatment prior to consented discharge (permit ref: BS3905 operated by Corus Strip Products, dated 18 March 2004). The water treatment plant is to be located upon a bunded and impermeably lined pad. Any build-up of rain or spillage water will be held upon the pad and passed through the treatment plant for consented discharge (permit ref: BS3905 operated by Corus Strip Products, dated 18 March 2004).” The site plans shows “Water Being Pumped to Treatment Plant” which flows to “Celtic Treatment Area”. To ensure we are clear, please confirm or correct the following process: - ground and surface water are first treated at the WTP proposed under this deployment application - ground and surface water are further treated under the Celtic deployment PAN-011383 - additional treatment occurs under the permit EPR/BS3905IP/V004 before being discharged to the Severn Estuary. - the deployment application consistently refers to “Perched water and surface water will require management and be treated prior to consented discharge (permit ref: BS3905 operated by Corus Strip Products, dated 18 March 2004).” Please note this is not a discharge consent, but a surface water emission point from the permit EPR/BS3905IP/V004 held by TATA STEEL UK LIMITED. It is Emission Point W1 (an emission point into the Severn Estuary) with limits as follows: We acknowledge that Appendix B contains details of the use of the emission point, but the deployment application itself does not contain information on how the discharge will be managed to ensure compliance with the limits set within the permit as referenced above, or Celtic deployment. - The water treatment plant operated by Celtic. This deployment (PAN-011383) is only valid until January 2020. What are your intentions for further treatment once this deployment is no longer in place? - Remediation Working Plan section 2.6.1 Soils / Solids you have noted that “Upon excavation any materials deemed unsuitable for treatment or re-use will be quarantined for further testing and will be clearly labelled. A quarantine area sufficient for at least 500m3, to the same design as and forming part of the treatment pad is shown in Drawings D4583_06A and D1910_13B. Quarantined materials that have been tested and subsequently considered unsuitable for treatment are to be removed from site.” What is the process for when the quarantine area is full or close to being full? The information provided is only for solid (soil) wastes. What are the quarantine procedures for liquid or asbestos wastes? - Monitoring has been proposed for Surface water, Soil gases, Air emissions, Noise, Odour, Volatile Organic Compounds. No groundwater or vibration monitoring has been proposed. Please explain the reasoning for this. - Table 2.1 gives the Environmental Monitoring Locations and Table 2.2 lists the monitoring targets. Table 2.1 does not include noise and neither table includes soil gasses or air emissions (although VOCs and asbestos fibres are). Please ensure the missing parameters are included in the monitoring locations and targeted where relevant. Where they are not included please explain the reasoning for this. - The remediation working plan states a background study will be undertaken to define the baseline to which results obtained during the works would be compared. We have reviewed the data provided and confirm sufficient baseline has been collected. Please clarify what will be used as the baseline for which results during works can be compared to. For example, will the average or maximum values for determinants be used from this data set for comparison? This isn’t set out within the remediation working plan. - In the monitoring plan table of Appendix D, the surface water is said to be tested in line with the discharge consent determinants rather than the baseline. Could you please clarify what will be used to demonstrate ‘no deterioration’? - The remediation working plan sets out that in order to demonstrate that there is no worsening of reen/ditch water quality during the remediation works, surface water monitoring (at positions WS01, WS02, WS03, WS04) will be undertaken and up to 4no monthly samples will be taken and analysed for an appropriate suite of contaminants (suite tbc depending on the results of the baseline analysis). Similarly, to the above query, can this suite now be provided following completion of the baseline analysis? - Please clarify how sampling locations WS01-WS04 are appropriate to demonstrate ‘no deterioration’ taking into consideration as positions WS01-WS04 appear to be located up stream of the water treatment site & discharge. Are samples taken at the WTP? are any sample sites proposed further downstream of the treatment sites?

Response to RFI recieved 05/11/2021: - phrasing confirmed - plan amended- see 1910_13D (and D1910_02) - plan amended- see D1910_02 - plan amended- see 1910_13D (and D1910_02) - information provided on pollution control units. Plan also amended- see 1910_13D - See plans 1910_13D and D1910_28 - treatment process confirmed - process of compliance with Tata Steel permit confirmed - Celtic deployment: “We understand that Celtic have been in permit pre-application discussions with NRW with the intention of continuing this activity under permit, but are currently awaiting a conclusion on the permit requirement from NRW. Contractually it is assumed that Celtic will be treating the water throughout our works. If this process changes an alternative treatment methodology would be agreed with the client to ensure the limited stated in the previous section are met. At no point would untreated water, or water exceeding the criteria in the table above be discharged.” - quarantine information provided - monitoring information provided - monitoring information and justifications provided - baseline data- maximum values will be used - baseline data- deterioration information provided also see Appendix E (Monitoring Plan) - baseline data- reen sampling suite is petroleum hydrocarbons, pH, TSS, Fe, Cr, Ni, Zn, Cu, Pb, As, Cd, Hg. - sample point- reasoning clarified. Geoscience accept the responses to their questions- see email saved 08/11/2021

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Permitting Officer: Louise Bailey Date: 08/11/2021 |  |
| ## Team leader or ToP approval: Huw Davies Date:08/11/2021 |  |

| Appendix 1 – Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1085 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 1165 minutes |

| Appendix 2 – Location Checks |  |  |
| --- | --- | --- |
| Designations | Present? | Comments |
| Special protection areas (SPA) Potential special protection areas (pSPA) | N |  |
| Special areas of conservation (SAC) Candidate special areas of conservation (cSAC) | N |  |
| Ramsar sites | N |  |
| Site of Special Scientific Interest (SSSI) | Y | Gwent Levels - Nash and Goldcliff |
| Ancient woodland | Y | All north of railway line |
| Areas of Outstanding Natural Beauty (AONB) | N |  |
| Heritage Coast | N |  |
| Local nature reserves | N |  |
| Local wildlife sites | Y | Hartridge Wood Hartridge Fields Former Steel Works Site Off Queensway Meadow Dockwell Wood Monk's Ditch Ringland Way Marsh |
| National nature reserves | N |  |
| National parks | N |  |
| Scheduled ancient monuments | Y | Listed buildings: - Parish Church of St Mary - Barn at The Barn Farm Historic monuments: - LLANWERN, CHURCHYARD CROSS;ST MARY'S CHURCHYARD CROSS - LLANWERN, VILLAGE EARTHWORKS - ST MARY, LLANWERN Scheduled ancient monument: - St. Mary's Churchyard Cross, Llanwern |
| World Heritage Sites | N |  |
| Controlled waters such as rivers and lakes | Y | Watercourses shown around edges of the site. |
| European Protected Species | Y | Reedbeds (south- blue area) Lowland dry acid grassland and Lowland calcareous grassland (north- green area) Eel, Silver eel, Yellow Eel (South-West pink square) Eel, Silver eel, Yellow Eel (south- pink strip in water way) |
| Nationally Protected Species | Y |  |
| Section 7 Priority species | Y |  |
| Section 7 Priority habitats | Y |  |

AONB’s, Ancient Woodlands, Scheduled Ancient Monuments, Local and National Nature Reserves, National Parks and Heritage Coasts.

Potential hazards include dust, odours and run-off / leachate.

Receptors to be affected include environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities.