| ## Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-015339 |
| Permit Reference Number: | EPR-KP3594VE |
| Permit Holder: | Trade Effluent Services Limited |
| Site Address: | Honkley Farm, Burton, Rossett, Wrexham, LL12 0AP |
| Grid Reference: | Farm location: SJ 3383 5933 Fields: 1 SJ 3330 5896 deleted from application 05/11/21 2 SJ 3332 5923 3 SJ 3363 5925 4 SJ 3344 5911 5 SJ 3383 5933 6 SJ 3396 5938 7 SJ 3401 5923 8 SJ 3378 5911 9 SJ 2266 5839 10 SJ 3383 5905 11 SJ 3388 5899 12 SJ 3407 5907 13 SJ 3410 5895 14 SJ 3395 5890 15 SJ 3381 5886 16 SJ 3383 5880 17 SJ 3361 5877 Storage: None |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: WrexhamFlintshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Amy.Henderson@cyfoethnaturiolcymru.gov.uk |
| Date due: | 04/10/2021 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating C - 110% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- DMS/PaLS |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | Y | Amount: 877.80 Date cleared on MyNRW: 29/09/2021 Transaction number: 2093919 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | n/a |  |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | N | Name provided (Rob Piggott) doesn’t match the permit name of Trade Effluent Services Limited- see RFI |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y | Grid reference provided is 10 digits but shows the location of the farm (Field 5) |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | N | List the CPH numbers for each field here in the comments section at the end. |
| ## Has all of parcels of land table been completed? | N | Table completed with missing information relating to the Waste types to be spread/used - RFI |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 50 ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | Not all details for all the landowners has been provided – Fields 2-17 are under the correct landowner – Field 1 is the exception – see comments and RFI. |
| ## Has consent been given to carry out the activity? Ensure that explicit consent has been given from each land owner/occupier for each farm/field (if more than one) | N | Consent needed for field 1 - See RFI |
| Has all of the previous land treatment table been completed? | Y | PAN-004507 – completed and expired 28/02/2020 |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | n/a |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | n/a |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | n/a |  |
| Has all of the waste types table been completed? | Y | Stackable/non-stackable information has not been provided. Waste producer name not given - See RFI |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | EA public register |
| Has all of waste storage table been completed? | n/a |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | n/a |  |
| Has checklist been completed? | Y | Checklist completed with the addition of ticked site-specific risk assessment which is not required under this risk band nor was it attached– mentioned in RFI |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | N | Ryan Griffiths- Patel not given authorisation to sign on behalf of Richard Street by the permit holder. |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | N | This needs to be verified by the landowner as Ryan Griffiths-Patel has signed on behalf of Richard Street. Richard Street has been given permission by the permit holder so would therefore need his signature of declaration. |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | n/a |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y | Surface water courses present (buffer zones included with applicant map) |
| Groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | Y | SPZ3 – low risk deployment (2a on screening report) |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | N/A |  |
| location of all storage sites for the waste to be spread | n/a |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | Issue present – cannot determine whether used for domestic use or food production purposes – toned to buffer 50 from source of issue – Shown in 1b on MyMap screening report. See RFI |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | N/A |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | N/A | See image 3 on screening report |
| location of any public rights of way (shown on map) | Y | See image 4 on screening report |
| any buildings or houses within 250m (shown on map) | Y | 2 buildings within 250m – illustrated in 5 in screening report |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - - Ditches - | Y | Protected habitats – 6a Scheduled ancient monuments – Local wildlife sites – 6b NVZ – 6c Ditches – potential ditch running alongside southern border (6d) but buffered by applicant |
| What is the slope of the fields? | - 0-7 degrees (transparent)- fields. – 6,7,10,11,12,13,15 - 7 -12 degrees (yellow)-fields 1,2,3,4,5,8,9,14,16,17 - 12 degrees plus (red)– Fields 16 and 17 contain small bits but covered by the buffer zone. Image 7 in screening report – illustrates the sloping areas within each field. The overall runoff risk in this area is low. |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | n/a |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y | Arable crop (maize) – Winter 2021 Grass Silage – Early summer 2022. |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | N | See RFI |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Highlighted issues: - K index- See nutrient calculator- offtake is higher than additions. - Pollution incidents in stream south of field 4/5/6- required 10m buffer zone is in place – The source of pollution originated from a slurry storage lagoon away from the spreading activity from the fields – information requested from Amy Henderson. - Farm is a dairy farm with slurry lagoon. concern raised that only FYM spread - see RFI.

MyMap and location assessment: See PAN-015399 MyMap screening report

Protection of habitats and ecosystems n/a

Agricultural benefit calculations: - Conductivity and Sodium are high – This has been addressed within the benefit statement where the high rates of sodium have a knock on effect on the conductivity of the soil which is said to “return to normal after a period of 10-12 weeks”.

Request for information sent 03/11/2021; response received 05/11/21: - To confirm permit holder name - Confirmed that permit holder is Trade Effluent Services Limited - Provide the CPH number for all fields - Provided the holding number - Holding number provided has extra 0 and doesn't match. - Include missing waste types to be spread - Confirmation the waste to be spread - Consent for field 1 landowner - Field 1 removed from application - Slurry application on any fields - Table 4 in agben amended with addition to cattle slurry. - Stackable/non-stackable confirmation - Confirmed non-stackable - Producer of waste left out - Confirmed waste producer - Risk assessment - none needed - ticked in error - Declaration authorisation - Confirmed Richard Street is signing on behalf of Trade Effluent Services Limited - 50m buffer at issue source - 50m buffer applied - map on revised agben - Consistent soil type - Confirmed soil type for fields are clay loam - How many waste applications - Advised that applications between Feb-March and/or after cuts of silage for grass fields

Other information to note: Each Field within the deployment with CPH and landowners - Field 1 - CPH - 56/010/0033 - G J Walton and Partners field removed 05/11/21 Field 2 - CPH - 56/010/0022 - JR Knowles and Son Field 3 - CPH - 56/010/0022 - JR Knowles and Son Field 4 - CPH - 56/010/0022 - JR Knowles and Son Field 5 - CPH - 56/010/0022 - JR Knowles and Son Field 6 - CPH - 56/010/0022 - JR Knowles and Son Field 7 - CPH - 56/010/0022 - JR Knowles and Son Field 8 - CPH - 56/010/0022 - JR Knowles and Son Field 9 - CPH - 56/010/0022 - JR Knowles and Son Field 10 - CPH - 56/010/0022 - JR Knowles and Son Field 11 - CPH - 56/010/0022 - JR Knowles and Son Field 12 - CPH - 56/010/0022 - JR Knowles and Son Field 13 - CPH - 56/010/0022 - JR Knowles and Son Field 14 - CPH - 56/010/0022 - JR Knowles and Son Field 15 - CPH - 56/010/0022 - JR Knowles and Son Field 16 - CPH - 56/010/0022 - JR Knowles and Son Field 17 - CPH - 56/010/0022 - JR Knowles and Son

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 11/11/2021 |
| ## Effective date | 11/11/2021 |
| ## Permitting Officer Date | Gianluca Broadley 11/11/2021 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 11/11/2021 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 3000 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 3080 |