| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-015177 |
| Permit Reference Number: | EPR/FB3302MM |
| Permit Holder: | RL Waste Consultancy Ltd |
| Site Address: | Bach Y Rhew, Ceredigion, SA44 5SB |
| Grid Reference: | Farm location: SN 36380 47520 Fields: S1 SN36034671 Fields removed from deployment due to CPH number uncertainty S3 SN36304684 G4 SN36474738 G5 SN36504729 G6 SN36564722 G2 SN36144724 S10 SN36634736 S5 SN36304706 S4 SN36444687 S7 SN36044758 S8 SN36114747 S9 SN36274736 S11 SN36454757 S12 SN36454746 Storage: SN 36380 47520 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: CeredigionEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Sean.Wannop@environment-agency.gov.uk |
| Date due: | 17/09/2021 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y |  |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | Y | Amount: 798.00 Date cleared on MyNRW: 27/08/2021 Transaction number: 2093939 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | n/a |  |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | No number given on LPD1 form, following numbers found on Mymap. CPH numbers S1 55/256/0055 +55/256/0056 S3 55/256/0055 S4 55/256/0055 S5 55/256/0055 S7 55/256/0055 S8 55/256/0055 S9 55/256/0055 S10 55/256/0053 S11 55/256/0055 S12 55/256/0055 G2 55/256/0056 G4 55/256/0055 G5 55/256/0012 G6 55/256/0012 RFI CPH number given |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the landowner or occupier been provided? | Y | Consent & contact information given for Mr Davies. However, contact information was the applicant contact not the landowner. Mentioned in RFI. Not all landowners provided. Landowners for each field: S1 CM & NS ACHERMANN & GA WADSWORTH S3 GA WADSWORTH S4 GA WADSWORTH S5 GA WADSWORTH S7 GA WADSWORTH S8 GA WADSWORTH S9 GA WADSWORTH S10 EJ, MA AND I LL JONES S11 GA WADSWORTH S12 GA WADSWORTH G2 CM AND NS ACHERMANN G4 GA WADSWORTH G5 Mr David and Mrs Jannette Jones G6 Mr David and Mrs Jannette Jones |
| ## Has consent been given to carry out the activity? | Y | Consent required by other landowners. Mentioned in RFI. (Completed) |
| Has all of the previous land treatment table been completed? | N/A |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High-risk deployment has a SSRA been provided? | n/a |  |
| ## What type of risk assessment has been submitted? | Benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N/A |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N/A |  |
| If no have details of the assessment evidence to support this conclusion been provided? | N/A |  |
| Has all of the waste types table been completed? | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| Has checklist been completed? | Y |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y | Drain located on field S3, identified as a non-spread area on map. |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | n/a | No sites. |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | n/a | No sites, one located 117 meters away, no buffer required. |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m | n/a | No sites, one located 117 meters away, no buffer required. |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a | No sites, one located 117 meters away, no buffer required. |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a | No sites. |
| location of any public rights of way (shown on map) | Y | Mapped, check MyMap report |
| any buildings or houses within 250m (shown on map) | Y | Mapped, check MyMap report. |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | n/a | Field S1 is partially on a protected habitat but no action needed. Check MyMap report. |
| What is the slope of the fields? | 0-7 degrees S4, S5, G2, S9, S12, G4, G5, G6 7 -12 degrees S3 (partly), S8 12 degrees+ S1 (Partly), S11 (mainly), S10 (mainly), S7 (mainly). Check MyMap report for assessment. |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Liming |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) Y |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y | Liming within acceptable limits |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | N | Only potash doesn’t match, mentioned in RFI. |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: - query on waste coding and description- see RFI - query on suitability of waste- replied directly to consultee as waste type is in the rule set - query on suitability of spreading location- this is acknowledged in the ABS with the statement “The material is fine grained and windy conditions should be avoided when spreading near sensitive receptors such as housing and watercourses. “

MyMap and location assessment: See MyMap report and location section of this document. Assessment of slope carried out- see RFI

Protection of habitats and ecosystems n/a

Agricultural benefit calculations: Nutrients supplied provided for Nitrogen, Phosphorus, Potassium and Sulphur. Potassium and Sulphur results do not match our calculations- see RFI

Request for information: - Missing CPH numbers for fields. Applicant gave one CPH number and removed other fields because they were unsure which CPH number to provide. - Details of the landowner or occupier. Applicant confirmed landowner details and contact information. - Incorrect permit number for waste producer. Applicant confirmed they made a typo. - Clarifying origins of the waste- Applicant provided an adequate description. - Clarifying a site-specific risk assessment wasn’t needed as the box was ticked. Applicant confirmed this was a mistake. - Brining awareness to some highly sloping land that land spreading should only take place in dry weather conditions. Applicant agreed with my assessment. - Missing conductivity reading from the waste analysis. Applicant gave a reasonable answer to why this wasn’t included in the waste analysis. - Missing grazing yield value. Applicant confirmed the grazing yield. - A review of the workings of nutrients supplied for potassium as there was a discrepancy. Applicant given instructions on how to calculate nutrients supplied and gave the correct numbers. - Clarifying how often land spreading activity will occur. Applicant clarified there is only one spreading campaign. - Clarifying if only one waste stream is being spread. Applicant confirmed only one waste stream is to be spread.

Other information to note: Landowners S1 55/256/0055 GA WADSWORTH BACH Y RHEW S3 55/256/0055 GA WADSWORTH BACH Y RHEW S4 55/256/0055 GA WADSWORTH BACH Y RHEW S5 55/256/0055 GA WADSWORTH BACH Y RHEW S7 55/256/0055 GA WADSWORTH BACH Y RHEW S8 55/256/0055 GA WADSWORTH BACH Y RHEW S9 55/256/0055 GA WADSWORTH BACH Y RHEW S10 55/256/0053 EJ, MA AND I LL JONES PANTYREFAIL S11 55/256/0055 GA WADSWORTH BACH Y RHEW S12 55/256/0055 GA WADSWORTH BACH Y RHEW G4 55/256/0055 GA WADSWORTH BACH Y RHEW G5 55/256/0012 Mr David and Mrs Jannette Jones Castell Farm G6 55/256/0012 Mr David and Mrs Jannette Jones Castell Farm G2 55/256/0056 CM AND NS ACHERMANN CWMBWCH

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 19/11/2021 |
| ## Effective date | 19/11/2021 |
| ## Permitting Officer Date | 19/11/2021 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 19/11/2021 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 4,150 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 4,230 |