# Natural Resources Wales permitting decisions

# Bespoke Variation

We have decided to issue the variation for Gloster Row SPS CSO operated by Dwr Cymru Cyfyngedig

The application number is PAN-013490

The permit number is BH0074301

The discharge is for creation of a new relief overflow point for St Mary’s CSO (also know as Gloster Row SPS) Cardigan to alleviate sewer flooding caused by tide locking of current outfall. Involved the construction of new storage in the foul sewer to a new pumping station that will only operate when the existing CSO is tide locked and the sewer has backed up to the new pumping station. Application to vary permit BH0074301.  Part of a multi-agency response to reduce flooding in Cardigan.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Key issues

- Annex 1 the decision checklist

# Key issues of the decision

There will be no increase in the number of spill or increase in load from this project with the current situation regulated by the permit BH0074301.

The sewage pumping station will continue to pump forward to the waste water treatment works the same volume of sewage which also happens to be greater than the industry standard for a satisfactory system.

Currently the system backs up and contributes to flooding in this area of Cardigan town at high tide when the current outfall is tidelocked.  Currently this storm sewage would either enter the estuary via run off or once high tide had passed and the CSO was no longer tidelocked, through the existing outfall.

It would not be passed forward in the sewer due to the capacity issue during a storm/flood event.

The two outfalls will not discharge at the same time, with the new outfall only operating at high tide

The project will increase the amount of storage within the sewer between the existing CSO weir and the new pumping station.

The new pumping station will pump only the storm sewage that has accumulated due to the sewer backing up at high tide so that it does not contribute to flooding in the town.

There will not be an increase in spills.  The 44 spills mentioned in their report as going through the new outfall is just a pro-rata figure based on the time during a 24 hour period that the existing outfall is tidelocked.  Currently they would discharge either over land or via the existing outfall following high tide.

Any discharge from the new outfall will be screened for plastics etc, so is an improvement for this proportion of the discharge as it currently goes through the existing outfall is unscreened.

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| The facility |  |  |  |
| The regulated facility | ###### The extent/nature of the facilities taking place at the site required clarification. The regulated facility is a standalone water discharge activity in accordance with EPR RGN 2 Understanding the meaning of regulated facility. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the sites has been carried out as part of the permitting process. We consider that the application will not affect the features of the sites We have not formally consulted on the application. The decision was taken in accordance with our guidance. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | Appropriate assessment: In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. (As documented in section 4 of OGN 200 form 1, and section 5 if applicable) |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment or similar methodology supplied by the operator and reviewed by ourselves, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The new outfall will not increase the current permitted discharges and will only discharge at high tide when the existing outfall is tide locked. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Monitoring | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods and to the frequencies specified. These monitoring requirements have been imposed in order to record and report the event duration monitoring and the frequency the new outfall is used to make a discharge in line with limits of specified activity S1.1 We made these decisions in accordance with How to comply with your permit 7.01 | ü |
| Reporting | We have specified reporting in the permit. Annual reporting of event duration monitoring conditions (EDM). We made these decisions in accordance with How to comply with your permit 7.01 | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, in our separate and distinct capacities as applicant and determining body, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |