# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Nanhcoch operated by Mr Edward Whelan.

The application number is PAN-013849.

The permit number is EPR-CB3390CD.

The discharge is for a new surface water discharge at Nantcoch, Heol Senni, Brecon, LD3 8SU. The discharge location NGR is SN 92564 22336.  A sewage effluent discharge of a low volume 1.5 m3/day has been estimated.

The package treatment plant system will be serving one residential property. The property has 4 bedrooms in total. The treated effluent is domestic in nature. The system incorporates a package treatment plant discharging to a seasonally dry surface water tributary.  The package treatment plant conforms to current British Standards BS 12566.  The flow of the tributary being discharged to is seasonal dry and flows under the roadway and into a neighbour’s field, for approximately 420m before entering into the Afon Senni (Usk SAC).  The proposal replaces an existing septic tank and infiltration system which has been in place for many years and is in need of upgrading. This existing system is currently unregulated and produces an unknown quality of effluent.  There is a high probability that due to its close proximity to a water culvert at the site, which flows into the Afon Senni (Usk SAC), there is a potential pathway for this system to increase nutrient loading in the Afon Senni (Usk SAC).  The proposed system would eliminate this pathway and would be preferred over the current system which provides an unknown quality of effluent

The applicant is voluntarily also planning to install a 10m perforated pipe to address the seasonal dryness of the discharge location.  A reed bed system will also be incorporated upon advise from NRW.  Both systems additions will provide additional cleansing to the discharge.

We consider in reaching this decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | No responses were received. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the site has been carried out as part of the permitting process. We consider that the application will not affect the features of the site. We have not formally consulted on the application. The decision was taken in accordance with our guidance. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. The existing Septic Tank and Infiltration system is unregulated and provides an unknown quality of effluent. As the current system is within approximately 5 metres of a stream culvert there is a high probability that this systems discharge has continuity with the adjacent culvert, that eventually flows to the Afon Senni, approx. 420 metres away, via network of surface water channels across boggy land. The new proposal intends to replace the existing Septic Tank and Infiltration System, built to an unknown standard, with a British Standard package treatment plant, with the last 10 metres of pipe being perforated and set within a reed bed. This will provide a significant upgrade and overall net-improvement on the current system, by substantially reducing the level of phosphate within the discharge and volume of effluent discharged. This proposed system remove the current impact pathway to the SAC to an insignificant level, therefore we can conclude that there is ‘No Likely Significant Effect’ as it is considered the impact pathway to the SAC has been removed. This application brings the site under NRW regulation which will better enable any future compliance, if needed. The regulation of this proposal is proportionate to the unique situation of the site and existing system. This approach complies with the current planning advice provided by NRW in that: “The following developments can be screened out as not likely to have a significant effect on a river SAC in relation to phosphorus inputs as there is unlikely to be a source of additional phosphorus or pathway for impacts: As the application - ……. does not increase the volume of foul wastewater - ……. improves existing water quality discharges by reducing the phosphorus load of wastewater, or by decreasing the volume of wastewater produced (e.g. by improvements to existing wastewater treatment infrastructure) |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment all emissions may be categorised as environmentally insignificant. |  |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes according to TGN 7.01 . The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. |  |
| The permit conditions |  |  |  |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand-alone water discharge activity permit under EPR2016. | ü |