| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-015812 |
| Permit Reference Number: | EPR-KP3594VE |
| Permit Holder: | Trade Effluent Services |
| Site Address: | Waterloo Farm, Sealand Road, Deeside, Flintshire, CH52LQ |
| Grid Reference: | Farm location: SJ 34653 68880 Fields: 1 JW1 SJ 34920 68770 2 JW2 SJ 34820 68600 3 JW3 SJ 34770 68440 4 JW4 SJ 34580 68570 5 JW5 SJ 34460 68620 6 JW6 SJ 34270 68700 Storage: 1 SJ 34690 68700 2 SJ 34844 68877 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: WrexhamFlintshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Amy.Henderson@cyfoethnaturiolcymru.gov.uk |
| Date due: | 09/11/2021 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating C- 110% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- DMS/PaLS |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: 877.80 Date cleared on MyNRW: 28/10/2021 Transaction number: 2094608 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | n/a |  |
| Site Specific Risk Assessment | N |  |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | n/a |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | CPH numbers for each field listed within comments section at the end |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | John Williams -Landowner |
| ## Has consent been given to carry out the activity? | Y |  |
| Has all of the previous land treatment table been completed? | N | PAN-012114 – Issue date was the 18/12/2020, expires on 17/12/2021 Also description as “commercial waste” not sufficient See RFI points 1 and 2 |
| ## Which risk band does the activity fall within? | Medium (2) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | N/A |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N/A |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N/A |  |
| If no have details of the assessment evidence to support this conclusion been provided? | N/A |  |
| Has all of the waste types table been completed? | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| Has checklist been completed? | Y | Checklist completed with the addition of ticked site-specific risk assessment which is not required under this risk band nor was it attached–RFI – Point 3 |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a | Stored in mobile storage tanks |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y | Shown in image 1 MyMap report |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | N | Not located within groundwater source protection zone. |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | N |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | N |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | N |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | N | None that fall within the buffer – Image 2 in MyMap report. |
| location of any public rights of way (shown on map) | N |  |
| any buildings or houses within 250m (shown on map) | Y | Image 3 within MyMap report shows 2 listed buildings |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | Protected Habitats – Shown in image 4a (MyMap) Nitrate Vulnerable Zone (NVZ) – Shown in 4b (MyMap) |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields. LW1 7 -12 degrees (yellow)- fields Northern + Eastern border of LW3. Southern and Western border of LW2. 12 degrees plus (red)- fields – Southern border of LW3-6. Shown in image 5 (MyMaps) Overall runoff risk is Low – Areas that display sloping are covered by buffer zones within site map (Figure 1 in Agricultural Benefit Statement) |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y |  |
| - If yes; have they been tested for? | Y | Beechdean, Croda Widnes, D Wise, Kelloggs, Maelor Foods, Rowan Foods, Secanim all tested for Fats and Oils. |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | N |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | N |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y | Mentions a few methods of waste application – RFI to confirm which one – Point 4 |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | N | Haven’t stated this – mention in RFI – Point 5 |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | N | Commercial waste – not specified – See RFI |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: - Comments concerning PAN-012114– Included within the RFI sent to ensure that this deployment is not carried out until the previous has expired. - Whilst JW2 and JW6 have a Phosphate index of 4 for these fields - overall offtake is greater than addition for these fields, and the waste is applying other nutrients to fields which have a requirement. - Issues raised about spreading when land condition is unsuitable. Permit condition 2.3.2 states waste cannot be spread on frozen or water logged land. Applicant has confirmed on the LPD1 form that they will comply with all conditions of the permit - Issued raised about odour complaints in the locality from a non-waste source. Applicant has outlined within application that the spreading of sludge will be done via sub-soil injection and within agricultural hours to minimise odour. RFI question(s) to ensure waste application is consistent. Operator has an Odour Management Plan in place which is saved onto the DMS. - issues raised re submitting of pre-notification forms for periods not stated in the application. This application states spreading in January/February 2022. No further action required.

MyMap and location assessment: See PAN-015812 MyMap screening report

Protection of habitats and ecosystems N/A

Agricultural benefit calculations: PH values for wastes Westland Horticulture, Kellogg and Rowan Foods are low, 4.54, 3.94 and 4.17 respectively – Justified through soil PH ranging between 6-7.4 to buffer this value to a more neutral PH.

Request for information: - sent 02/12/2021; response received 03/12/21: - Confirm deployment will not commence until PAN-012114 expire – Confirmation to expire previous deployment. - Provide waste types previously applied on the land rather than "commercial waste" - updated table provided with previous wastes that had been spread - Confirm error in checklist with inclusion of SSRA - Error confirmed - Provide principal application method due to prior odour issues - confirmed that will not be applied via shallow injector or surface application - Confirm how often waste will be spread - confirmed that spreading will be between Mid-December to February 2022 – justification given within email in RFI response 2 saved in DMS.

Other information to note: CPH numbers and landownership details: 1 JW1 SJ 34920 68770 - 56/209/0002 - John Williams 2 JW2 SJ 34820 68600 - 56/209/0002 - John Williams 3 JW3 SJ 34770 68440 - 56/209/0002 - John Williams 4 JW4 SJ 34580 68570 - 56/209/0002 - John Williams 5 JW5 SJ 34460 68620 - 56/209/0002 - John Williams 6 JW6 SJ 34270 68700 - 56/209/0002 - John Williams Information on MyNRW matches that as given on the LPD1 form- 56/209/0002 and Mr John Williams of Waterloo Farm as the landowner who has given consent.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 08/12/2021 |
| ## Effective date | 08/12/2021 |
| ## Permitting Officer Date | Gianluca Broadley 08/12/2021 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 08/12/2021 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1980 |
| Sign off | 10 |
| Issuing | 20 |
| Total | 2070 |