| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-015750 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | ByProduct Recovery Ltd |
| Site Address: | Creigydd Farm, Ponthir, Carleon, NP18 1HU |
| Grid Reference: | Farm location: ST 32542 94458 Fields: Creigydd 1 ST 32668 94350 2 ST 32552 94240 3 ST 32470 94109 4 ST 32456 94343 5 ST 32500 94761 6 ST 32716 94510 7 ST 32945 94006 Ty-Isaf 8 ST 34349 93622 9 ST 34463 93679 10 ST 34710 93628 11 ST 34925 93849 12 ST 34791 93751 13 ST 34534 93803 14 ST 34468 93858 Brook 15 ST 33174 93106 16 ST 33457 93353 17 ST 33544 93466 18 ST 33438 93549 19 ST 33356 93414 20 ST 33258 93278 21 ST 33455 93157 22 ST 33613 93147 23 ST 33566 92909 24 ST 33729 93060 25 ST 33943 93162 Storage: Creigydd ST 32484 94478 ST 32542 94385 ST 32597 94229 ST 32872 94359 ST 33016 93885 Ty-Isaf ST 34600 93829 ST 34314 93597 Brook ST 33251 93385 ST 33239 93361 ST 33302 93097 ST 33478 92867 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: SE.waste.queries@cyfoethnaturiolcymru.gov.uk Environment team: environmentteammonmouthshireandtorfaen@cyfoethnaturiolcymru.gov.uk NLO: Ewan.Marshall-Harries@environment-agency.gov.uk |
| Date due: | 04/11/2021 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y |  |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: 798.00 Date cleared on MyNRW: 13/08/2021 Transaction number: 2092533 ‘BS/13082021/9 BR PSCAPPBYPRO’ |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | n/a |  |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | CPH number given: Creigydd 60/034/0015 Ty-Isaf 60/034/0020 Brook 60/034/0088 Fields 17,18,19, 21 no record. Field 22, 23 (Creigydd Farm) 60/034/0015 Fields T1, T2, T5, T8, T9, T10 (Ty-Isaf Farm). - 60/034/0020 Fields 1, 2, 3, 4, 5, 6, 8, 9,10,12,13 (Brook Farm) 60/034/0088 |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has consent been given to carry out the activity? | Y |  |
| Has all of the previous land treatment table been completed? | Y |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | n/a |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | n/a |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | n/a |  |
| Has all of the waste types table been completed? | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| Has checklist been completed? | Y |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | Different |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring(or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | N | See ‘Protection of habitats and ecosystems Notes’ at the end of the DD. |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields 17,1,6,9,10,12,13,T1,T6 7 -12 degrees (yellow)- fields 18,19 ,2,3,4,5,8,T2,T9,T10,21,23 12 degrees plus (red)- fields Fields 20, 22 ,T5 and T8 The surface runoff risk is high, paired with the fact 2 wastes streams are liquids (water treatment effluent and Anaerobic digestate from source segregated wastes). |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Fe, Al, S (19,09,02) |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y | These wastes are a source of the major plant nutrients N, P, K, Mg, Ca and S and provide small quantities of trace elements. |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | N | Check previous deployment (PAN-011644) |
| - The nutrients supplied by the material? | N |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Query on suitability of the land spreading activity as some phosphate soil indices were at 3. The land spreading activity was adequately justified in the ABS as providing other nutrients N, K, Mg, Ca and S not only P and provide small quantities of trace elements. Query on not spreading waste in the winter period. On the ABS the spreading activity was proposed to begin 2022 (late Feb – Apr), and after cutting of silage through summer and autumn 2022.

MyMap and location assessment: Fields 20, 22, T8, T5 were identified as having parts of land that were highly sloping (12o+). Waste should not be spread on these parts of the fields due to high risk of surface runoff (mentioned in RFI).

Protection of habitats and ecosystems Protected species sites including trout and European Eels. Spreading fields also border with the Purple moor grass protected habitat. Land spreading fields also border an Ancient Semi Natural Woodland. Risk to these species/areas are minimal due to field boundary no spread buffer zones, See Mymap report for further details on all.

Agricultural benefit calculations: Phosphate offtake and phosphate added by one waste stream is over the recommended amount by 4.55kg/ha (mentioned in RFI).

Request for information: - Verifying the BACS reference. Applicant provided a dated bank statement of the payment that has been cross referenced on myNRW to match. There was a naming error on myNRW which prevented the identification of the payment. - CPH numbers missing. Applicant has provided CPH numbers. - Previous land treatment table incorrect. Applicant has corrected the table to display the correct PAN. - Same storage locations as previous deployment. Applicant has moved the location of the field heaps to different areas. - High sloping areas of fields requested to be removed from deployment. Applicant has removed high sloping fields from the deployment or created buffer zones around areas of highly sloping land. - Phosphate offtake and phosphate added by the waste is over the recommended amount by 4.55 kg/ha. Applicant has justified the overapplication due to the aftermath of grazing.

Other information to note: N/A

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 08/12/21 |
| ## Effective date | 08/12/21 |
| ## Permitting Officer Date | Owen Clee 08/12/2021 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 08/12/2021 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 3,115 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 3195 |