Notice of variation and consolidation with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

Dŵr Cymru Cyfyngedig

Llanelli Wastewater Treatment Works

Berwick Road

Bynea

Llanelli

SA14 9SN

Variation application number

PAN-014897

Permit number

BP0252601

Llanelli Wastewater Treatment Works

Permit Number BP0252601

Introductory note

### This introductory note does not form a part of the permit

The following notice gives notice of the variation and consolidation of an environmental permit.

The permit has been varied to include updated Peak Flow Equivalent Treatment (PFET) sample points and increase Pass Forward Flow (PFF) at the works via the use of the PFET system. This variation also includes updated definitions for Dry Weather Flow exceedance.

Permit BP027140101 regulating a seasonal discharge of treated effluent from the

Wastewater Treatment Works is not being consolidated as part of this variation.

The schedules specify the changes made to the permit.

The status log of the permit sets out the permitting history, including any changes to the permit reference number. It is not backdated before 6 April 2010.

| ### Status log of the permit |  |  |
| --- | --- | --- |
| ### Description | ### Date | ### Comments |
| ### Variation determined | ### 25/06/2010 | ### P removal following habitats review. ### Permit issued to Dŵr Cymru ### Cyfyngedig |
| ### Application PAN-001708 (variation and consolidation) | ### 12/07/2017 | ### Application to vary the Fe limit and ### update the permit to modern ### conditions |
| ### Variation determined ### BP0252601 | ### 03/10/2017 | ### Permit issued to Dŵr Cymru ### Cyfyngedig |
| ### Application PAN-002226 ### (Variation and ### consolidation) | ### 09/01/2018 | ### Application to consolidate wastewater treatment works (BP0252601) and settled storm (previously BP0252602) and include PFET storm discharge |
| ### Variation determined ### BP0252601 | ### 31/07/2018 | ### Permit issued to Dŵr Cymru ### Cyfyngedig |
| ### Application PAN-014897 (variation and consolidation) | ### 09/08/2021 | ### Application to include PFET final effluent monitoring point. |
| ### Variation determined BP0252601 | ### 17/12/2021 | ### Permit issued to Dŵr Cymru Cyfyngedig |

End of introductory note

Notice of variation and consolidation

The Environmental Permitting (England and Wales) Regulations 2016

### The Natural Resources Body for Wales (“Natural Resources Wales”) in exercise of its powers under regulation 20 of the Environmental Permitting (England and Wales) Regulations 2016 varies and consolidates

### Permit number

### BP0252601

### issued to:

### Dŵr Cymru Cyfyngedig (“the operator”)

### whose registered office is

### Linea

### Fortran Road

### St Mellons

### Cardiff

### CF3 0LT

### company registration number 02366777

### to operate a regulated facility at

### Llanelli Wastewater Treatment Works

### Berwick Road

### Bynea

### Llanelli

### SA14 9SN

### to the extent set out in the schedules.

### The notice shall take effect from 17/12/2021

| Name | Date |
| --- | --- |
| Melissa Broad | 16/12/2021 |

### Authorised on behalf of Natural Resources Wales

#### Schedule 1

- The following conditions were varied as a result of the application made by the operator:

- 3.1.5

- 3.3.11

- Table S1.1

- Table S1.2

- Table S3.3

- Table S3.3b

- Table S3.4

- Table S3.5

- Table S4.1

- Table S4.2

- Site Plan

### Schedule 2 – consolidated permit

### Consolidated permit issued as a separate document.

## Permit

### The Environmental Permitting (England and Wales) Regulations 2016

Permit number

BP0252601

This is the consolidated permit referred to in the variation and consolidation notice for application PAN-014897 authorising,

Dŵr Cymru Cyfyngedig (“the operator”),

whose registered office is

Linea

Fortran Road

St Mellons

Cardiff

CF3 0LT

company registration number 02366777

to operate a water discharge activity at

Llanelli Wastewater Treatment Works

Berwick Road

Bynea

Llanelli

SA14 9SN

to the extent authorised by and subject to the conditions of this permit.

Name 									          Date

| Melissa Broad | 16/12/2021 |
| --- | --- |

Authorised on behalf of Natural Resources Wales

# Conditions

Management

  - General management

The operator shall manage and operate the activities:

in accordance with a written management system that identifies and minimises risks of pollution so far as is reasonably practicable, including those risks arising from operations, maintenance, accidents, incidents, non-conformances and those drawn to the attention of the operator as a result of complaints; and

using sufficient competent persons and resources.

Records demonstrating compliance with condition 1.1.1 shall be maintained.

Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of the permit.

Operations

Permitted activities

The only activities authorised by the permit are the activities specified in schedule 1 table S1.1.

The site

The discharge activities shall take place at the discharge point marked on the site plans at schedule 7 to this permit, and as listed in table S3.2; and, the operating techniques that are the subject of conditions prefixed by 2.3 shall be applied at the locations shown, or otherwise described, in schedule 7.

Operating techniques

For the activities (A1 and A2) referenced in schedule 1, table S1.1 The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 which identifies and minimises the risks of pollution relevant to that plan , and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

For the activity A1 referenced in schedule 1, table S1.1 The operator shall comply with the relevant requirements of the Urban Waste Water Treatment (England and Wales) Regulations 1994.

For the activity A1 referenced in schedule 1, table S1.1 the operator shall manage and operate the activity so as to ensure as much total nitrogen is removed from the discharge as is reasonably practicable.

For the discharges specified in table S3.3(a):

      - The discharge shall only occur when and only for as long as the flow passed forward is equal to or greater than the overflow setting indicated due to rainfall and/or snow melt.

      - Sewage can only go to storm storage if, and for as long as, the flow of sewage is equal to or greater than the storm overflow setting due to rainfall and/or snowmelt.

      - Storm storage must be fully utilised before it can discharge to land or water.

      - The storage shall be emptied and its contents returned to the continuation flow as soon as reasonably practical.

      - The discharge shall not be comminuted or macerated and shall have passed through screens as specified and shall not contain a significant quantity of solid matter with a particle size greater than any indicated. All screenings shall be removed from the discharge.

      - Where a mechanically cleaned screen is installed, a telemetry alarm system shall be installed and maintained so as to give the operator immediate notification of a failure of the screen cleaning mechanism, unless otherwise agreed in writing by Natural Resources Wales. The operator shall take all appropriate measures to return the screen cleaning mechanism to normal operation as soon as reasonably practicable after receipt of notification of the failure.

      - The discharge in schedule 3 table S3.3b shall consist solely of biologically and secondary treated sewage effluent which has been disinfected by means of ultra violet (UV) radiation as specified in that table.

The full dose limit in schedule 3 table S3.3b must be exceeded in at least 99% of the specified measurements in any period of 12 consecutive months.

No more than 10% of measurements taken consecutively during any 24 hour period from midnight to midnight should fall below the 50% dose limit specified in schedule 3 table S3.3b.

In the event of a failure of the flow monitor and/or UV transmittance used in the control of the UV dosing system:

the maximum available number of duty banks of UV lamps shall be automatically activated;

the minimum applied UV dose at maximum effluent flow rates calculated using the 5%ile transmittance shown in schedule 3 table S3.3b, shall not be less than the full dose limit shown in that table.

There shall be no failure to achieve the full dose limit in schedule 3 table S3.3b which is as a result of planned maintenance.

From the date of installation of EDM monitoring equipment at the site: The operator shall implement any remedial action specified by Natural Resources Wales as a result of any investigation carried out in accordance with condition 3.1.6, and any remedial actions shall be completed to a timescale specified by Natural Resources Wales.

Emissions and monitoring

Emissions to water

The limits given in schedule 3 table S3.1 shall not be exceeded.

The limits in schedule 3 table S3.1 to which this condition applies may be exceeded where: in any series of samples of the discharge taken at regular but randomised intervals in any period of twelve consecutive months as listed in column 1 of schedule 3A, no more than the relevant number of samples, as listed in column 2 of schedule 3A, exceed the applicable limit for that relevant parameter. For relevant parameters subject to schedule 3C the assessment is based on a fixed calendar year from 1 January to 31 December inclusive.

For the emission limits in schedule 3 table S3.1 to which this condition applies, no sample of the discharge taken at a time when unusual weather conditions are adversely affecting the operation of the waste water treatment works, shall be taken into account in deciding whether or not the emission limit has been complied with.

On any occasion where unusual weather conditions adversely affect the operation of the waste water treatment works, the operator shall use its best endeavours to mitigate that adverse effect.

For any sample of the discharge taken to be considered for the purposes of (a) above, the operator shall notify Natural Resources Wales in writing within 14 days of becoming aware that an emission limit has been exceeded. That notification shall include a full description of the unusual weather conditions and their impact on the operation of the works.

For the emission limits in schedule 3, table S3.1 to which this condition applies, no sample of the discharge taken at a time when abnormal operating conditions are adversely affecting the operation of the waste water treatment works, shall be taken into account in deciding whether or not the emission limit has been complied with.

On any occasion where abnormal operating conditions adversely affect the operation of the waste water treatment works, the Operator shall use its best endeavours to mitigate that adverse effect.

For any sample of the discharge taken to be considered for the purposes of (a) above, the Operator shall notify Natural Resources Wales in writing within 14 days of becoming aware that an emission limit has been exceeded. That notification shall include a full description of the abnormal operating conditions and their impact on the operation of the works.

The permitted Dry Weather Flow limit in schedule 3 table S3.1 is set at the operator`s planned annual 80% exceeded daily volume discharged.

      - For compliance purposes an exceedance shall be recorded for a calendar year only when the limit in effect on 31 December of that calendar year is exceeded by 90% or more of the ‘good’ recorded Total Daily Volumes in that calendar year.

      - Until 1 January 2026

        - If an exceedance of the Dry Weather Flow limit is recorded in a calendar year then the operator shall, as soon as is practicable, investigate the reasons for the exceedance.

        - The operator shall report the reasons for the exceedance to Natural Resources Wales and the steps that it proposes to take to restore compliance.

        - An exceedance of the Dry Weather Flow limit shall not be recorded as a failure of the Dry Weather Flow limit in that calendar year if the operator takes appropriate steps to restore compliance.

- From 1 January 2026 The limit has been complied with in an assessment calendar year unless;

        - the limit was exceeded in the compliance assessment calendar year and,

        - two or more exceedances have occurred in the preceding 4 calendar years.

    - Effective from the date of installation of EDM monitoring equipment at the site:

If the measured number of block counted spills exceeds the limit specified in Table S3.1 then    the operator shall:

within 3 months of the trigger breach, submit an initial report to Natural Resources Wales for approval, with reference to the Best Practice Guidance in Schedule 4 Table 4.2, and, where appropriate proposals, including timeframes, for a full investigation into the operation of the overflow including a timetable for the investigation;

investigate the reasons for the exceedance in accordance with the approved investigation proposals and timescales;

An exceedance of the adjusted number of spills limit shall not be recorded as a failure if the operator is meeting the requirements of conditions 3.1.6 (a) and 2.3.7.

Emissions of substances not controlled by emission limits

For the activity A1 referenced in schedule 1, table S1.1 The operator shall take appropriate measures to minimise so far as reasonably practicable the polluting effects of the emissions of substances in the discharge not controlled by emission limits (excluding odour).

Monitoring

The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables in schedule 3 to this permit:

point source emissions specified in tables S3.1 and S3.4;

inlet quality specified in tables S3.1 and S3.4;

efficacy monitoring specified in tables S3.4 and S3.5.

The operator shall maintain records of all monitoring required by this permit.

Monitoring equipment, techniques, personnel and organisations employed for the emissions monitoring programme (and the environmental or other monitoring) specified in condition 3.3.1 shall have either MCERTS certification or MCERTS accreditation (as appropriate), where available, unless otherwise agreed in writing by Natural Resources Wales.

Accessible monitoring points shall be provided and maintained to enable the emissions monitoring programme (and the environmental or other monitoring) sampling/monitoring of emissions to be carried out at the monitoring points specified in table S3.4 of schedule 3 and shown marked on the site plans in schedule 7.

The monitoring programme for the parameters subject to schedule 3B shall be:

pre-scheduled to cover a calendar year and the programme recorded before the start of a calendar year sample period; and

spot samples collected at approximately equal intervals during the year, including samples from different days of the week and different times.  Approximately 10% of samples should be outside the normal sampling window which is 9am-3pm, Monday to Friday.

After becoming aware, or following a notification that a sample has not been taken on the schedule 3B Monitoring Programme pre-scheduled date, or is lost, or a result for that sample cannot be reported, the operator shall record the details and reschedule the sample.

The monitoring programme for the parameters subject to schedule 3C shall be:

- (a) 	pre-scheduled before each calendar year.

(b) 	Unless otherwise agreed in writing by the Natural Resources Wales, the operator shall submit the monitoring programme for the following calendar year to the Natural Resources Wales before the 1st of December; and

(c) 	samples must be collected at approximately equal intervals during the year from different days of the week and approximately 10% of samples should be taken at weekends

Unless otherwise agreed in writing by the Natural Resources Wales, after becoming aware, or following notification that a sample has not been taken on the schedule 3C Monitoring Programme pre-scheduled date, or is lost, or a result for that sample cannot be reported, the operator shall notify Natural Resources Wales of the missed event and the reschedule date as soon as reasonably practicable

Continuous recorders, with on-site visual display from which readings may be readily obtained, shall be provided and maintained by the operator and the following shall be measured and recorded at 15-minute intervals:

the instantaneous flow rate through each UV irradiation channel;

the instantaneous measured applied UV dose for each UV irradiation channel;

the number of operational UV lamps for each UV irradiation channel;

the instantaneous measured UV transmittance at the inlet to the UV irradiation plant;

any other parameters used in calculating the UV dose.

Effective from the date of installation of EDM monitoring equipment at the site: The Operator shall take appropriate measures to ensure that the event recorder remains fully operational at all times within the reference period specified in table S3.1. Any failures shall be remedied as soon as reasonably practicable.

- Total Daily Volumes shall be calculated from the average of the available ‘good’ 15-minute flow readings taken from midnight to midnight where;

Total Daily Volume (m3) = {Sum of ‘good’ readings (l/s) / number of ‘good’ readings} x {86,400 (s) /1000}.

Where there are 87 or more ‘good’ 15-minute flow readings the Total Daily Volume shall be reported as ‘good’, where there are 1 – 86 ‘good’ readings it shall be reported as ‘suspect’ and where there are no ‘good’ readings as ‘missing’.

- The operator shall record all failures of the flow measurement system and any other breaks in the flow record and the reasons for all issues, failures and breaks that lead to missing or suspect Total Daily Volume records and all steps taken to prevent a re-occurrence.

- There shall be no more than 37 days and/or no more than 14 consecutive days with ‘suspect’ or ‘missing’ Total Daily Volumes in a calendar year, unless otherwise agreed in writing by the Natural Resources Wales.

- All 15-minute flow readings shall be flagged as ‘good’, ‘suspect’ or ‘missing’ using an appropriate methodology set out in the operator`s flow monitoring quality management system.

Information

Records

All records required to be made by schedule 3, 4 and 5 to this permit shall:

be legible;

be made as soon as reasonably practicable;

if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made.

The operator shall maintain convenient access, in either electronic or hard copy, to the records, plan and management system required to be maintained by this permit.

Reporting

The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

Within the time period after the end of the reporting period specified in schedule 4 table S4.1 the operator shall, unless otherwise agreed in writing by Natural Resources Wales, submit reports of the monitoring and assessment carried out in accordance with the conditions of this permit, as follows:

in respect of the parameters and monitoring points specified in schedule 4 table S4.1;

[for the reporting periods specified in schedule 4 table S4.1 and] using the forms specified in schedule 4 table S4.2; and

giving the information from such results and assessments as may be required by the forms specified in those tables.

The operator shall supply the relevant Local Food Authority / Environmental Health Authority with an annual written report of the operation of the combined sewer overflow, to include date, start time and duration of each spill in accordance with the format provided by Natural Resources Wales.

Provision of this report will coincide with the annual classification under the EU Food Hygiene Regulations (852/853/854). The report shall cover the 12-month period 1 April – 31 March inclusive, and shall be provided by the end of the following May.

    - On request, the Operator shall supply Natural Resources Wales with written details of individual spill events.

Notifications

Natural Resources Wales shall be notified as soon as reasonably practicable following detection, within the site of the regulated facility of:

    - any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution; and

any breach of a limit specified in schedule 3 table S3.1 (including individual exceedances of limits which are covered by condition 3.1.2).

Any other significant adverse environmental effects, which may have been caused by the activity, shall also be notified to Natural Resources Wales as soon as reasonably practicable following detection.

The information provided under condition 4.3.1 shall be supported by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling specified in schedule 3B/3C, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

any change in the operator’s trading name, registered name or registered office address; and

any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

      - any change in the operator’s name or address; and

any steps taken with a view to the dissolution of the operator.

For the activity A1 referenced in schedule 1, table S1.1 where the operator proposes to make a change in the nature of the activity by increasing the concentration of, or the addition of, or allowing the introduction of, a substance to the activity to an extent that the operator considers could have a significant adverse environmental effect on the receiving waters, and the change is not permitted by emission limits specified within schedule 3 table S3.1 or the subject of an application for approval under the EP Regulations or under the terms of this permit:

Natural Resources Wales shall be notified in writing at least 14 days before the increase or addition or allowing the introduction; and

the notification shall contain a description of the proposed change.

The operator shall notify the Local Food Authority / Environmental Health Authority in the event of the discharge of sewage in an emergency from any discharge points listed in schedule 3 Table S3.2. Such notification must be made as soon as practicable and no later than 24 hours after the event, and shall detail the reasons why the situation occurred, and the actions taken by the operator.

The operator shall notify the Local Food Authority / Environmental Health Authority in the event of a discharge of sewage effluent which has not been subjected to the required UV dose as specified in condition 2.3.6 c) and d), or of power failure causing loss of secondary treatment.  Such notification must be made as soon as practicable and no later than 24 hours after the event, and shall detail the reasons why the situation occurred, and the actions taken by the operator.

For the activity A1 referenced in schedule 1, table S1.1 the operator shall inform Natural Resources Wales in writing of any change, or proposed change, to the population equivalent such as would make a material change to the application of the Regulations and shall, on request, inform the Natural Resources Wales in writing of the actual population equivalent.

Interpretation

In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “as soon as reasonably practicable”, in which case it may be provided by telephone.

# Schedule 1 – Operations

| Table S1.1 Activities |  |  |
| --- | --- | --- |
| Activity reference | Description of activity | Limits of specified activity |
| A1 | Discharge of Secondary treated sewage effluent with nutrient removal and PFET sewage effluent subject to disinfection by ultra violet irradiation via Outlet 1 - Discharge Point | Flows up to 599 l/s are subject to biologically treatment and disinfection by ultra violet irradiation. Flows in excess of 599 l/s up to 1249 l/s are subject to PFET and disinfection by ultra violet irradiation. |
| A2 | Discharge of settled storm sewage via Outlet 1 -Discharge Point | N/A |

| Table S1.2 Operating techniques |  |  |  |
| --- | --- | --- | --- |
| Activity reference | Description of documentation | Parts | Date Received |
| A1 & A2 | Llanelli WwTW - PFET Operating Technique | All | 09/08/2021 |

# Schedule 2 – Waste types, raw materials and fuels

Schedule 2 not in use.

# Schedule 3 – Emissions and monitoring

| Table S3.1 Point Source emissions to water (other than sewer) – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Effluents and discharge points | Parameter | Limit (including unit) | Reference Period | Limit of effective range | Monitoring frequency | Compliance Statistic |
| A1 Discharge of Secondary treated sewage effluent with nutrient removal and PFET sewage effluent subject to disinfection by ultra violet irradiation via Outlet 1 - Discharge Point | Dry weather flow | 25920 m3/day | Total daily volume | N/A | Continuous | Conditions 3.1.5 and 3.3.11 apply |
|  | 15-minute instantaneous or averaged flow | No limit set. Record as l/s | 15 minute | N/A | Continuous | N/A |
|  | Visible oil or grease | No significant trace present so far as is reasonably practicable | Instantaneous (visual examination ) | N/A | N/A | No significant trace (Condition 3.1.3 applies) |
|  | pH | 6 to 9 | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Minimum and maximum |
|  | ATU-BOD as O2 | 25 mg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Look up table (Conditions 3.1.2 and 3.1.3 apply) |
|  | ATU-BOD as O2 | 60 mg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum (Condition 3.1.3 applies) |
|  | Ammoniacal nitrogen (expressed as N) | 10 mg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Look up table (Conditions 3.1.2 and 3.1.3 apply) |
|  | Ammoniacal nitrogen (expressed as N) | 37 mg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum (Condition 3.1.3 applies) |
|  | Suspended solids (measured after drying at 105oC) | 60 mg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Look up table (Conditions 3.1.2 and 3.1.3 apply) |
|  | Total phosphorus as P | 1 mg/l | 24 hour composite | N/A | As specified in schedule 3B | Annual mean (Condition 3.1.3 applies) |
|  | Total iron as Fe | 4000 µg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum (Condition 3.1.2 and 3.1.3 apply) |
|  | Total iron as Fe | 8000 µg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum (Condition 3.1.3 applies) |
|  | Total Aluminium (Al) | 1000 µg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum (Condition 3.1.3 applies) |
|  | ATU-BOD as O2 (UWWTR) | Minimum of 70 % removal compared to influent | 24 hour composite | To be compliant a sample has to meet the 70% removal standard or the 25 mg/l limit not both | As specified in schedule 3C | Look up table (Conditions 3.1.2 and 3.1.4 apply) |
|  | ATU-BOD as O2 (UWWTR) | 25 mg/l |  |  |  |  |
|  | ATU-BOD as O2 (UWWTR) | 60 mg/l | 24 hour composite | This limit does not apply if a sample has met the 70% removal standard | As specified in schedule 3C | Maximum (Condition 3.1.4 applies) |
|  | COD as O2 (UWWTR) | Minimum of 75 % removal compared to influent | 24 hour composite | To be compliant a sample has to meet the 75% removal standard or the 125 mg/l limit not both | As specified in schedule 3C | Look up table (Conditions 3.1.2 and 3.1.4 apply) |
|  | COD as O2 (UWWTR) | 125 mg/l |  |  |  |  |
|  | COD as O2 (UWWTR) | 250 mg/l | 24 hour composite | This limit does not apply if a sample has met the 75% removal standard | As specified in schedule 3C | Maximum (Condition 3.1.4 applies) |
|  | Total nitrogen as N (UWWTR) | 15 mg/l | 24 hour composite | To be compliant a sample has to meet the 70% removal standard or the 15 mg/l limit not both | As specified in schedule 3C | Annual mean (Condition 3.1.4 applies) |
|  | Total nitrogen as N (UWWTR) | Minimum of 70% removal compared to influent |  |  |  |  |
| A2 Settled storm sewage via Outlet 1 -Discharge Point | Discharge start and end times | N/A | N/A | N/A | Whenever a discharge occurs | N/A |
|  | Block counted significant spills | 26 spills | 1st January to 31st December inclusive | N/A | 15 minutes | Condition 3.1.6 applies - the spills threshold is a trigger for investigation only and not a statistic for use in compliance of asset |
|  | Block counted spills | N/A | 1st January to 31st December inclusive | N/A | 15 minutes | N/A |
|  | Event recorder status operational | N/A | N/A | N/A | 15 minutes | N/A |

| Table S3.2 Discharge points |  |  |  |
| --- | --- | --- | --- |
| Effluent Name | Discharge Point | Discharge point NGR | Receiving water/Environment |
| A1 Discharge of Secondary treated sewage effluent with nutrient removal and PFET sewage effluent subject to disinfection by ultra violet irradiation via Outlet 1 - Discharge Point | Outlet 1 - Discharge point | SS 54576 97854 | Loughor Estuary |
| A2 Settled storm sewage |  |  |  |

| Table S3.3a Storm sewage discharge settings |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Effluent and discharge point | Description of discharge | Overflow setting l/s | Maximum size of solid matter | Screen aperture size | Minimum screen capacity flow l/s | Minimum storage capacity m3 (online) |
| A2 Settled storm sewage via Outlet 1 - Discharge Point | Settled storm sewage | 1249 | No greater than 6 mm in more than 1 dimension | 6 mm x 6 mm | All flows shall be screened | 3500 |

| Table S3.3b UV process details and process limits |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Effluents and discharge points | UV source | Dose type | Full dose mJ/cm2 (see condition 2.3.6(b)) | 50% dose mJ/cm2 (see condition 2.3.6(c)) | Effective dates | 5%ile transmittance |
| Discharge of Secondary treated sewage effluent with nutrient removal subject to disinfection by ultra violet irradiation via Outlet 1 - Discharge Point | Narrow spectrum low pressure artificial UV source with at least 85% of available UV radiation emitted in the wavelength range 250 to 260nm | Measured applied | 32 | 16 | 1st Jan - 31st Dec | 45% |
| PFET sewage effluent subject to disinfection by ultra violet irradiation via Outlet 1 - Discharge Point | Narrow spectrum low pressure artificial UV source with at least 85% of available UV radiation emitted in the wavelength range 250 to 260nm | Measured applied | 35.47 | 17.73 | 1st Jan - 31st Dec | 45% |

| Table S3.4 Monitoring points |  |  |  |
| --- | --- | --- | --- |
| Effluents and discharge points | Monitoring type | Monitoring point NGR | Monitoring point reference |
| A1 Discharge of Secondary treated sewage effluent with nutrient removal and PFET sewage effluent subject to disinfection by ultra violet irradiation via Outlet 1 - Discharge Point | Inlet monitoring point | SS 54098 98273 | Inlet Sample Point and Flow Monitoring Point |
|  | UWWTD final effluent sampling point | SS 54210 98128 | UWWTD Final Effluent Sampling Point |
|  | Final and PFET effluent sampling points | SS 54228 98112 | Final Effluent Sampling Point |
|  | MCERTS flow monitoring | SS 54098 98273 | Inlet Sample Point and Flow Monitoring Point |
|  | Pre UV treatment monitoring | SS 54202 98125 | Pre-UV Sampling point |
|  | Post UV treatment monitoring | SS 54201 98119 | Post-UV Sampling point |
|  | UV irradiation plant inlet monitoring as specified by condition 3.3.9 |  |  |
|  | PFET Pre UV treatment monitoring | SS 54165 98301 | PFET Pre-UV Sampling point |
|  | PFET Post UV treatment monitoring | SS 54161 98282 | PFET Post-UV Sampling point |
| A2 Settled storm sewage via Outlet 1 - Discharge Point | Storm effluent sampling | SS 54154 98127 | Storm Effluent Sampling Point |
|  | Event Duration Monitoring |  |  |

| Table S3.5 UV efficacy monitoring point requirements |  |  |
| --- | --- | --- |
| Monitoring type | Parameters | Monitoring frequency |
| PFET Pre UV treatment monitoring points | Escherichia coli Intestinal Enterococci Suspended solids | For 2 years from date of issue. As specified for sanitary parameters in table 3B. Monitoring for all parameters in table S3.5 to coincide. |
| Pre UV treatment monitoring point | No monitoring | As specified for sanitary parameters in table 3B. Monitoring for all parameters in table S3.5 to coincide. |
| UV irradiation plant inlet monitoring point | UV transmittance at 254nm |  |
| UV Irradiation channel monitoring points | Instantaneous flow rate Measured applied dose |  |
| Post UV treatment monitoring point | Escherichia coli Intestinal Enterococci Suspended solids |  |
| Final and PFET effluent sampling points | Escherichia coli Intestinal Enterococci Suspended solids |  |

Schedule 3A - Look up table

| Look up table |  |
| --- | --- |
| Number of samples taken in any period of 12 months | Maximum number of samples permitted to exceed limit for given parameter |
| 4–7 | 1 |
| 8–16 | 2 |
| 17–28 | 3 |
| 29–40 | 4 |
| 41–53 | 5 |
| 54–67 | 6 |
| 68–81 | 7 |
| 82–95 | 8 |
| 96–110 | 9 |
| 111–125 | 10 |
| 126–140 | 11 |
| 141–155 | 12 |
| 156–171 | 13 |
| 172–187 | 14 |
| 188–203 | 15 |
| 204–219 | 16 |
| 220–235 | 17 |
| 236–251 | 18 |
| 252–268 | 19 |
| 269–284 | 20 |
| 285–300 | 21 |
| 301–317 | 22 |
| 318–334 | 23 |
| 335–350 | 24 |
| 351–365 | 25 |

Schedule 3B - OSM tier 3 sampling frequency

| Parameter | ‘Normal frequency’ of samples per year | Reduced Sampling frequency after 12 consecutive months of numeric permit compliance, samples per year or pro rata over the remainder of a year | On numeric limit failure return to normal frequency as soon as reasonably practicable, samples per 12 months1 | Out of hours samples |
| --- | --- | --- | --- | --- |
| Sanitary | 24 | 12 | 24 | For 24 samples 2 out of hours samples per annum |
| Non sanitary | 12 | 12 | 12 | For 12 samples 1 out of hours sample per annum |

1 For UV efficacy monitoring use sanitary frequency and increase following failure of UV requirements only.

Schedule 3C – Urban Waste Water Treatment Regulations sampling frequency

| Population equivalent | Samples per year | Reduced sampling frequency after a year without an UWWTR exceedance or failure, samples per year | Following an UWWTR exceedance or failure return to the higher frequency in the year that follows, samples per year |
| --- | --- | --- | --- |
| 2,000 to 9,999 | 12 | 4 | 12 |
| 10,000 to 49,999 | 12 | N/A | N/A |
| 50,000 or over | 24 | N/A | N/A |

# Schedule 4 – Reporting

Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

| Table S4.1 Reporting of monitoring data |  |  |  |
| --- | --- | --- | --- |
| Parameter | Monitoring point reference | Reporting period | Period begins |
| Dry Weather Flow (daily flows total) | Inlet Sample Point and Flow Monitoring Point | Annually Report to be submitted within 2 months of the end of the calendar year | 1st January |
| 15-minute flow | Inlet Sample Point and Flow Monitoring Point | Reports to be provided to Natural Resources Wales upon request Report to be submitted within 28 days unless otherwise specified in writing by Natural Resources Wales | Upon request by Natural Resources Wales |
| UWWTR - ATU-BOD as O2, COD as O2, total nitrogen as N | UWWTD Final Effluent Sampling Point | Monthly Report to be submitted within 28 days | 1st of month |
| Operator Self Monitoring - ATU-BOD as O2, ammoniacal nitrogen (expressed as N), suspended solids (measured after drying at 105oC), total phosphorus as P, total iron as Fe Total Aluminium (Al), | Final Effluent Sampling Point | Quarterly Report to be submitted within 28 days | 1st of month |
| Operator Self Monitoring summary report | Final Effluent Sampling Point | Annually Report to be submitted within 2 months of the end of the calendar year | 1st January |
| UV disinfection measurements specified by condition 3.3.9 | Post-UV Sampling Point and PFET Pre & Post-UV Sampling Point | Quarterly Report to be submitted within 2 months | 1st January, 1st April, 1st July, 1st October |
| Disinfection efficacy Parameter required by condition 3.3.1 and table S3.5 | Post-UV Sampling Point and PFET Pre & Post-UV Sampling Point | Quarterly Report to be submitted within 2 months | 1st January, 1st April, 1st July, 1st October |
| UV disinfection performance reports | Post-UV Sampling Point and PFET Pre & Post-UV Sampling Point | Reports to be provided to Natural Resources Wales upon request Report to be submitted within 28 days unless otherwise specified in writing by Natural Resources Wales | Upon request by Natural Resources Wales |
| Settled storm overflow performance, including adjusted number of significant spills. | Storm Effluent Sampling Point | Annually Report to be submitted within 2 months | 1st January |
| Duration of significant spills | Storm Effluent Sampling Point | Annually Report to be submitted within 2 months | 1st January |
| Settled storm overflow performance, including adjusted number of total spills | Storm Effluent Sampling Point | Annually Report to be submitted within 2 months | 1st January |
| Duration of total spills | Storm Effluent Sampling Point | Annually Report to be submitted within 2 months | 1st January |
| Event recorder status operational | Storm Effluent Sampling Point | Annually Report to be submitted within 2 months | 1st January |
| Storm sewage discharge start and end times | Storm Effluent Sampling Point | Reports to be provided to Natural Resources Wales upon request | As specified by Natural Resources Wales |

| Table S4.2 Reporting forms |  |
| --- | --- |
| Parameter | Reporting format |
| Dry Weather Flow (daily flows total) | WISKI electronic format specified by Natural Resources Wales |
| 15-minute flow | WISKI electronic format specified by Natural Resources Wales |
| UWWTR – ATU-BOD as O2, COD as O2, total nitrogen as N | Electronic format specified by Natural Resources Wales |
| Operator Self Monitoring - ATU-BOD as O2, ammoniacal nitrogen (expressed as N), suspended solids (measured after drying at 105oC), total phosphorus as P, total iron as Fe Total Aluminium (Al) | Quarterly - Electronic format specified by Natural Resources Wales |
| Operator Self Monitoring summary Report | Annually - Summary report of compliance with the monitoring programme specified in Table 3.1 and schedule 3B in a format specified by Natural Resources Wales |
| Final Effluent UV disinfection measurements specified by condition 3.3.9 | Form as agreed in writing by Natural Resources Wales |
| Final Effluent Disinfection efficacy | Form as agreed in writing by Natural Resources Wales |
| Final Effluent UV disinfection performance reports | Form as agreed in writing by Natural Resources Wales |
| PFET UV disinfection measurements specified by condition 3.3.9 | Form as agreed in writing by Natural Resources Wales |
| PFET Effluent Disinfection efficacy | Form as agreed in writing by Natural Resources Wales |
| PFET Effluent UV disinfection performance reports | Form as agreed in writing by Natural Resources Wales |
| UV maintenance | Form as agreed in writing by Natural Resources Wales |
| Settled storm overflow performance including number and duration of block counted spills | Annual summary report or other form as agreed in writing by Natural Resources Wales |
| Settled storm overflow performance including number and duration of block counted significant spills | Annual summary report or other form as agreed in writing by Natural Resources Wales |
| Storm sewage discharge start and end times | Form as agreed in writing by Natural Resources Wales |
| Event recorder status | Annual summary report or other form as agreed in writing by Natural Resources Wales Percentage of time in the recording period that event duration monitoring equipment is operational |
| Investigation into the operation of the overflow | Electronic format as agreed in writing by Natural Resources Wales. |

# Schedule 5 – Notification

These pages outline the information that the operator must provide.

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

## Part A

| Permit Number | BP0252601 |
| --- | --- |
| Name of operator | Dŵr Cymru Cyfyngedig |
| Location of Facility | Llanelli Wastewater Treatment Works, Berwick Road, Bynea, Llanelli, SA14 9SN |
| Time and date of the detection |  |

| (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| To be notified within 24 hours of detection unless otherwise agreed in writing by Natural Resources Wales |  |
| Date and time of the event |  |
| Reference or description of the location of the event |  |
| Description of where any release into the environment took place |  |
| Substances(s) potentially released/type or nature of sewage released |  |
| Best estimate of the quantity or rate of release of substances and/or duration of discharge |  |
| Best estimate of the environmental impact of the discharge |  |
| Measures taken, or intended to be taken, to stop any emission |  |
| Description of the failure or accident. |  |

| (b) Notification requirements for the breach of a limit specified in schedule 3 table S3.1 (including individual exceedances of limits which are covered by condition 3.1.2) |  |
| --- | --- |
| The information specified below is to be notified to Natural Resources Wales as soon as reasonably practicable following detection. |  |
| Monitoring point reference/ source |  |
| Self monitoring regime (where relevant) | e.g. OSM/UWWTR |
| Type of failure | e.g. LUT failure/LUT exceedance/upper tier/other |
| Date of sample/event |  |
| Parameter |  |
| Result and units |  |
| Limit and units |  |

## Part B – to be submitted as soon as reasonably practicable unless otherwise agreed in writing by Natural Resources Wales

| Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident/breach/exceedance |  |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |

| Name* |  |
| --- | --- |
| Post |  |
| Signature |  |
| Date |  |

* authorised to sign on behalf of the operator

## Schedule 6 – Interpretation

“abnormal operating conditions” include but are not limited to, the circumstances described in Regulation 40(1) of, or paragraph 6(5) of Schedule 21 to, the Environmental Permitting Regulations 2016 (illegal discharge to sewer), unusual weather, or during a defined period where the permit authorising the permitted activity has been varied for reasons such as capital works construction.

“accident” means an accident that may result in pollution.

"annually" means once every year.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

For the activity A1 referenced in schedule 1, table S1.1 “appropriate measures” for the purposes of the emission of substances not controlled by emission limits condition (condition 3.2.1) do not require the operator to undertake treatment to a level beyond that specified in schedule 1 table S1.1, or to carry out routine monitoring for substances not controlled by emission limits.

“ATU-BOD as O2” means the biochemical oxygen demand (measured after 5 days at 20oC with nitrification suppressed by the addition of allylthiourea).

“block counted spill” one or more overflow events within a period of 12 hours or less will be considered to be one spill, one or more overflow events extending over a period of greater than 12 hours up to 36 hours will be considered to be 2 spills. Each subsequent 24 hour duration counts as 1 additional spill and the whole of the 24 hour block is included.

"event recorder" a recorder which records the start time and end time of individual overflow events.

“COD as O2” means the chemical oxygen demand (measured using the standard dichromate procedure).

“disinfection” means the use of a process designed specifically to reduce the number of viable, potentially infectious micro organisms in the effluent.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the permitted activities, which are not controlled by an emission limit.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“MCERTS” means Natural Resources Wales’ Monitoring Certification Scheme.

“minimum screen capacity flow” means the minimum flow passed through the screens to the outfall when the screen bypass operates.

“Natural Resources Wales” means the Natural Resources Body for Wales established by article 3 of the Natural Resources Body for Wales (Establishment) Order 2012. The Natural Resources Body for Wales (Functions) Order 2013 transferred the relevant functions of the Countryside Council for Wales, and functions of the Environment Agency and the Forestry Commission in Wales to the Natural Resources Body for Wales.

“overflow” for the purposes of schedule 7, overflow means the weir or orifice or other control which directs excess flow away from the continuation sewer when the flow exceeds the overflow setting due to rainfall and/or snowmelt.

“overflow setting” means the minimum flow passed forward to the continuation sewer when the overflow operates.

“PFET” means Peak Flow Equivalent Treatment.

"quarter" means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“significant pollution” means a high level impact incident as indicated by NRW incident classification system.

“unusual weather conditions” includes, but is not limited to:

low ambient temperatures, or the freezing of mechanical equipment in the works;

significant snow deposits;

tidal or fluvial flooding;

weather conditions causing unforeseen loss of power supply to the sewage treatment that could not be ameliorated by the reasonable provision and operation of standby generation facilities.

“Urban Waste Water Treatment (England and Wales) Regulations 1994(UWWTR)” means Urban Waste Water Treatment (England and Wales) Regulations 1994 SI 2841and the words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“year” means calendar year ending 31 December.

# Schedule 7 – Site plans

© Crown Copyright and database right 2021. Ordnance Survey licence number 100019741.

END OF PERMIT