Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

Mr Andrew Lewis and Mrs Debra Lewis

Ddole Farm Poultry Unit

Llanbister

Llandrindod Wells

Powys

LD1 6SS

Permit number

EPR/BV5220IH

Ddole Farm Poultry Unit

Permit number EPR/BV5220IH

Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.

Ddole Farm Poultry Unit is situated approximately 2 km southeast by east of the village of Llanbister in the county of Powys.  It is approximately centred on National Grid Reference SO 12810 72140.

The installation is operated by Mr Andrew Lewis and Mrs Debra Lewis and consists of six poultry houses for the production of broiler birds. The installation has a maximum capacity of 228,000 poultry places. The housing compromises of established housing and new housing.

Each house employs fan assisted ventilation, and biomass boilers for house heating.

The newer housing has been constructed to BAT with BWL accredited heat exchangers fitted to all houses, all condensate is contained in dirty water tanks. The heat exchangers mitigates the concentrations of ammonia generated from the increase in the number of places.

Birds are housed at day old and de populated at around thirty-two to forty days of age with approximately seven days empty, which gives between 6 to 8.5 cycles per annum, this is done on an all out all in basis.

Before bird arrival the houses are pre-warmed by hot water blown radiators, from biomass boilers. Floors are covered to a minimum depth of 2 cm of bulk wood shavings. Temperature and humidity is closely monitored on a daily basis to achieve the target level of 21º C and a relative humidity of 60-65%, to achieve litter with a dry matter content of between 60-70%, to minimise emissions.

Ventilation is controlled by maintaining a negative pressure system using roof-mounted high velocity fans with sidewall vent inlets. Water is via a nipple drinking system fitted with cups to reduce leakage and spills leading to drier litter.

Birds are fed a minimum of three diets during their growth, with gradually reducing levels of protein and phosphorous to increasing bird age.

Feed is delivered from an UKASTA accredited feed mill and blown into bulk feed bins situated at the ends of the houses, from the feed bins the feed is augured into the houses and distributed to the birds via a pan feeding system.

At depletion, the litter is removed from the site for use on the farm or sold. The farm is then pressure washed, disinfected, dried out prior to the cycle beginning again. All wash waters are contained in underground sealed tanks.

Fallen stock during the production cycle will be collected and recorded daily. These will be collected by a licensed collection agent under the National Fallen Stock Scheme.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Application received EPR/BV5220IH/A001 | Duly made 21/10/2003 | Application for an intensive farm installation. |
| Permit determined EPR/BV5220IH | 19/03/2004 | Permit issued to Mr Andrew Lewis and Mrs Debra Lewis. |
| Variation application EPR/BV5220IH/V002 | Duly made 17/08/2007 | Application to increase the capacity to 108,000 places comprising of 88,000 broilers and 20,000 free range places. Installation boundary increased. |
| Request for further information | 13/09/2007 |  |
| Variation determined EPR/BV5220IH | 21/10/2007 | Varied permit issued. |
| Variation application EPR/BV5220IH/V004 | 02/09/2008 | Application to increase the installation boundary to allow for free range activities. |
| Variation determined EPR/BV5220IH | 24/11/2008 | Varied permit issued. |
| Variation application EPR/BV5220IH/V005 | Duly made 27/10/2009 | Application for two new poultry sheds and increase numbers from 108,000 to 125,000. |
| Request for further information | 17/06/2010 | 17/06/2010 |
| Request for further information | 18/06/2010 | 19/06/2010 |
| Variation determined EPR/BV5220IH | 22/07/2010 | Varied permit issued. |
| Variation application EPR/BV5220IH/V006 | Duly made 04/01/2012 | Application to increase the bird numbers from 125,000 to 150,000. |
| Variation determined EPR/BV5220IH | 29/02/2012 | Varied permit issued. |
| Variation application EPR/BV5220IH/V007 | Duly made 19/11/2012 | Application to vary the permit to include four biomass boilers. |
| Variation determined EPR/BV5220IH | 14/01/2013 | Varied permit issued. |
| Application EPR/BV5220IH/V008 (variation and consolidation) | Duly made 10/03/2014 | Consolidation of existing permits with IED conditions and the installation of two additional broiler houses and two biomass boilers |
| Variation determined EPR/BV5220IH | 14/07/2014 | Varied and consolidated permit issued |
| Variation issued EPR/BV5220IH/V009 | 19/02/2018 | Natural Resources Wales Variation and consolidation following the implementation of the Intensive Farming BAT conclusions. |

End of introductory note

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

EPR/BV5220IH

This is the consolidated permit referred to in the variation and consolidation notice for application EPR/BV5220IH/V009 authorising,

Mr Andrew Lewis and Mrs Debra Lewis

whose principal office is

Vronganllwyd

Llanbister

Llandrindod Wells

Powys

LD1 6SS

to operate an installation at

Ddole Farm Poultry Unit

Llanbister

Llandrindod Wells

Powys

LD1 6SS

to the extent authorised by and subject to the conditions of this permit as set out in Appendix 1 and in Appendix 2.

| Signed | Date |
| --- | --- |
|  | 19 February 2018 |

Anna Lewis, Principal Permitting Officer

Authorised on behalf of Natural Resources Wales

APPENDIX 1

# Sector Specific Conditions

# Introductory note

## This introductory note does not form a part of the Condition set

### The conditions set out below are incorporated into this permit.

### End of introductory note.

# Management

## General management

### The operator shall manage and operate the activities:

#### in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

#### using sufficient competent persons and resources.

### Records demonstrating compliance with condition 1.1.1 shall be maintained.

### Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

## 1.2	Energy efficiency

### 1.2.1	The operator shall:

(a)	take appropriate measures to ensure that energy is used efficiently in the activities;

(b)	review and record at least every four years whether there are suitable opportunities to improve the energy efficiency of the activities; and

(c)	take any further appropriate measures identified by a review.

## 1.3    Efficient use of raw materials

1.3.1	The operator shall:

(a)	take appropriate measures to ensure that raw materials and water are used efficiently in the activities;

(b)	maintain records of raw materials, water, fuel and electrical energy used in the activities;

(c)	review and record at least every four years whether there are suitable alternative materials that could reduce environmental impact or opportunities to improve the efficiency of raw material and water use; and

(d)	take any further appropriate measures identified by a review.

## 1.4	Avoidance, recovery and disposal of wastes produced by the activities

    - The operator shall take appropriate measures to ensure that:

### (a)	the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied  to the generation of  waste by the activities; and

### (b)	any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

### (c)	where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

    - The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

# 2	Operations

2.1	Permitted activities

### 2.1.1	The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”).

### 2.2	The site

### See Appendix 2.

## 2.3	Operating techniques

#### 2.3.1	(a) 	The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

#### (b)	If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan , and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

#### 2.3.2	The operator shall maintain and implement a system to record the number of animal places and animal movements.

2.3.3	The operator shall maintain and implement a system to record the quantities of solid manure or slurry exported from the installation, as defined on the site plan in schedule 7.  The record shall include as a minimum:

- the date of export from the site

- the quantity exported

- details of the receiving site

### 2.3.4	The operator shall ensure that where waste produced by the activities is sent to a relevant waste operation, that operation is provided with the following information, prior to the receipt of the waste:

### (a)	the nature of the process producing the waste;

### (b)	the composition of the waste;

### (c)	the handling requirements of the waste;

### (d)	the hazardous property associated with the waste, if applicable; and

### (e)	the waste code of the waste.

### 2.3.5	The operator shall ensure that where waste produced by the activities is sent to a landfill site, it meets the waste acceptance criteria for that landfill.

2.3.6      The operator shall ensure that a diet formulation and a nutritional strategy is used that minimises as far as is practicable the amount of Phosphorus and Nitrogen excreted.

# 3	Emissions and monitoring

## 3.1	Emissions to water, air or land

3.1.1	There shall be no point source emissions to water, air or land except from the sources and emission points listed in tables S3.2, S3.3 and S3.4.

  - Emissions of substances not controlled by emission limits

    - Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

### 3.2.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

#### (b)	implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

#### 3.2.3	All liquids in containers, whose emission to water or land could cause pollution, shall be provided with secondary containment, unless the operator has used other appropriate measures to prevent or where that is not practicable, to minimise, leakage and spillage from the primary container.

    - Periodic monitoring shall be carried out at least once every 5 years for groundwater and 10 years for soil, unless such monitoring is based on a systematic appraisal of the risk of contamination.

## 3.3	Odour

#### 3.3.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour.

3.3.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to odour, submit to Natural Resources Wales for approval within the period specified, an odour management plan which identifies and minimises the risks of pollution from odour;

#### (b)	implement the approved odour management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.4	Noise and vibration

#### 3.4.1	Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.

3.4.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to noise and vibration, submit to Natural Resources Wales for approval within the period specified, a noise and vibration management plan which identifies and minimises the risks of pollution from noise and vibration;

#### (b)	implement the approved noise and vibration management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.5	Monitoring

### 3.5.1	The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables:

- emissions specified in tables S3.1, S3.2, S3.3 and S3.4.

3.5.2	The operator shall maintain records of all monitoring required by this permit including records of the taking and analysis of samples, instrument measurements (periodic and continual), calibrations, examinations, tests and surveys and any assessment or evaluation made on the basis of such data.

## 3.6	Pests

3.6.1 	The activities shall not give rise to the presence of pests which are likely to cause pollution, hazard or annoyance outside the boundary of the site. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved pests management plan, have been taken to prevent or where that is not practicable, to minimise the presence of pests on the site.

3.6.2 	The operator shall:

(a)          if notified by Natural Resources Wales, submit to Natural Resources Wales for approval within the period specified, a pests management plan which identifies and minimises risks of pollution, hazard and annoyance from pests;

(b)          implement the pests management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

# 4	Information

## 4.1	Records

    - All records required to be made by this permit shall:

- be legible;

- be made as soon as reasonably practicable;

- if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

- be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

- off-site environmental effects; and

- matters which affect the condition of the land and groundwater.

    - The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.

4.2	Reporting

4.2.1	The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

### 4.2.2	A report or reports on the performance of the activities over the previous year shall be submitted to Natural Resources Wales by 31 January (or other date agreed in writing by Natural Resources Wales) each year.  The report(s) shall include as a minimum:

#### (a)	a review of the results of the monitoring and assessment carried out in accordance with the permit including an interpretive review of that data;

#### (b)	the annual production /treatment data set out in schedule 4 table S4.2; and

#### (c)	the performance parameters set out in schedule 4 table S4.3 using the forms specified in table S4.4 of that schedule.

### 4.2.3	The operator shall, unless notice under this condition has been served within the preceding four years, submit to Natural Resources Wales, within six months of receipt of a written notice, a report assessing whether there are other appropriate measures that could be taken to prevent, or where that is not practicable, to minimise pollution.

## 4.3	Notifications

4.3.1	(a) 	In the event that the operation of the activities gives rise to an incident or accident which significantly affects or may significantly affect the environment, the operator  must immediately:

(i) 	inform Natural Resources Wales;

(ii)	take the measures necessary to limit the environmental consequences of such an incident or accident; and

(iii)	take the measures necessary to prevent further possible incidents or accidents.

(b)	In the event of a breach of any permit condition the operator must immediately:

(i) 	inform Natural Resources Wales; and,

(ii)	take the measures necessary to ensure that compliance is restored within the shortest possible time.

(c) 	In the event of a breach of permit condition which poses an immediate danger to human health or threatens to cause an immediate significant adverse effect on the environment, the operator must immediately suspend the operation of the activities or the relevant part of it until compliance with the permit conditions has been restored.

### 4.3.2       Any information provided under condition 4.3.1, shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

### 4.3.3	Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

### 4.3.4	Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

### Where the operator is a registered company:

- any change in the operator’s trading name, registered name or registered office address; and

#### (b)	any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

### Where the operator is a corporate body other than a registered company:

### (a) 	any change in the operator’s name or address; and

- any steps taken with a view to the dissolution of the operator.

### In any other case:

- the death of any of the named operators (where the operator consists of more than one named individual);

- any change in the operator’s name(s) or address(es); and

- any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

### 4.3.5	Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

- Natural Resources Wales shall be notified at least 14 days before making the change; and

- the notification shall contain a description of the proposed change in operation.

### 4.3.6	Natural Resources Wales shall be given at least 14 days notice before implementation of any part of the site closure plan.

## 4.4	Interpretation

### 4.4.1	In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

### 4.4.2	In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone.

### Schedule 1

### Operations

### See Appendix 2

### Schedule 2

### Waste types, raw materials and fuels

### See Appendix 2

## Schedule 3

## Emissions and Monitoring

| ### Table S3.1 BAT associated emission levels and monitoring requirements Note 1 and note 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ### Emission point ref. & location | ### Parameter | ### Source | ### Limit (incl. unit) | ### Reference period | ### Monitoring frequency | ### Monitoring standard or method |
| Weaners | Nitrogen (N) | Excreted | 4.0 kg N /animal place | Per year | Annually | Calculation by using a mass balance of nitrogen and phosphorus based on the feed intake, dietary content of crude protein, total phosphorus and animal performance or Estimation by using manure analysis for total nitrogen and total phosphorus content |
|  | Phosphorus (as P2O5) | Excreted | 2.2 kg P2O5 /animal place | Per year | Annually | As above |
|  | Ammonia as NH3 | Animal house | 0.53 kg NH3 /animal place | Per year | Annually | Estimation using emission factors |
| Fattening Pigs | Nitrogen (N) | Excreted | 13.0 kg N /animal place | Per year- | Annually | Calculation by using a mass balance of nitrogen and phosphorus based on the feed intake, dietary content of crude protein, total phosphorus and animal performance or Estimation by using manure analysis for total nitrogen and total phosphorus content |
|  | Phosphorus (as P2O5) | Excreted | 5.4 kg P2O5 /animal place | Per year | Annually | As above |
|  | Ammonia as NH3 | Animal house | 2.6 kg NH3 /animal place | Per year | Annually | Estimation using emission factors |
| Sows (including suckling piglets) | Nitrogen (N) | Excreted | 30.0 kg N /animal place | Per year | Annually | Calculation by using a mass balance of nitrogen and phosphorus based on the feed intake, dietary content of crude protein, total phosphorus and animal performance or Estimation by using manure analysis for total nitrogen and total phosphorus content |
|  | Phosphorus (as P2O5) | Excreted | 15 kg P2O5 /animal place | Per year | Annually | As above |
| Mating and gestating sows | Ammonia as NH3 | Animal house | 2.7 kg NH3 /animal place | Per year | Annually | Estimated using excretion factors |
| Farrowing sows (including suckling piglets) with crates. | Ammonia as NH3 | Animal house | 5.6 kg NH3 /animal place | Per year | Annually | Estimated using excretion factors |
| Laying hens | Nitrogen (N) | Excreted | 0.8 kg N /animal place | Per year | Annually | Calculation by using a mass balance of nitrogen and phosphorus based on the feed intake, dietary content of crude protein, total phosphorus and animal performance or Estimation by using manure analysis for total nitrogen and total phosphorus content |
|  | Phosphorus (as P2O5) | Excreted | 0.45 kg P2O5 /animal place | Per year | Annually | As above |
| Laying hens in Caged housing | Ammonia as NH3 | Animal house | 0.08 kg NH3 /animal place | Per year | Annually | Estimation by using emission factors |
| Laying hens in Non caged housing | Ammonia as NH3 | Animal house | 0.13 kg NH3 /animal place Note 3 | Per year | Annually | Estimation by using emission factors |
| Broilers | Nitrogen (N) | Excreted | 0.6 kg N /animal place | Per year | Annually | Calculation by using a mass balance of nitrogen and phosphorus based on the feed intake, dietary content of crude protein, total phosphorus and animal performance or Estimation by using manure analysis for total nitrogen and total phosphorus content |
|  | Phosphorus (as P2O5) | Excreted | 0.25 kg P2O5 /animal place | Per year | Annually | As above |
|  | Ammonia as NH3 | Animal house | 0.08 kg NH3 /animal place | Per year | Annually | Estimation by using emission factors |
| Ducks | Nitrogen (N) | Excreted | 0.8 kg N /animal place | Per year | Annually | Calculation by using a mass balance of nitrogen and phosphorus based on the feed intake, dietary content of crude protein, total phosphorus and animal performance or Estimation by using manure analysis for total nitrogen and total phosphorus content |
| Turkeys | Nitrogen (N) | Excreted | 2.3 kg N /animal place | Per year | Annually | Calculation by using a mass balance of nitrogen and phosphorus based on the feed intake, dietary content of crude protein, total phosphorus and animal performance or Estimation by using manure analysis for total nitrogen and total phosphorus content |
|  | Phosphorus (as P2O5) | Excreted | 1.0 kg P2O5 /animal place | Per Year | Annually | As above |

Note 1.  There are no BAT emission factors specified for pullets.

Note 2. The limits specified in this permit only apply after 20 February 2021

Note 3. For poultry houses operational on 21 February 2017 using a forced ventilation system and an infrequent manure removal (in case of deep litter with a manure pit), in combination with a measure achieving a high dry matter content of the manure, the upper end of the BAT-AEL is 0.25Kg NH3/ animal place / year.

# Table S3.2 – Point source emissions to air emissions limits and monitoring requirements - see Appendix 2.

# Table S3.3 – Point source emissions to water (other than sewer) – emission limits and monitoring – see Appendix 2

# Table S3.4 – Point source emissions to land – emission limits and monitoring – see Appendix 2

### Schedule 4

### Reporting

### See Appendix 2

# Schedule 5 - Notification

### These pages outline the information that the operator must provide.

### Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

## Part A

| ## Permit Number |  |
| --- | --- |
| ## Name of operator |  |
| ## Location of Facility |  |
| ## Time and date of the detection |  |

| ## (a) Notification requirements for any activity that gives rise to an incident or accident which significantly affects or may significantly affect the environment |  |
| --- | --- |
| ## To be notified within 24 hours |  |
| ## Date and time of the event |  |
| ## Reference or description of the location of the event |  |
| ## Description of where any release into the environment took place |  |
| ## Substances(s) potentially released |  |
| ## Best estimate of the quantity or rate of release of substances |  |
| ## Measures taken, or intended to be taken, to stop any emission |  |
| ## Description of the failure or accident. |  |

| ## (b) Notification requirements for the breach of a permit condition |  |
| --- | --- |
| ## To be notified within 24 hours |  |
| ## Emission point reference/ source |  |
| ## Parameter(s) |  |
| ## Limit |  |
| ## Measured value and uncertainty |  |
| ## Date and time of monitoring |  |
| ## Measures taken, or intended to be taken, to stop the emission |  |

| ### Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| ### Parameter | ### Notification period |

| ## (c) In the event of a breach of permit condition which poses an immediate danger to human health or threatens to cause an immediate significant adverse effect on the environment: |  |
| --- | --- |
| ## To be notified within 24 hours |  |
| ## Description of where the effect on the environment was detected |  |
| ## Substances(s) detected |  |
| ## Concentrations of substances detected |  |
| ## Date of monitoring/sampling |  |

## Part B - to be submitted as soon as practicable

| ### Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| ### Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| ### Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| ### The dates of any unauthorised emissions from the facility in the preceding 24 months. |  |

| ### Name* |  |
| --- | --- |
| ### Post |  |
| ### Signature |  |
| ### Date |  |

### * authorised to sign on behalf of the operator

# Schedule 6 - Interpretation

“accident” means an accident that may result in pollution.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act.

“BAT” means “best available techniques” as detailed in the Commission Implementing Decision (EU) 2017/302 published on 21 February 2017.

### “building” means a construction that has the objective of providing sheltering cover and minimising emissions of noise, particulate matter, odour and litter.

“emissions to land” includes emissions to groundwater.

“emission factors” are figures which have been agreed in writing with Natural Resources Wales and can be used in specific calculations.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission limit..

“excretion factors” are figures which have been agreed in writing with Natural Resources Wales and can be used in specific calculations.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.675 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

### “Industrial Emissions Directive” means DIRECTIVE 2010/75/EU OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 24 November 2010 on industrial emissions

### “Manure and slurry” have the following meaning:

- Manures may be either slurries or solid manures.

- Slurries consist of excreta produced by livestock whilst in a yard or building mixed with rainwater and wash water and, in some cases, waste bedding and feed. Slurries can be pumped or discharged by gravity.

- Slurry includes muck effluent, seepage from manure and wash water.

- Solid manures include farmyard manure (FYM) and comprise material from straw-based housing systems, excreta with lots of straw/sawdust/woodchips in it, or solids from mechanical separators.

- Most poultry systems produce solid manure (litter).

- Solid manure can generally be stacked.

### “Manure management plan” means the requirements described in Section 2.3 of SGN 6.09 How to Comply – Intensive Farming.

“Pests” means Birds, Vermin and Insects.

### “SGN How to comply – Intensive Farming” The EPR Sector Guidance Note 6.09 for intensive pig and poultry farmers, version 2 published January 2010.

“year” means calendar year ending 31 December.

### Schedule 7 – Site plan – see Appendix 2

# APPENDIX 2

# Site Specific Conditions

# Introductory note

## This introductory note does not form a part of the Condition set

### The conditions set out below are incorporated into this permit.

### End of introductory note.

# 2	Operations

#### 2.2	The site

### 2.2.1	The activities shall not extend beyond the site, being the land shown edged in green on the site plan at schedule 7 to this permit.

## 2.4	Improvement programme

### 2.4.1 	The operator shall complete the improvements specified in schedule 1 table S1.3 by the date specified in that table unless otherwise agreed in writing by Natural Resources Wales.

### 2.4.2	Except in the case of an improvement which consists only of a submission to Natural Resources Wales, the operator shall notify Natural Resources Wales within 14 days of completion of each improvement.

### Schedule 1 - Operations

| ### Table S1.1 activities |  |  |
| --- | --- | --- |
| ### Activity listed in Schedule 1 of the EP Regulations | ### Description of specified activity | ### Limits of specified activity |
| ### Section 6.9 A(1)(a) Rearing of poultry or pigs intensively in an installation with more than 40,000 places for poultry. | ### The rearing of poultry in a broiler facility. | ### To house a maximum capacity of 228,000 birds. From receipt of birds, raw materials and fuels onto the site, to removal of birds and associated wastes from site. |
| ### Directly Associated Activity |  |  |
| ### Biomass Boilers | Operation of six Biomass Boiler(s) (and associated heat exchangers) for site heating. | ### From receipt of raw materials and ### fuels, to release of combustion ### products to air and associated ### wastes removed from site. With a maximum thermal rated input of 1294.8 kW (4x 208.2kW and 2x 231 kW |

| ### Table S1.2 Operating techniques |  |  |
| --- | --- | --- |
| ### Description | ### Parts | ### Date Received |
| Application BV5220IH | Site Condition Report documentation submitted in support of Application | 21/10/2003 |
| Variation Application EP3131UC and HP3338MB | Site Condition Report documentation submitted in support of variation applications | 17/08/2007 |
| Application for Variation EPR/BV5220IH/V006 | Responses to Part C, of the application form and supporting documents. Odour management plan and Noise management plan in response to Part C2 and Part C3 of the variation application form. | 04/01/2012 |
| Application for Variation EPR/BV5220IH/V007 | Responses to Part C3 section 3 of the application form and referenced supporting documentation. | 19/11/2012 |
| Application for Variation EPR/BV5220IH/V008 | Responses to Part C3 section 3 Table 3: Technical Standards of the application form Appendix 5: Non-technical summary Appendix B: Environmental Management System Summary Biomass Operation – Management/Emergency Plan Minimising Waste and minimising water use Routine Maintenance Schedule | 10/03/2014 |

| Table S1.3 Improvement programme requirements |  |  |
| --- | --- | --- |
| Reference | Requirement | Date |
| IC1 | The operator shall comply with the requirements of the BAT Conclusions listed below, as far as they are applicable to the installation, on or before 20 February 2021. These BAT Conclusions for the intensive rearing of poultry or pigs were published as a Commission Implementing Decision (EU 2017/302) in the Official Journal of the EU on the 21 February 2017. BAT Conclusions: 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 19, 23, 24, 25, 26, 27, 29, 31, 32 | 20 February 2021 |
| IC2 | The operator shall submit a written report to Natural Resources Wales for approval, detailing compliance with the applicable parts of the BAT conclusions listed in IC1 | 20 May 2021 |

# Schedule 2 - Waste types, raw materials and fuels

| ### Table S2.1 Raw materials and fuels |  |
| --- | --- |
| ### Raw materials and fuel description | ### Specification |
| ### Clean virgin woodchip, miscanthus and straw | ### Not to include any waste material |

#### Schedule 3 – Emissions and Monitoring

| ### Table S3.2 Point source emissions to air – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ### Emission point ref. & location | ### Parameter | ### Source | ### Limit (incl. unit) | ### Reference period | ### Monitoring frequency | ### Monitoring standard or method |
| Roof fan outlets on buildings 1 to 6 shown on the site plan in Schedule 7 | None set | Poultry sheds 1 to 6 | None set | None set | None set | None set |
| Exhaust from the standby generator located between the west of poultry sheds 1 as shown on the site plan in Schedule 7 | None set | Standby generator | None set | None set | None set | None set |
| Exhaust from biomass boilers as shown on the site plan in Schedule 7 | None set | Biomass boilers 1 to 6 | None set | None set | None set | None set |

| ### Table S3.3 Point Source emissions to water (other than sewer) – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ### Emission point ref. & location | ### Parameter | ### Source | ### Limit (incl. unit) | ### Reference Period | ### Monitoring frequency | ### Monitoring standard or method |
| ### Outflow to ditch leading to the River Wye as shown on the site plan in Schedule 7 | ### None set | ### Roof and yard drainage from sheds 3 to 6 | ### None set | ### None set | ### None set | ### None set |

| ### Table S3.4 Point Source emissions to land – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ### Emission point ref. & location | ### Parameter | ### Source | ### Limit (incl. unit) | ### Reference Period | ### Monitoring frequency | ### Monitoring standard or method |
| Outlets to soakaway located to the south east as shown on the site plan in Schedule 7 | ### None set | ### Clean, uncontaminated rainwater draining from the roofs of poultry houses and yard area | ### None set | ### None set | ### None set | ### None set |
| Outlets to soakaway located to the west as shown on the site plan in Schedule 7 | ### None set | ### Clean, uncontaminated rainwater draining from the roofs of poultry houses and yard area | ### None set | ### None set | ### None set | ### None set |
| Outlet to swale located to the south east as shown on the site plan in Schedule 7 | ### None set | ### Clean, uncontaminated rainwater draining from storm overflow | ### None set | ### None set | ### None set | ### None set |

# Schedule 4 - Reporting

Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

| Table S4.1 Reporting of monitoring data |  |  |  |
| --- | --- | --- | --- |
| Parameter | Emission or monitoring point/reference | Reporting period | Period begins |
| As detailed in table S3.1 and as applicable to your operation | As detailed in table S3.1 and as applicable to your operation | Annually | 1 January |

| Table S4.2: Annual production/treatment |  |
| --- | --- |
| Parameter | Units |
| Live birds removed from site | Birds |

| Table S4.3 Performance parameters |  |  |
| --- | --- | --- |
| Parameter | Frequency of assessment | Units |
| Ammonia | Annually | kg NH3/animal place/year |
| Total Nitrogen excreted | Annually | kg N excreted/animal place/year |
| Total Phosphorus excreted | Annually | kg P2O5 excreted/animal place/year |

| Table S4.4 Reporting forms |  |  |
| --- | --- | --- |
| Media/parameter | Reporting format | Date of form |
| kg N excreted/animal place/year and kg P2O5 excreted/animal place/year | Form N&P Excretion 1 or other form as agreed in writing by Natural Resources Wales | 19 February 2018 |
| kg NH3/animal place/year | Form Air 1 or other form as agreed in writing by Natural Resources Wales | 19 February 2018 |

# Schedule 7 - Site plan

© Crown Copyright and database right 2017. Ordnance Survey licence number 100019741.

### Site layout/drainage plan (not to scale):

END OF PERMIT