Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

Mills Poultry Limited

Minffordd Poultry Farm

Llangadfan

Welshpool

Powys

SY21 0PL

Permit number

EPR/MP3939UM

Minffordd Poultry Farm

Permit number EPR/MP3939UM

Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.

Minffordd Poultry Farm is situated approximately 0.5 kilometres north east of the village of Llangadfan, Powys. The installation is approximately centred on National Grid Reference SJ 01314 10911.

The installation is operated by Mills Poultry Limited and comprises six poultry houses, numbered 1 to 6. All manure is exported from the installation for spreading on land owned by third parties. There will be four biomass boilers on site with a total aggregated thermal input of 1,789Kwth.

The six poultry houses provide a combined capacity for 195,000 bird places.

Birds will be brought in from a hatchery and transported to a processing plan at the end of the growing period. Both the hatchery and processing plant are at different locations and run by different operators. The average cycle length is 63 days.

The site is comprised of six poultry sheds, two of which have side inlets and roof ridge extraction fans, two have ridge inlets and side extraction fans and the remaining two have high-velocity ridge extraction fans. All are sited on concrete bases with associated feed storage in enclosed, galvanised steel bins and fibre glass bins.

Birds that die during the production cycle are recorded and removed from houses on a daily basis. Carcasses are held in vermin proof bins prior to collection. At the end of the growing cycle, wash water is held in underground tanks prior to spreading. Lightly contaminated roof water from all buildings will be collected via a combination of stone soakaways and French drains running parallel to the poultry sheds.  Used litter is removed from site and spread on separately owned agricultural land.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Application received | Duly made 29/01/07 | Application for rearing of poultry. intensively in an installation with more than 40,000 places |
| Permit determined EPR/MP3939UM | 06/07/07 | Original permit issued to Mills Poultry Limited |
| Application EPR/MP3939UM/V002 (variation) | Duly made 26/04/11 | Application to increase the number of places for broiler rearing by 116%. |
| Variation determined EPR/MP3939UM | 20/07/11 | Varied permit issued. |
| Application EPR/MP3939UM/V003 | Duly made 19/12/16 | Application to add four biomass boilers with a total thermal input of 1,789KWth. |
| Variation determined EPR/MP3939UM/V003 | 13/03/17 | Varied permit issued |

End of introductory note

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

EPR/MP3939UM

This is the consolidated permit referred to in the variation and consolidation notice for application EPR/AP3230UK/V003 authorising,

Mills Poultry Limited (“the operator”),

whose registered office is

Minffordd

Llangadfan

Welshpool

Powys

SY21 0PL

company registration number 04553061

to operate an installation at

Minffordd Poultry Farm

Minffordd

Llangadfan

Welshpool

Powys

SY21 0PL

to the extent authorised by and subject to the conditions of this permit.

| Name | Date |
| --- | --- |
|  | 13/03/2017 |

Victoria Seller

Authorised on behalf of Natural Resources Wales

Conditions

# Management

## General management

### The operator shall manage and operate the activities:

#### in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

#### using sufficient competent persons and resources.

### Records demonstrating compliance with condition 1.1.1 shall be maintained.

### Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

## 1.2	Energy efficiency

### 1.2.1	The operator shall:

(a)	take appropriate measures to ensure that energy is used efficiently in the activities;

(b)	review and record at least every four years whether there are suitable opportunities to improve the energy efficiency of the activities; and

(c)	take any further appropriate measures identified by a review.

## 1.3	Efficient use of raw materials

1.3.1	The operator shall:

(a)	take appropriate measures to ensure that raw materials and water are used efficiently in the activities;

(b)	maintain records of raw materials and water used in the activities;

(c)	review and record at least every four years whether there are suitable alternative materials that could reduce environmental impact or opportunities to improve the efficiency of raw material and water use; and

(d)	take any further appropriate measures identified by a review.

## 1.4	Avoidance, recovery and disposal of wastes produced by the activities

    - The operator shall take appropriate measures to ensure that:

### (a)	the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied  to the generation of  waste by the activities; and

### (b)	any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

### (c)	where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

    - The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

# 2	Operations

2.1	Permitted activities

### 2.1.1	The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”).

#### 2.2	The site

### 2.2.1	The activities shall not extend beyond the site, being the land shown edged in green on the site plan at schedule 7 to this permit.

## 2.3	Operating techniques

#### 2.3.1	(a) 	The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

#### (b)	If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan , and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

#### 2.3.2	The operator shall maintain and implement a system to record the number of animal places and animal movements.

2.3.3	Any raw materials or fuels listed in schedule 2 table S2.1 shall conform to the specifications set out in that table.

2.3.4	The operator shall maintain and implement a system to record the quantities of solid manure or slurry exported from the installation, as defined on the site plan in schedule 7.  The record shall include as a minimum:

- the date of export from the site

- the quantity exported

- details of the receiving site

### 2.3.5	The operator shall ensure that where waste produced by the activities is sent to a relevant waste operation, that operation is provided with the following information, prior to the receipt of the waste:

### (a)	the nature of the process producing the waste;

### (b)	the composition of the waste;

### (c)	the handling requirements of the waste;

### (d)	the hazardous property associated with the waste, if applicable; and

### (e)	the waste code of the waste.

### 2.3.6	The operator shall ensure that where waste produced by the activities is sent to a landfill site, it meets the waste acceptance criteria for that landfill.

# 3	Emissions and monitoring

## 3.1	Emissions to water, air or land

### 3.1.1	There shall be no point source emissions to water, air or land except from the sources and emission points listed in schedule 3 tables S3.1, S3.2 and S3.3.

  - Emissions of substances not controlled by emission limits

    - Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

### 3.2.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

#### (b)	implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

#### 3.2.3	All liquids in containers, whose emission to water or land could cause pollution, shall be provided with secondary containment, unless the operator has used other appropriate measures to prevent or where that is not practicable, to minimise, leakage and spillage from the primary container.

    - Periodic monitoring shall be carried out at least once every 5 years for groundwater and 10 years for soil, unless such monitoring is based on a systematic appraisal of the risk of contamination

## 3.3	Odour

#### 3.3.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour.

## 3.4	Noise and vibration

#### 3.4.1	Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.

## 3.5	Monitoring

### 3.5.1	The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables in schedule 3 to this permit:

- point source emissions specified in tables S3.1, S3.2 and S3.3;

3.5.2	The operator shall maintain records of all monitoring required by this permit including records of the taking and analysis of samples, instrument measurements (periodic and continual), calibrations, examinations, tests and surveys and any assessment or evaluation made on the basis of such data.

## 3.6	Pests

3.6.1 	The activities shall not give rise to the presence of pests which are likely to cause pollution, hazard or annoyance outside the boundary of the site. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved pests management plan, have been taken to prevent or where that is not practicable, to minimise the presence of pests on the site.

3.6.2 	The operator shall:

(a)          if notified by Natural Resources Wales, submit to Natural Resources Wales for approval within the period specified, a pests management plan which identifies and minimises risks of pollution, hazard and annoyance from pests;

(b)          implement the pests management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

# 4	Information

## 4.1	Records

    - All records required to be made by this permit shall:

- be legible;

- be made as soon as reasonably practicable;

- if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

- be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

- off-site environmental effects; and

- matters which affect the condition of the land and groundwater.

    - The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.

4.2	Reporting

4.2.1	The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

### 4.2.2	The operator shall, unless notice under this condition has been served within the preceding four years, submit to Natural Resources Wales, within six months of receipt of a written notice, a report assessing whether there are other appropriate measures that could be taken to prevent, or where that is not practicable, to minimise pollution.

## 4.3	Notifications

4.3.1	(a) 	In the event that the operation of the activities gives rise to an incident or accident which significantly affects or may significantly affect the environment, the operator  must immediately:

(i) 	inform Natural Resources Wales;

(ii)	take the measures necessary to limit the environmental consequences of such an incident or accident; and

(iii) 	take the measures necessary to prevent further possible incidents or accidents.

(b)	 In the event of a breach of any permit condition the operator must immediately:

(i) 	inform Natural Resources Wales; and

(ii) 	take the measures necessary to ensure that compliance is restored within the shortest possible time.

(c) 	In the event of a breach of permit condition which poses an immediate danger to human health or threatens to cause an immediate significant adverse effect on the environment, the operator must immediately suspend the operation of the activities or the relevant part of it until compliance with the permit conditions has been restored.

### 4.3.2       Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

### 4.3.3	Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

### 4.3.4	Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

### Where the operator is a registered company:

- any change in the operator’s trading name, registered name or registered office address; and

#### (b)	any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

### Where the operator is a corporate body other than a registered company:

### (a) 	any change in the operator’s name or address; and

- any steps taken with a view to the dissolution of the operator.

### In any other case:

- the death of any of the named operators (where the operator consists of more than one named individual);

- any change in the operator’s name(s) or address(es); and

- any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

### 4.3.5	Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

- Natural Resources Wales shall be notified at least 14 days before making the change; and

- the notification shall contain a description of the proposed change in operation.

### 4.3.6	Natural Resources Wales shall be given at least 14 days notice before implementation of any part of the site closure plan.

## 4.4	Interpretation

### 4.4.1	In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

### 4.4.2	In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone

### Schedule 1 - Operations

| Table S1.1 activities |  |  |
| --- | --- | --- |
| Activity listed in Schedule 1 of the EP Regulations | Description of specified activity | Limits of specified activity |
| Section 6.9 A(1)(a)(i) Rearing of poultry intensively in an installation with more than 40,000 places | The rearing of poultry in a facility with a capacity for 195,000 broiler places. | From receipt of birds, raw materials and fuels onto the site to removal of birds and associated wastes from site |
| Directly Associated Activity |  |  |
| Biomass Boiler | Operation of four Biomass Boilers with an aggregated thermal rated input not exceeding 1,789kwth, for site heating requirements, burning biomass fuel not comprising waste or animal carcasses. | From receipt of raw materials and fuels, to release of combustion products to air and associated wastes removed from site. |

| Table S1.2 Operating techniques |  |  |
| --- | --- | --- |
| Description | Parts | Date Received |
| Application | The responses to sections B2.3.1, B2.3.2, B2.3.3, B2.6.1, B2.6.2, B2.6.3, B2.7.1, B2.7.2 and site plan B1.3/009 in the Application. | 29/01/07 |
| ### Information relating to application to vary permit (V002) | ### Responses provided in the application form Part C2 sections 2b Table 1, 5a, 5c and 6, and Part C3 Table 1a, Table 2 (except incorrect reference to point source emissions to land), Table 3 and Table 4 and the Application report referenced therein. Drainage details shown in amended site drainage plans Minffordd clean water drainage system and Minffordd dirty water drainage system, submitted by email 21/06/11. | 02/06/11 31/03/11 ### 21/06/11 |
| ### Application EPR/MP3939UM/V003 | Responses to Part B3 section 3 of the application form and referenced supporting documentation. The contents of the documents entitled ”Odour Management Plan” and “Noise Management Plan” | 7/10/16 |

| Table S1.3 Improvement programme requirements |  |  |
| --- | --- | --- |
| Reference | Requirement | Date |
| IC1 | A written plan shall be submitted to NRW for approval, following a review of all site drainage at the installation. The plan should take into account the appropriate measures for the management of drainage systems and run-off in S3.3 of TGN How to Comply, Version 1, including specific reference to collection and treatment of drainage from the ventilation of animal houses, and include a timetable of any improvements to the drainage system. The notification requirements of condition 2.5.2 shall be deemed to have been complied with on submission of the plan. The plan shall be implemented by the operator from the date of approval in writing by NRW subject to such amendments or additions as notified by NRW. | Completed |
| IC2 | A written plan shall be submitted to NRW for approval following a reviw of existing poultry housing and management practces at the installation. The plan shall take into account the appropriate measures in S6.2.1 & S6.2.2 of TGN How to Comply, Version 1. The plan shall identif measures to reduce emissions to all media, the likely cost of such measures and a proposed timetable for their implementation. The notification requirements of condition 2.5.2 shall be deemed to have been complied with on submission of the plan. The plan shall be implemented by the operator from the date of approval in writing by NRW subject to such amendments or additions as notified by NRW. | Completed |

Schedule 2 - Waste types, raw materials and fuels

| Table S2.1 Raw materials and fuels |  |
| --- | --- |
| Raw materials and fuel description | Specification |
| ### Virgin timber (including wood chips and pellets), miscanthus or straw | ### Not to include any waste material |

# Schedule 3 – Emissions and monitoring

| ### Table S3.1 Point source emissions to air – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ### Emission point ref. & location | ### Parameter | ### Source | ### Limit (incl. unit) | ### Reference period | ### Monitoring frequency | ### Monitoring standard or method |
| Roof fan outlets on buildings shown on the plan reference 4752/1 dated 6/06/09. Provided with the variation application (V002). | ### --- | ### Poultry sheds 1, 4, 5 and 6 | ### --- | ### --- | ### --- | ### --- |
| Side fan outlets on buildings shown on the plan reference 4752/1 dated 6/06/09. Provided with the variation application (V002). | ### --- | ### Poultry sheds 2 and 3 | ### --- | ### --- | ### --- | ### --- |
| Vent from LPG tank as shown on the site plan reference B1.3/009. Provided with the original application. | ### --- | ### LPG Tanks | ### --- | ### --- | ### --- | ### --- |
| Vents from oil tanks as shown on the site plan reference B1.3/009. Provided with the original application. | ### --- | ### Oil Tank | ### --- | ### --- | ### --- | ### --- |
| Emissions from Biomass Boiler 1 in biomass boiler building as shown on the site plan in the application. | ### --- | ### Biomass Boiler 1 | ### --- | ### --- | ### --- | ### --- |
| Emissions from Biomass Boiler 2 in biomass boiler building as shown on the site plan in the application. | ### --- | ### Biomass Boiler 2 | ### --- | ### --- | ### --- | ### --- |
| Emissions from Biomass Boiler 3 in biomass boiler building as shown on the site plan in the application. | ### --- | ### Biomass Boiler 3 | ### --- | ### --- | ### --- | ### --- |
| Emissions from Biomass Boiler 4 in biomass boiler building as shown on the site plan in the application. | ### --- | ### Biomass Boiler 4 | ### --- | ### --- | ### --- | ### --- |

| ### Table S3.2 Point Source emissions to water (other than sewer) – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ### Emission point ref. & location | ### Parameter | ### Source | ### Limit (incl. unit) | ### Reference Period | ### Monitoring frequency | ### Monitoring standard or method |
| Outlets to the drain leading to Afon Banwy as shown on the plan ‘Minffordd clean water drainage system’ submitted by email 21/06/11 to support the variation application (V002). | ### --- | ### Clean yard surface water | ### --- | ### --- | ### --- | ### --- |

| ### Table S3.3 Point source emissions to land– emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ### Emission point ref. & location | ### Parameter | ### Source | ### Limit ### (incl. Unit) | ### Reference period | ### Monitoring frequency | ### Monitoring standard or method |
| Soakaway as shown on the plan ‘Minffordd clean water drainage system’ submitted by email 21/06/11 to support the variation application (V002). | ### --- | Roof water from poultry sheds 2 and 3 | ### --- | ### --- | ### --- | ### --- |
| French drains running alongside poultry sheds as shown on the plan ‘Minffordd clean water drainage system’ submitted by email 21/06/11 to support the variation application (V002). | ### --- | Roof water from poultry sheds 1, 4, 5 and 6 | ### --- | ### --- | ### --- | ### --- |

# Schedule 4 - Reporting

### There is no reporting under this schedule.

# Schedule 5 - Notification

### These pages outline the information that the operator must provide.

### Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

## Part A

| ## Permit Number |  |
| --- | --- |
| ## Name of operator |  |
| ## Location of Facility |  |
| ## Time and date of the detection |  |

| ## (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Date and time of the event |  |
| ## Reference or description of the location of the event |  |
| ## Description of where any release into the environment took place |  |
| ## Substances(s) potentially released |  |
| ## Best estimate of the quantity or rate of release of substances |  |
| ## Measures taken, or intended to be taken, to stop any emission |  |
| ## Description of the failure or accident. |  |

| ## (b) Notification requirements for the breach of a limit |  |
| --- | --- |
| ## To be notified within 24 hours of detection unless otherwise specified below |  |
| ## Emission point reference/ source |  |
| ## Parameter(s) |  |
| ## Limit |  |
| ## Measured value and uncertainty |  |
| ## Date and time of monitoring |  |
| ## Measures taken, or intended to be taken, to stop the emission |  |

| ### Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| ### Parameter | ### Notification period |

| ## (c) Notification requirements for the detection of any significant adverse environmental effect |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Description of where the effect on the environment was detected |  |
| ## Substances(s) detected |  |
| ## Concentrations of substances detected |  |
| ## Date of monitoring/sampling |  |

## Part B - to be submitted as soon as practicable

| ### Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| ### Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| ### Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| ### The dates of any unauthorised emissions from the facility in the preceding 24 months. |  |

| ### Name* |  |
| --- | --- |
| ### Post |  |
| ### Signature |  |
| ### Date |  |

### * authorised to sign on behalf of the operator

# Schedule 6 - Interpretation

“accident” means an accident that may result in pollution.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act.

### “building” means a construction that has the objective of providing sheltering cover and minimising emissions of noise, particulate matter, odour and litter.

“emissions to land” includes emissions to groundwater.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission limit..

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2010 No.675 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

### “Industrial Emissions Directive” means DIRECTIVE 2010/75/EU OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 24 November 2010 on industrial emissions

### “Manure and slurry” have the following meaning:

- Manures may be either slurries or solid manures.

- Slurries consist of excreta produced by livestock whilst in a yard or building mixed with rainwater and wash water and, in some cases, waste bedding and feed. Slurries can be pumped or discharged by gravity.

- Slurry includes duck effluent, seepage from manure and wash water.

- Solid manures include farmyard manure (FYM) and comprise material from straw-based housing systems, excreta with lots of straw/sawdust/woodchips in it, or solids from mechanical separators.

- Most poultry systems produce solid manure (litter).

- Solid manure can generally be stacked.

“Biomass” means:

a)	vegetable matter from agriculture and forestry (waste material);

b)	vegetable waste from the food processing industry, if the heat generated is  recovered (waste material);

c)	fibrous vegetable waste from virgin pulp production and from production of paper from pulp, if it is co-incinerated at the place of production and the heat generated is recovered (waste material);

d)	cork waste (waste material);

e)	wood waste with the exception of wood waste which may contain halogenated organic compounds or heavy metals as a result of treatment with wood preservatives or coating, and which includes in particular such wood waste originating from construction and demolition waste (waste material); and

f)	virgin wood, wood chips and pellets (not waste material).

### “quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

### “SGN How to comply – Intensive Farming” The EPR Sector Guidance Note 6.09 for intensive pig and poultry farmers, version 2 published January 2016.

“year” means calendar year ending 31 December.

Where a minimum limit is set for any emission parameter, for example pH, reference to exceeding the limit shall mean that the parameter shall not be less than that limit.

### Unless otherwise stated, any references in this permit to concentrations of substances in emissions into air means:

- in relation to emissions from combustion processes, the concentration in dry air at a temperature of 273K, at a pressure of 101.3 kPa and with an oxygen content of 3% dry for liquid and gaseous fuels, 6% dry for solid fuels; and/or

- in relation to emissions from non-combustion sources, the concentration at a temperature of 273K and at a pressure of 101.3 kPa, with no correction for water vapour content

# Schedule 7 - Site plan

#### Installation boundary plan

### Crown Copyright. All rights reserved. Natural Resources Wales, 100019742, 2017.

### END OF PERMIT