Non-Technical Summary

Blaenbwch Farm poultry Unit is applying for a permit for 100,000 bird places for the production of broilers, in two poultry houses.
Birds will be housed at day old and depopulated at the end of the rearing cycle, this will be done on an all-out all in basis. There will be approximately one cycle per annum. 
Before bird arrival the house floors will be covered to a sufficient depth of bulk shavings. Temperature and humidity will be closely monitored on a daily basis to achieve bird comfort and a relative humidity of 55-60%, this should achieve litter with a high dry matter content, which is important to minimising emissions. Ventilation is controlled by temperature, ventilation will be passed through air scrubbing units fitted to each house at the western gable ends to reduce emissions. Heating will be provided by hot water blown air radiators produced by LPG boilers.
Water is via a nipple drinking system fitted with cups to reduce leakage and spills leading to drier litter.

Birds are fed a minimum of three diets during their cycle, with gradually reducing levels of protein and phosphorous as bird age increases.

Feed is delivered from a UKASTA accredited feed mill and blown into bulk feed bins located adjacent to the houses, from the feed bins the feed is augered into the houses and distributed to the birds via a pan feeding system.

Fallen stock will be recorded daily and securely stored in vermin proof containers awaiting regular collection by a licenced renderer.

At depletion any remaining litter will be removed from the site and used on operator controlled land as fertiliser. The site will then be pressure washed, disinfected and dried out prior to the cycle beginning again. All wash waters will be contained in sealed underground tanks.

The above measures are designed to reduce emissions, trees and hedges will trap dust particles reducing odour. Ammonia emissions will be reduced by reduced protein feed, maintaining good litter conditions with high dry matter content Containment of wash waters will prevent pollutants being released to the environment.

Records of tonnages of litter and wash water removal are recorded, wash water will be spread on operator controlled land.
