# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for 9xDewatering Activities for Dinorwig to Pentir 400kV Cable Duct Installation along A4086 operated by Morgan Sindall Group PLC.

The application number is PAN-015202

The permit number is EPR/CB3394HT

The discharge is for site drainage resulting from rainfall and groundwater intrusion into working trenches. Limits are set to control the discharge rate, suspended solid concentration and pH.

Morgan Sindall on behalf of National Grid are installing a new underground electricity cable to connect the hydroelectric power station at Llanberis to the national grid node at Pentir.

Part of the route for the new cable will be installed within the A4086  along a section between Llanberis and Craig yr Undeb to the north. The road runs alongside Llyn Padarn and Llwyn Y Coed, both sites of special scientific interest. The lake is sensitive to silt.

The new cable will be installed beneath the road surface. A 12 to 20m length will be installed each day by excavating a trench, placing the cement bound sand (CBS), that requires a dry trench and is a dry mix, cable ducting and backfilling with CBS with tiles placed over the ducting and then reinstating the road surface. The works will move northwards along the road.

Any dewatering will be through silt sock to reduce turbidity of discharged water discharged via the highway drainage system which discharges to the adjacent land via a drainage ditch and then soaks into the surrounding land.

In total nine discharge locations will be required as the works progress along the A4806.

It is important to not only two discharge point will be active at any time as only Two 12 metre excavation trenchs will be open at any time.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to consultation, web publicising | There were no responses to web publicising or consultation with Anglesey council ecologist. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the site has been carried out as part of the permitting process. We consider that the application will not affect the features of the site Formal consultation has been carried out with Anglesey local authority ecologist. No response was given to the consultation. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. The proposed discharge locations are within screening distance used in the Risk Screening, but are not hydrologically connected with the activity, or within close proximity. Therefore no impact pathway exists. A copy of the risk screening report and Form1 can be found on the DMS file PAN-015202 | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant suspended sediment, which is controlled to ensure current water Quality of Llyn Padarn is maintained. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. Techniques used are appropriate and comply with requirements set out in TGN EPR 7.01 The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. - Suspended solids (measured after drying At 105o C): 50 mg/l - pH: 6 to 9 The above limits have been set to ensure the current water quality of Llyn Padarn is maintained, thus protecting the status of the SSSI and associated features. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |