WASTE AND WATER 2015

Waste disposal of bulk and spent tanks remains the same as before the amount of waste has increased with increased production. Also with the extension to the site we disposed of many chemicals as possible last year. Also with the extension we will need to dispose of about 150 tons of soil. As there is some contamination and there is Japanese Knot weed within 7 meters of the site this will also have to go as special waste.
There has been some improvements in handling waste. All waste PPE and contaminated cloths are being compacted to reduce volume. There has also being training sessions given to employees to ensure re-cycling bins are being used properly. 

Water consumption has been increasing and we have been making efforts to reduce it.

We have changed all water flow restrictor valves, and put in place a regular checking procedure to ensure they a working correctly. Also we are ensuring that the rinses are switched off between shifts. This has seen a reduction in water usage. 

At the start of May Welsh Water installed a new modern water meter. And we have purchased a data logging system so that we can monitor water on a 0.25 hour basis to see if the usage is as we expect it to be and any unexpected usage can be investigated.
On the effluent treatment side the new water filter on the Cyanide pit has been installed to try and remove as much cadmium and copper as possible. Also a new flocculent dosing system has been installed to improve metal removal at the end of the effluent treatment plant. The pH and Redox probes have been placed on a service contract to ensure they are working correctly.  So we are now monitoring result to see any improvements.

Chris Rochelle

Technical Manager

May 2015

[image: image1.jpg]



New flocculent Dosing system
[image: image2.jpg]



Filter on Cyanide effluent pit

[image: image3.jpg]



Waste compacter
