Appendix 10						Dust Management Plan

Introduction

This Plan has been prepared as there is one sensitive receptor (neighbouring dwelling house) (Ystum Colwyn Farmhouse) within 100 metres of the installation (Ystum Colwyn Farm).  The following table sets out:

- The likely sources of dust arising from a typical broiler chicken unit

- The procedures followed at The Ystum Colwyn Farm in order to prevent or minimise dust levels

Typical dust Sources and Actions Taken to Minimise Dust

| Dust Related Issue | Potential Risks and Problems | Actions taken to minimise dust and dust risks at Ystum Colwyn Farm | Completion date |
| --- | --- | --- | --- |
| Manufacture and selection of feed | •Milling and mixing of compound feeds •The use of poor quality ingredients •Feeds which are ‘unbalanced’ in nutrients, leading to increased excretion and litter moisture which then to be dried using extra ventilation and dust production | •No on-site milling. Mixing will be done within a sealed steel building. •Feed specifications are prepared by the feed compounder’s nutrition specialist •Feed is supplied only from UKASTA accredited feed mills, so that only approved raw materials are used | In place |
| Feed delivery and storage | •Spillage of feed during delivery and storage •Creation of dust during feed delivery | •Feed delivery systems are sealed to minimise atmospheric dust •The condition of feed bins is checked frequently so that any damage or leaks can be identified • | In place |
| Ventilation system | •Inadequate air movement in the house, leading to high humidity, wet litter followed by excessive ventilation •Inadequate system design, | •The ventilation system is regularly adjusted according to the age and requirements of the flock •The ventilation system is designed to efficiently remove moisture from the house. The ventilation is low velocity in the side of the sheds and not directed at any receptors. | In place |

| Dust Related Issue | Potential Risks and Problems | Actions taken to minimise dust and dust risks at Ystum Colwyn farm | Completion date |
| --- | --- | --- | --- |
| Litter management | The use of insufficient or poor quality litter •Spillage of water from drinking systems •Disease outbreaks, leading to wet litter | •Controls on feed and ventilation (see above) help to maintain litter quality. Additional controls include:-• Use of nipple drinking systems with cups which minimise spillage •Insulated walls and ceilings to prevent condensation •Concrete floors to prevent water ingress •Stocking density at optimal levels to prevent overcrowding • Addition of extra bedding material. Use of a health plan, with specialist veterinary input used as necessary | In place |
| House Clean Out | •Creation of dust associated with litter removal from houses • | •Litter is carefully placed into trailers positioned at the entrance to each house. When full, the trailer is covered • Doors kept closed whenever possible | In place |
| Used litter | •Storage of used litter on site •Transport of litter and applications to land | •There is no storage of used litter outside the houses at any time •Litter is transported in covered trailers •All of the litter is spread to land which is under the control of a separate farming business. A written agreement is in place | In place |