Notice of variation with introductory note

Environmental Permitting (England & Wales) Regulations 2016

Convatec Limited

Convatec Rhymney Gelling Fibre Plant

Heads of the Valley Industrial Estate

Rhymney

Tredegar

NP22 5RL

Variation number

EPR/NP3433BC/V005

Permit number

EPR/NP3433BC

Convatec Rhymney Gelling Fibre Plant

Permit number EPR/NP3433BC

Introductory note

This introductory note does not form a part of the notice

The following notice gives notice of the variation of an environmental permit.

This variation updates permit EPR/NP3433BC relating to the Hydrocel Plant located within Unit 1 of Convatec Limited’s site in Rhymney. The variation updates the permit to replace the current bioscrubber on site with a Regenerative Thermal Oxidiser (RTO), and to increase the permit boundary to accommodate this new abatement equipment. Although the production capacity will be doubled, this is already permitted within the existing permit.

The schedules specify the changes made to the original permit.

The status log of a permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Application for permit NP3433BC received | 06/10/2004 |  |
| Additional information received | 26/01/2005 |  |
| Application determined, permit NP3433BC granted | 24/03/2005 |  |
| Variation application PP3433LP (NP3433BC/V002) received | 07/09/2005 |  |
| Application PP3433LP (NP3433BC/V002) determined, variation notice issued | 01/12/2005 |  |
| Variation application EPR/NP3433BC/V003 received | 24/06/2008 |  |
| Application EPR/NP3433BC/V003 determined, variation notice issued | 04/03/2009 |  |
| Variation application EPR/NP3433BC/V004 received | 01/03/2012 | Variation application to add a new boiler |
| Application EPR/NP3433BC/V004 determined, variation notice issued | 30/03/2012 |  |
| Variation application EPR/NP3433BC/V005 received | Duly made 16/03/2018 | Variation application to replace bioscrubber with RTO |
| Request for further information (Schedule 5) issued | 15/05/2018 | Request for site plan and for information on noise and odour |
| Additional information received | 31/05/2018 | Statutory deadline extended by 16 days |
| Application EPR/NP3433BC/V005 determined, variation notice issued | DD/MM/YY |  |

| Other Part A installation permits relating to this installation |  |  |
| --- | --- | --- |
| Operator | Permit number | Date of issue |
| Convatec Limited | TP3535LH | 17/11/2006 |

End of introductory note

Notice of variation

Environmental Permitting (England and Wales) Regulations 2016

The Natural Resources Body for Wales (“Natural Resources Wales”) in exercise of its powers under regulation 20 of the Environmental Permitting (England and Wales) Regulations 2016 varies

Permit number

EPR/NP3433BC

issued to:

Convatec Limited (“the operator”)

whose registered office is

Gdc First Avenue

Deeside Industrial Park

Deeside

Flintshire

CH5 2NU

company registration number 01309639

to operate a regulated facility at

Convatec Rhymney Gelling Fibre Plant

Heads of the Valley Industrial Estate

Rhymney

Tredegar

NP22 5RL

to the extent set out in the schedules.

The notice shall take effect from [DD/MM/YYYY]

| Signed | Date |
| --- | --- |
| [name of authorised person] Type name, signature not needed | [DD/MM/YYYY] |

Authorised on behalf of Natural Resources Wales

Schedule 1 – conditions to be deleted

The following conditions are deleted as a result of the application made by the operator:

2.2.7.2 No emission from the Permitted installation shall be made to land.

Schedule 2 – conditions to be amended

The following conditions and tables are amended as a result of the application made by the operator:

| Table 1.1.1 |  |  |
| --- | --- | --- |
| Activity listed in Schedule 1 of the EPR Regulations/Associated Activity | Description of specified activity | Limits of specified activity |  |
| Section 4.5 A(1)(a) – Producing pharmaceutical products | Manufacture of gelling fibres | Receipt of raw materials to despatch of finished product. |  |
| Section 7 Part B – SED Activities. Manufacturing of pharmaceutical products. | Manufacture of pharmaceutical products | Receipt of raw materials to despatch of finished product. |  |
| Directly Associated Activity | Storage and disposal of waste | From generation of waste to dispatch off site. |  |
| Directly Associated Activity | Control and abatement systems for emissions to air | Treatment of waste gases via a regenerative thermal oxidiser (RTO). |  |

| Table 1.4.1: Improvement programme |  |  |
| --- | --- | --- |
| Reference | Requirement | Date |  |
| IP1 | The Operator shall establish a mass balance calculation to determine the total VOC emission from the Installation, as a percentage of the total solvent input, and submit a copy to the Agency for approval. | Complete |  |
| IP2 | The Operator shall provide a summary report, detailing plant commissioning operational performance. Details to be agreed in writing with the Agency. | 31st March 2006 |  |
| IP3 | The Operator shall implement a formal procedure for the inspection and subsequent maintenance of tanks, impervious surfaces, containment kerbs and underground sumps with the purpose of preventing fugitive releases to ground. A copy of the procedure shall be submitted to the Agency. | 1st December 2005 |  |
| IP4 | The Operator shall implement a system to enable specific energy consumption monitoring of the installation. | 31st March 2006 |  |
| IP5 | The Operator shall undertake an assessment of the impact of noise arising from the Installation on the amenity of the locality. This assessment shall be undertaken to a specification agreed with the Environment Agency. A report shall be sent to the Agency detailing proposals for the implementation of any further improvement options representing BAT. | 31st December 2005 |  |
| IP6 | The Operator shall establish a Site Closure Plan having regard for the Agency Sector Guidance Note IPPC S4.02 and submit a copy to the Agency for approval. | Complete |  |
| IP7 | The Operator shall obtain ISO 14001 certification for the installation as detailed in the application. | 31st January 2006 (unless otherwise agreed in writing with the Agency) |  |
| IP8 | The Operator shall review the provision of MCERTS accreditation for the monitoring equipment, personnel and organisations employed for the emissions monitoring programme in condition 2.10.1 and propose a timetable for achieving this standard for any elements which are not MCERTS certified. | 31st December 2005 |  |
| IP9 | The Operator shall conduct an assessment of all monitoring methods and procedures, both in-house and those employed by external contractors, for conformance with appropriate monitoring Standards such as CEN, BSI, ISO, etc., having regard for the Agency’s Sector Guidance IPPC S4.02 and Monitoring Guidance M2 and the Sampling and Analysis Standards presented. A report on the findings and proposed actions will be submitted to the Agency. | 31st December 2005 |  |
| IP10 | The Operator shall use measured VOC emission data to review the environmental impact of the installation using the Environment Agency’s H1 assessment tool, or other agreed system. | 31st December 2006 |  |
| IP11 | The Operator shall monitor releases to A2. This monitoring shall be agreed with the Agency prior to work commencing. A report shall be submitted to the Agency and agreed in writing and shall include monitoring results, a H1 assessment, further modelling if required, measures if required to minimise releases and a plan giving timescales for implementation of these. | 31st March 2006 |  |
| IP12 | Following successful commissioning and establishment of routine steady operation of the RTO, the Operator shall undertake noise monitoring at the nearest local receptors. This shall include: - A full noise monitoring survey and assessment meeting the BS4142:2014 standard - 1/3rd octave and narrow band (FFT) measurements to identify any tonal elements or low frequency noise - Reference to the World Health Organisation guidelines for community noise Upon completion of the work, a written report shall be submitted to Natural Resources Wales. If rating levels likely to cause adverse impact at sensitive receptors are detected, the report shall include an assessment of the most suitable abatement techniques, an estimate of the cost and a proposed timetable for their installation. | Within 6 months of the completion of commissioning of the RTO |  |
| IP13 | Emissions from the new RTO shall be monitored and reported on a quarterly basis rather than on an annual basis for the 1st year of monitoring. |  |  |

| Table 2.2.1: Emission points to air |  |  |
| --- | --- | --- |
| Emission point ref. or description | Source | Location of emission point |  |
| A1 | Bioscrubber stack | Point A1 on site layout plan (Schedule 4) |  |
| A1A | RTO stack | Point A1A on site layout plan (Schedule 4) |  |
| A2 | Reactor room extract | Point A2 on site layout plan (Schedule 4) |  |
| A3 | Dust extraction | Point A3 on site layout plan (Schedule 4) |  |
| A4 | Steam boiler | Point A4 on site layout plan (Schedule 4) |  |
| A5 | Medicel room extract EP34 | Point A5 on site layout plan (Schedule 4) |  |
| A6 | Medicel LEV extract EP34 | Point A6 on site layout plan (Schedule 4) |  |
| A7 | Silver hydrocel room extract EP36 | Point A7 on site layout plan (Schedule 4) |  |
| A8 | Silver hydrocel LEV extract EP37 | Point A8 on site layout plan (Schedule 4) |  |
| A9 | Hydrocel room extract EP38 | Point A9 on site layout plan (Schedule 4) |  |
| A10 | Hydrocel LEV extract EP39 | Point A10 on site layout plan (Schedule 4) |  |
| A11 | Fallow suite room extract EPFS1 | Point A11 on site layout plan (Schedule 4) |  |
| A12 | Fallow suite LEV extract EPFS2 | Point A12 on site layout plan (Schedule 4) |  |
| A13 | Gas fired boiler | Point A13 on site layout plan (Schedule 4) |  |

| Table 2.2.2: Emission limits to air and monitoring |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. or description | Parameter | Limit (including Reference Period) 1 | Monitoring frequency |  |  | Monitoring method 2 |  |
| A1 | VOC 3 | No limit 4 | Continuous and annual extractive |  |  | BS EN 12619 |  |
| A1A | Oxides of Nitrogen (NO and NO2 expressed as NO2) | 50 mg/m3 | Annual |  |  | BS EN 14792 |  |
|  | Carbon Monoxide | 100 mg/m3 | Annual |  |  | BS EN 15058 |  |
|  | VOC | 20 mg/m3 | Annual |  |  | BS EN 12619 |  |
| A3 | Dust | No visible release | N/A |  |  | N/A |  |

Note 1: See section 6 for reference conditions

Note 2: Or as agreed in writing with Natural Resources Wales

Note 3: Expressed as Total Carbon

Note 4: Limit to be reviewed twelve months after commencement of operations

## 2.2.2	Emissions to water or land

### 2.2.2.1	This Part 2.2.2 of this Permit shall not apply to releases of odour, noise or vibration or to releases to groundwater

### 2.2.2.2	Conditions 2.2.2.3 – 2.2.2.6 shall not apply to emissions to sewer

### 2.2.2.3	There shall be no point source emissions to water or land except from the sources and emission points listed in table 2.2.3. The limits given in table 2.2.3 shall not be exceeded.

### 2.2.2.4		No condition applies

### 2.2.2.5		No condition applies

### 2.2.2.6 	No condition applies

| Table S2: Reporting of monitoring data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Emission point | Reporting period | Period begins |  |  |
| VOC mg/m3 (continuous) | A1 | Every 3 months | 1st April 2005 |  |  |
| VOC mg/m3 (extractive) | A1, A1A | Annually | 1st April 2005 |  |  |
| VOC % of total input | All emissions | Every 3 months | 1st April 2005 |  |  |
| Oxides of Nitrogen | A1A | Annually | DD/MM/YY |  |  |
| Carbon Monoxide | A1A | Annually | DD/MM/YY |  |  |
| Chemical oxygen demand mg/l | S1 | Every 3 months | 1st April 2005 |  |  |
| Suspended solids mg/l | S1 | Every 3 months | 1st April 2005 |  |  |
| pH | S1 | Every 3 months | 1st April 2005 |  |  |
| Water usage | N/A | Every 12 months | 1st April 2005 |  |  |
| Energy usage | N/A | Every 12 months | 1st April 2005 |  |  |
| Waste disposal and/or recovery | N/A | Every 12 months | 1st April 2005 |  |  |

Schedule 3 – conditions to be added

The following conditions as a result of the application made by the operator:

| Table 2.2.3: Point source emissions to water (other than sewer) and land |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. or description | Parameter | Limit (including Reference Period) | Monitoring frequency |  |  | Monitoring method |  |
| W1 | N/A | N/A | N/A |  |  | N/A |  |

Schedule 4 – amended plan

Amended plan attached: