# Natural Resources Wales permitting decisions

Decision Document – Brynthomas Farm Poultry Unit

# Bespoke permit application

## The application number is:	PAN-002113

## The Applicant / Operator is:	Mr Mark Owen and Mrs Nicola owen

## The Installation is located at:	Brynthomas Farm Poultry Unit, Brynthomas Farm, Pennybont, Llandrindod Wells, Powys, LD1 5SP

We have decided to issue the permit variation for Brynthomas Farm Poultry Unit operated by Mr Mark and Mrs Nicola Owen.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

## Purpose of this document

This decision document:

explains how the application has been determined

provides a record of the decision-making process

shows how all relevant factors have been taken into account

justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

## Structure of this document

- Table of contents

- Key issues

- the consultation and web publicising responses

## Table of Contents

## Contents

Natural Resources Wales permitting decisions	1

Bespoke permit application	2

The application number is:	PAN-002113	2

The Applicant / Operator is:	Mr Mark Owen and Mrs Nicola owen	2

The Installation is located at:	Brynthomas Farm Poultry Unit, Brynthomas Farm, Pennybont, Llandrindod Wells, Powys, LD1 5SP	2

Purpose of this document	2

Structure of this document	3

Table of Contents	4

Contents	4

Key issues of the decision	6

Receipt of application	6

Consultation	6

Operator	7

The facility	7

European Directives	7

The site	7

Site condition report	8

Biodiversity, Heritage, Landscape and Nature Conservation	8

Environmental Risk Assessment	8

Air	8

Water	9

Odour	9

Noise	9

Fugitive emissions	10

Operating techniques	10

Incorporating the application	11

Environment management system	11

Consultation Reponses	12

A)	Advertising and Consultation on the Application	12

1)	Consultation Responses from Statutory and Non-Statutory Bodies	12

2)	Consultation Responses from Members of the Public and Community Organisations	13

a)	Representations from Local MP, Assembly Member (AM), Councillors and Parish / Town / Community Councils	13

b)	Representations from Community and Other Organisations	13

c)	Representations from Individual Members of the Public	14

# Key issues of the decision

# Receipt of application

The Application was received on 21 November 2017 and was duly made as of 5 January 2018, following the receipt of additional information. This means we considered it was in the correct form and contained sufficient information for us to begin our determination, but not that it necessarily contained all the information we would need to complete that determination.

### Consultation

The consultation requirements were identified and implemented.  The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements.

A copy of the Application and all other documents relevant to our determination (see below)  are available for the public to view. Anyone wishing to see these documents could arrange for copies to be made.

We sent copies of the Application to the following bodies, which includes those with whom we have “Working Together Agreements”:

Public Health Wales

Powys County Council – Environmental Health

Powys County Council – Planners

These are bodies whose expertise, democratic accountability and/or local knowledge make it appropriate for us to seek their views directly.

The consultation started on 18/01/2018 and ended on 15/02/2018.

An advert was also placed on our website, this started on 18/01/2018 and ended on 15/02/2018.

On completion of the draft determination, we advertised this draft determination on our website from 3 May 2018 Until 4 June 2018. No responses were received.

Further details along with a summary of consultation comments and our response to the representations we received can be found in Annex 3.  We have taken all relevant representations into consideration in reaching our determination.

### Operator

The application is appropriately signed and our records show no reason why the applicant would be unsuitable as an operator.

### The facility

The regulated facility is an installation which comprises the following activities listed in Part 2 of Schedule 1 to the Environmental Permitting Regulations and the directly associated activities in the table below.

Intensive rearing of poultry with a maximum of 200,000 broiler birds;

- Section 6.9 A(1)(a)(i) Rearing of poultry or pigs intensively in an installation with more than 40,000 places for poultry

Supported by the following directly associated activities;

Poultry Manure Burner

Dirty water tank

Fuel storage

## European Directives

All applicable European directives have been considered in the determination of the application.

## The site

There will be four poultry houses and these will be designed and managed in accordance with Best Available Technique (BAT) standards. The working area where vehicles operate will be laid with concrete and hardstanding.  The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility. In addition the operator has provided a site layout and site drainage plan which includes discharge points.

A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary.

### Site condition report

The operator has provided a description of the condition of the site.

We consider this description is satisfactory.  The decision was taken in accordance with our guidance on site condition reports – guidance and templates (H5).

### Biodiversity, Heritage, Landscape and Nature Conservation

The site is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat .

A full assessment of the application and its potential to affect the sites has been carried out as part of the permitting process.

Specifically we looked at the potential impacts on:

The River Wye SAC and on 12 SSSI’s.  Further details are available on the Form 1 and Appendix 4 documents which were prepared.  However a key factor in the decision is the fact that this proposed installation is replacing an existing operation and will result in a net reduction in emissions and a net reduction in impacts on these identified habitats.

## Environmental Risk Assessment

### Air

The Applicant has assessed the Installation’s potential emissions to air against the relevant air quality standards, and the potential impact upon human health.  These assessments predict the potential effects on local air quality from the Installation’s emissions.

We are in agreement with this approach.

### Water

Based upon the information in the application we are satisfied that the appropriate measures will be in place to prevent pollution of ground and surface water. The only water discharge proposed is for the drainage of clean, uncontaminated rainwater from roofs and yard areas to discharge to surface water and attenuation pond.

### Odour

The applicant submitted an odour modelling report.  This report was reviewed by NRW and we concluded that the predicted results were reliable.  These results showed no significant impacts at properties not owned by the operator.

We are satisfied that the risk from the permitted facility of odour pollution at nearby receptors is not significant.

### Noise

The applicant has described a detailed series of measures that will be put into place to minimise noise emissions in the noise management plan. We have compared the measures proposed for the site to the BAT standards in EPR 6.09 and are satisfied that the techniques represent appropriate measures for the installation. The techniques described in the documents submitted in support of the application have been incorporated into table S1.2 of the permit as operating techniques.  Permit condition 2.3.1 requires the operator to operate the installation in accordance with the techniques listed in Table S1.2 of the permit.

We are satisfied that vibration is unlikely to be an issue at the installation.  The nature of the intensive farming operation means that there are no significant sources of vibration on site. Therefore vibration does not need to be included in the noise management plan.

Movement of vehicles outside the installation boundary is outside the regulatory scope of the Environmental Permitting (England and Wales) Regulations 2016 and is a matter for the local planning authority.

Permit condition 3.4.1 requires that emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site. This will be checked during NRW site inspections and if this is not the case, we will take appropriate action.

Noise is not generally a source of complaints for the intensive farming sector in Wales.  This conclusion is supported by information on noise complaints from NRW’s own databases.

In summary, we are satisfied that pollution due to noise will be managed to acceptable levels.  We consider the permit conditions and operating techniques to be sufficiently protective and are satisfied that the operational measures taken to minimise noise are compliant satisfactory.

### Fugitive emissions

Based upon the information in the application we are satisfied that the appropriate measures will be in place to prevent or where that is not practicable to minimise fugitive emissions and to prevent pollution from fugitive emissions.

## Operating techniques

We have reviewed the techniques used by the operator and compared these with the relevant guidance notes.

The proposed techniques/ emission levels for priorities for control are in line with the benchmark levels contained in the Best Available Techniques fot the industry and we consider them to represent appropriate techniques for the facility.

### Incorporating the application

We have specified that the applicant must operate the permit in accordance with descriptions in the application, including all additional information received as part of the determination process.

These descriptions are specified in the Operating Techniques table in the permit.

### Environment management system

There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions.  The decision was taken in accordance with RGN 5 on Operator Competence.

# Consultation Reponses

### A)	Advertising and Consultation on the Application

The Application has been advertised and consulted upon in accordance with Natural Resources Wales Public Participation Statement.

### 1)	Consultation Responses from Statutory and Non-Statutory Bodies

| Response Received from: Powys Teaching Health Board |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| Overall conclusion included “Based upon the information submitted by the applicant and provided that Best Available Techniques (BAT) and management controls are applied as per relevant sector guidance, then we see no public health reasons to object to the proposed activities associated with this permit. We would recommend that the regulator consider a cumulative impact assessment should there be any other existing or proposed poultry units in the locale. A site ammonia and odour management plan is required to ensure that local peopleare not exposed to elevated levels of ammonia or odours that may impact on health. We note that modelling of odours and ammonia have been undertaken and shown to be below concentrations likely to cause nuisance orhealth affects at the nearest receptors. The applicant should be required to produce and maintain a manure management plan. Dusts and bio-aerosols may have possible health effects. Appropriate assessment and operational management of such emissions in accordance with BAT is recommended. All on-site storage of liquids should be accompanied by bunding. The regulator must be sure that the noise from the proposed activities does not cause nuisance at nearby sensitive receptors. Recommendation that a timetable be agreed for the applicant to seek external accreditation for the Environmental Management System eg ISO4001 | The applicant has proposed BAT and has stated that they have the approopriatemanagement controls. There are no other units of any significance in the locale As stated the modelling has demonstrated that there are no predicted adverse impacts. Odour management plan in place along with odour condition and ammonia reporting requirements within the permit. This is outside of the installation boundary and is beyond the scope of what can be regulated by the permit. The site is operated in accordance with BAT and no sensitive receptors are located within close proximity to the site. Fuel tank for back up generator is to be bunded. A noise management plan is in place and the permit includes relevant noise condition. While NRW would welcome such an accredited management system, we have reviewed the proposed management system and while not externally accredited, it is fit for this installation and is in keeping with the industry standards. |

### 2)	Consultation Responses from Members of the Public and Community Organisations

### a)	Representations from Local MP, Assembly Member (AM), Councillors and Parish / Town / Community Councils

| Response Received from |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| None | N/A |

### b)	Representations from Community and Other Organisations

| Response Received from |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| None | N/A |

### c)	Representations from Individual Members of the Public

| Response Received from |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| None | N/A |