Domgay Hall Broiler Unit

Decision Document

# New bespoke permit

## The application number is: PAN-001438

## The Applicant / Operator is: Mr Martin Evans

## The Installation is located at: Domgay Hall Broiler Unit, Domgay Hall, Four Crosses, Powys, SY22 6SN

We have decided to grant the permit for Domgay Hall Broiler Unit operated by Mr Martin Evans.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

## Purpose of this document

This decision document:

explains how the application has been determined

provides a record of the decision-making process

shows how all relevant factors have been taken into account

justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

## Structure of this document

- Table of contents

- Key issues

- Annex 1 the consultation and web publicising responses

## Table of Contents

## Contents

New bespoke permit	2

The application number is: PAN-001438	2

The Applicant / Operator is: Mr Martin Evans	2

The Installation is located at: Domgay Hall Broiler Unit, Domgay Hall, Four Crosses, Powys, SY22 6SN	2

Purpose of this document	2

Structure of this document	2

Table of Contents	3

Contents	3

Key issues of the decision	5

1	Our decision	5

2	How we reached our decision	6

2.1 Receipt of Application	6

2.2	 Consultation on the Application	6

2.3	 Requests for Further Information	8

3	The Legal Framework	8

4	The Installation	9

4.1 Description of the Installation and related issues	9

4.2 The site and its protection	10

4.3 Operation of the Installation – general issues	11

5	Minimising the Installation’s environmental impact	14

5.1	Assessment of Impact on Air Quality	15

5.2	Assessment of odour impact	15

5.3	Assessment of impact to surface and ground water	16

5.4	Emissions to sewer	17

5.5	Fugitive emissions	17

5.6	 Noise Assessment	17

5.7	Impact on Habitats sites, SSSIs, non-statutory conservation sites etc	18

6  Setting ELVs and other Permit conditions	21

6.1	Translating BAT into Permit conditions	21

6.2	Monitoring	21

6.3	Reporting	21

6.4 Pre-operational conditions	21

6.5 Improvement conditions	22

ANNEX 1:	Consultation Reponses	23

A)	Advertising and Consultation on the Application	23

1)	Consultation Responses from Statutory and Non-Statutory Bodies	23

2)	Consultation Responses from Members of the Public and Community Organisations	23

a)	Representations from Local MP, Assembly Member (AM), Councillors and Parish / Town / Community Councils	23

b)	Representations from Community and Other Organisations	24

c)	Representations from Individual Members of the Public	24

# Key issues of the decision

## Our decision

Based on the information currently available to us we are currently minded to issue a permit to the Applicant.  This would, if issued, allow it to operate the Installation, subject to the conditions in the Permit.

We consider that, in reaching that decision, we have taken into account all relevant considerations and legal requirements and that the permit will ensure that a high level of protection is provided for the environment and human health.

This Application is to operate an installation which is subject principally to the Industrial Emissions Directive (IED).

The permit contains many conditions taken from our standard Environmental Permit template including the relevant Annexes. We developed these conditions in consultation with industry, having regard to the legal requirements of the Environmental Permitting Regulations and other relevant legislation. This document does not therefore include an explanation for these standard conditions. Where they are included in the permit, we have considered the Application and accepted the details are sufficient and satisfactory to make the standard conditions appropriate.

## How we reached our decision

### 2.1 Receipt of Application

The Application was accepted as duly made on 24/05/17.  This means we considered it was in the correct form and contained sufficient information for us to begin our determination, but not that it necessarily contained all the information we would need to complete that determination.

The Applicant made a claim for no claim for commercial confidentiality. We have not received information in relation to the Application that appears to be confidential in relation to any party.

### 2.2	 Consultation on the Application

We carried out consultation on the Application in accordance with the EPR, our statutory Public Participation Statement (PPS) and our Regulatory Guidance.

We advertised the Application by a notice placed on our website, which contained all the information required by the IED, including telling people where and when they could see a copy of the Application.

A copy of the Application and all other documents relevant to our determination (see below)  are available for the public to view. Anyone wishing to see these documents could arrange for copies to be made.

We sent copies of the Application to the following bodies, which includes those with whom we have “Working Together Agreements”:

- Powys County Council Planning Authority

- Powys County Council Environmental Protection Department

- Food Standards Agency

- Health Protection Agency

- Public Health Wales (PHW)

These are bodies whose expertise, democratic accountability and/or local knowledge make it appropriate for us to seek their views directly.

Further details along with a summary of consultation comments and our response to the representations we received can be found in Annex 1.  We have taken all relevant representations into consideration in reaching our determination.

We are now carrying out a consultation on our draft  decision. This consultation will begin on 17/11/17 and  end on 15/12/17.

### 2.3	 Requests for Further Information

In order for us to be able to consider the application duly made, we needed more information. We requested further information relating to the completion of Part B2 Section 3a of the application, relating to relevant offences. We also requested the correct screening distances for the ammonia assessment to be used. Upon receipt of this information we were able to consider the application Duly Made.

Further information was also requested by way of a Schedule 5 Notice requiring the applicant to submit an odour modelling report, clarifying the occupants of the nearby dwellings, confirm the fan types and point source emissions to air and complete an appropriate assessment of the proposed biomass boiler. The Schedule 5 Notice was sent on 2 August 2017 with a response date of 30 August 2017. The Applicants response to the Schedule 5 Notice was provided on 28 September 2017. The additional information supplied satisfied the requirements of the Schedule 5 notice issued on 2 August 2017.

A copy of the information notice and e-mails requesting further information were placed on our public register as were the responses when received.

## The Legal Framework

The Permit will be granted, under Regulation 13 of the EPR.  The Environmental Permitting regime is a legal vehicle which delivers most of the relevant legal requirements for activities falling within its scope.  In particular, the regulated facility is:

an installation as described by the IED;

subject to aspects of the Well-Being of Future Generations (Wales) Act 2015 and the Environment (Wales) Act 2016 which also have to be addressed.

We address the legal requirements directly where relevant in the body of this document.  NRW is satisfied that this decision is consistent with its general purpose of pursuing the sustainable management of natural resources in relation to Wales, and applying the principles of sustainable management of natural resources. In particular, NRW acknowledges that it is a principle of sustainable management to take action to prevent significant damage to ecosystems. We consider that, in granting the Permit a high level of protection will be delivered for the environment and human health through the operation of the Installation in accordance with the permit conditions.

## 4	The Installation

### 4.1 Description of the Installation and related issues

#### 4.1.1	The permitted activities

The Installation is subject to the EPR because it carries out an activity listed in Schedule 1 Part 2 of the EPR:

- Section 6.9 Part A(1)(a)(i) Rearing poultry in an installation with more than 40,000 places.

An installation may also comprise “directly associated activities”, which at this Installation includes.

- Feed silos

- Dirty water tank

- Fuel and Chemical storage

- Biomass Boiler

Together, these listed and directly associated activities comprise the Installation.

#### 4.1.2	The Site

Domgay Hall Poultry Unit is located to the north east of the village of Four Crosses in the County of Powys. The site is located on an intensive arable field, adjacent to Domgay Hall. The site sits in a shallow valley at an altitude of approximately 65m. There are nearby residences within 400 metres of the poultry houses. The National Grid Reference is 327888, 319346.

The footprint of the proposed intensive development will be totalling approximately 0.51 hectares (ha) within a site of 1.187ha. The buildings will 5.0m high and the associated feed bins 8m high. Hours of operation will be 24 hours a day 7 days a week due to the fact that it is a livestock enterprise requiring continual husbandry. Short seven day periods between crop cycles will occur when there will be no birds on the site.

The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility. In addition the operator has provided a site layout/drainage plan which includes discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary.

#### 4.1.3	What the Installation does

The farm business trading as E M Evans has been established within the last thirty years and is made up of Mr M Evans. The farm business has made the decision to diversify into a poultry enterprise to support their successful dairy and arable enterprises.

Domgay Hall Broiler Unit proposes to operate as a broiler meat production unit with capacity for 100,000 broiler chickens per crop cycle and an annual meat production capacity of 1,578 tonnes and an annual manure production capacity of around 1,800 tonnes.

The proposed poultry houses will each be ventilated by 6 high velocity roof fans (12 high velocity roof fans in total) and 5 gable end fans (10 gable end fans in total).

### 4.2 The site and its protection

#### 4.2.1	Proposed site design: potentially polluting substances and prevention measures

There will be two fan ventilated poultry houses, this will give a bird stocking density of approximately 100,000 under the current legislative/welfare stocking density. The working area where vehicles operate is laid to concrete and hard standing. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins. Manure is removed from houses at the end of the production cycle. Immediately following depopulation, litter is removed off site,  for  use on operator controlled land or sold to a local Anaerobic Digester. The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks will have been installed to catch all wash waters. Dead birds are removed from the houses and stored in sealed containers awaiting collection from a licensed renderer. Diesel fuel storage is in a bunded tank, Chemicals are stored in a frost free secure bunded store (off site).

The site will also be operated in compliance with “how to comply” routine maintenance schedules are followed and recorded and with any abnormal operations recorded.

#### 4.2.2	Closure and decommissioning

Permit condition 1.1.1 requires the Operator to have a written management system in place which identifies and minimises risks of pollution including those arising from closure.

At the definitive cessation of activities, the Operator has to satisfy us that the necessary measures have been taken so that the site ceases to pose a risk to soil or groundwater, taking into account both the baseline conditions and the site’s current or approved future use.  To do this, the Operator has to apply to us for surrender, which we will not grant unless and until we are satisfied that these requirements have been met.

The applicant has completed a Site Condition Report when applying for the permit. The land had previously been used predominantly for arable farming and general agricultural use. There have been no previous pollution incidents. We consider that the description provided is satisfactory. The decision was taken in accordance with our guidance on site condition reports – guidance and templates (H5).

### 4.3 Operation of the Installation – general issues

#### 4.3.1	Administrative issues

The Applicant is the sole Operator of the Installation. We are satisfied that the Applicant is the person who will have control over the operation of the Installation if the Permit were to be granted; and that the Applicant will be able to operate the Installation so as to comply with the conditions included in the Permit, if issued.

#### 4.3.2	Relevant convictions

Our Enforcement Database has been checked to ensure that all relevant convictions have been declared. No relevant convictions were found.

#### 4.3.3	Management

The Applicant has stated in the Application that they will implement an Environmental Management System (EMS) that will meet the requirements for an EMS in our “How to comply with your environmental permit guidance”. The Applicant submitted a summary of the EMS with their application.

All written management systems will be subject to regular review by the Operator.

We are satisfied that appropriate management systems and management structures will be in place for this Installation, and that sufficient resources are available to the Operator to ensure compliance with all the Permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence.

#### 4.3.4	Accident management

In order to ensure that the management system proposed by the Applicant sufficiently manages the residual risk of accidents, permit condition 1.1.1(a) requires the implementation of a written management system which addresses the pollution risks associated with, amongst other things, accidents.

The Operator has an emergency plan which will be subject to regular review. The plan includes contingencies for events such as fire, power failure, flood, disease and containment failure.

#### 4.3.5	Site security

Having considered the information submitted in the Application, we are satisfied that appropriate infrastructure and procedures will be in place prior to start up to ensure that the site remains secure.

#### 4.3.6	Off-site conditions

Based on the information submitted in the application, we do not consider that it is necessary to impose offsite conditions.

#### 4.3.7	Operating techniques

The chickens will be grown in 7.6, 42-day crop cycles per annum with 7-day turn around periods per crop. Feed is delivered from a UKASTA accredited feed mill and blown into bulk feed bins situated at the ends of the houses. From the feed bins, the feed is augered into the houses and distributed to the birds via a pan feeding system. The house is ventilated by high speed ridge mounted fans with gable end fans providing additional ventilation in hot weather conditions.

Fallen stock will be recorded daily and securely stored in vermin proof containers awaiting regular collection by a licenced renderer.

#### 4.3.8	Energy efficiency

We are satisfied that the Applicant will ensure that energy is used in the most efficient way possible.

#### 4.3.9	Avoidance, recovery or disposal of wastes produced by the activities

At depletion any remaining litter will be removed from the site and used on operator controlled land or sold as fuel for Anaerobic Digestion. At the end of the cycle the site will be pressured washed, disinfected and dried out prior to the cycle beginning again. All wash waters will be contained in sealed underground tanks. Wash water along with sediment traps will be emptied and removed with litter, reducing emissions. Records of tonnage of litter and wash water removal are recorded.

Inorganic waste generated by the farm will mainly consist of paper, plastic and glass. Plastic waste will normally be in two forms, wrapping from bales of wood shavings and bottles from used disinfectants and detergents. The amount of plastic waste will be minimised through good managerial techniques. By good management of the litter quality, fewer bales of wood shaving will be needed, thus lowering the amount of plastic wrapping discarded. Large empty plastic bottles from detergents will be ‘recycled’ and used for food dip containers or smaller rubbish bins for the storerooms.

Poultry carcasses are, under normal circumstances, collected and stored in sealed containers awaiting regular collection under the fallen stock scheme by a licensed collection agent. As a contingency plan or if an outbreak of high mortality should arise, carcasses will be placed in sealed containers and removed, as detailed in the emergency plan.

In the event of high mortality caused by disease, the operator will follow the guidance of the allocated veterinarian dealing with the outbreak. The mortality would be disposed of at an approved landfill site under the advice of that veterinarian, after consideration of weather conditions and geographical haulage parameters.

We are satisfied that waste from the Installation that cannot be recovered will be disposed of offsite using a method that minimises any impact on the environment.  Permit condition 1.4.1  will ensure that this position is maintained.

## 5	Minimising the Installation’s environmental impact

Regulated activities such as Intensive Farming can present different types of risk to the environment, these include odour, noise and vibration; accidents, fugitive emissions to air and water; as well as point source releases to air, discharges to ground or groundwater and generation of waste. All these factors are discussed in this and other sections of this document.

For an installation of this kind, the principal emissions are :

- Ammonia

- Dust

- Odour

- Noise

- Effluent discharges

The following sections of this document explain how we have considered the critical issue of assessing the likely impact of emissions from the Installation on human health and the environment and what measures we are requiring to ensure a high level of protection.

### 5.1	Assessment of Impact on Air Quality

The applicant has carried out a risk assessment identifying potential risks to human health including dust and ammonia. Operating procedures have been put in place to minimise the risks, in line with BAT procedures. It is considered that if the site is operated in line with these procedures, there is no significant risk to human health as a result of activities at the site.

### 5.2	Assessment of odour impact

The Applicant submitted detailed dispersion modelling of the impact of odour from the proposed facility.

H4 Odour Management guidance explains that the odour benchmarks are based on the 98th percentile of hourly average concentrations of odour modelled over a year at the site/installation boundary. The benchmarks are:

- 1.5 odour units for most offensive odours

- 3 odour units for moderately offensive odours

- 6 odour units for less offensive odours

Odours from poultry rearing are usually placed in the moderately offensive category. Therefore for their modelling the applicant has used the benchmark of 3.0 OUE/m3 to assess the potential impact of odour on nearby sensitive receptors. Modelling has been carried out based on emissions from two poultry houses.

The applicant has described the following measures which will be in place to minimise odour emissions during house depopulation/de-littering:

- Litter will be placed carefully into trailers positioned close to house doors;

- Trailers containing spent litter will be sheeted before leaving the fill position;

- Clean out will be carried out as soon as possible following destocking;

The applicant has submitted an odour management plan (OMP) for the installation as required by EPR 6.09 “How to Comply with your Permit for Intensive Farming” because there are sensitive receptors within 400 metres of the installation.  The OMP describes the measures and controls in place to minimise odour and includes daily olfactory checks.

We have compared the measures proposed for the site to the BAT standards in EPR 6.09 and are satisfied that the techniques represent appropriate measures for the installation. The OMP has been incorporated into the operating techniques section of the permit.

Permit condition 3.3.1 requires that emissions from the activities are free from odour at levels likely to cause pollution outside the site.  We are satisfied that this condition will be sufficiently protective in conjunction with the measures described by the applicant for minimising odour production at the installation.

We are satisfied that the risk from the permitted facility of odour pollution at nearby receptors is not significant. NRW has assessed the modelling in detail and is satisfied that it accurately represents the predicted odours.  It is recognised that this modelling does only represent the expected odour concentrations for 98% of the time and that odours may be higher for the remaining 2% of the time. NRW is not able to ensure that odour impacts on nearby receptors are reduced to zero, but is determined to ensure that they are minimised.

### 5.3	Assessment of impact to surface and ground water

Based upon the information in the application we are satisfied that the appropriate measures will be in place to prevent pollution of ground and surface water. Clean, uncontaminated rainwater from roofs and yard areas will drain to a balancing pond (volume of 166m3 and 1m depth). A hydro brake will be used to limit the outfall of the attenuation pond to 5 litres per second. Any lightly contaminated yard wash will be directed to a sealed underground tank. Concerns were raised by the local NRW area team regarding the potential run off of contaminated dirty water during the operational and clean out phases of the poultry houses. To mitigate the potential of any contaminated water from entering the nearby watercourse the applicant will install a curb around the hardstanding areas to prevent run off, also clean and dirty water drains will be kept separate as shown on the site drainage plan.

### 5.4	Emissions to sewer

There are no emissions to sewer. When birds are removed from the poultry houses, the site will be pressure washed, disinfected and dried out prior to the next cycle. All wash waters will be contained in a sealed underground tank.

### 5.5	Fugitive emissions

Based upon the information in the application we are satisfied that the appropriate measures will be in place to prevent or where that is not practicable to minimise fugitive emissions and to prevent pollution from fugitive emissions.

### 5.6	 Noise Assessment

A risk assessment of the potential impact of noise from the site on nearby sensitive receptors has been carried out by the applicant. Potential sources of noise include vehicles travelling to and from site, ventilation fans, feed transferring from lorries to bins, bird catching and clean out operations.

There are sensitive receptors within 400 metres of the installation. The applicant completed a noise assessment report, the report predicted that the resultant noise impact from the proposed poultry sheds would be low during the day and very low to negligible during the night. Modelling predicted that with 100% extract fans operating the noise emissions at the nearest dwelling would not exceed 30dB during the day and 25dB during the night time. To put these noise levels in context a whisper is around 30dB and rustling leaves around 20dB.

The applicant also submitted a noise management plan (NMP) for the installation as required by EPR 6.09 “How to Comply with your Permit for Intensive Farming”. The NMP describes the measures and controls in place to minimise noise and includes twice daily inspections of the site.

The noise management plan states that roof mounted ventilation fans will be subject to regular, end of cycle maintenance by qualified electricians. Good maintenance and cleaning procedures will ensure additional noise from out of balance or worn roof mounted ventilation fans is unlikely to occur. Effective inspection and maintenance forms a key part of compliance with permit condition 1.1.1 on environmental management systems and condition 1.1.2 on associated record keeping.

We are satisfied that vibration is unlikely to be an issue at the installation.  The nature of the intensive farming operation means that there are no significant sources of vibration on site.

We have compared the measures proposed for the site to the BAT standards in EPR 6.09 and are satisfied that the techniques represent appropriate measures for the installation. The NMP has been incorporated into the operating techniques section of the permit.

Permit condition 3.4.1 requires that emissions from the activities are free from noise at levels likely to cause pollution outside the site, as perceived by an officer of NRW.  We are satisfied that this condition will be sufficiently protective in conjunction with the measures described by the applicant for minimising odour production at the installation.

### 5.7	Impact on Habitats sites, SSSIs, non-statutory conservation sites etc

The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat.

A full assessment of the application and its potential to affect the sites has been carried out as part of the permitting process.  We consider that the application will not affect the features of the sites.

The following sites have been considered:

SAC, SPA and Ramsar sites (within 10km):

Midland Meres & Mosses Ramsar, which lies approximately 4.89km north north-east of the site.

Granllyn SAC, which lies approximately 9.13km south west of the site.

Montgomery Canal SAC, which lies approximately 1.34km west of the site.

Tanat and Vyrnwy Bat Sites SAC, which lies approximately 9.79km north west of the site.

SSSI (within 5km):

Morton Pool & Pasture SSSI is located approximately 4.9km north east from the proposed installation.

Llanymynech & Llynclys Hills (Wales) SSSI is located approximately 2.6km north north-west from the proposed installation.

Llanymynech & Llynclys Hills (England) SSSI is located approximately 3.1km north north-west from the proposed installation.

Blodwel Marsh SSSI is located approximately 4.3km north north-west from the proposed installation.

Breidden Hill SSSI is located approximately 4.6km south from the proposed installation.

Gweunydd Ty-Brith (Ty-Brith Meadows) SSSI is located approximately 3.5km west south-west from the proposed installation.

Montgomery Canal SSSI is located approximately 1.3km west from the proposed installation.

We also considered the potential impacts on nearby ancient woodlands, local wildlife sites, local nature reserves and national nature reserves (within 2km). Only one area of plantation on Ancient Woodland was identified within 2km. This area of woodland is 1.98km west of the proposed installation.

The applicant has carried out screening of the potential impact of dispersion and deposition of ammonia from the site. The submitted report concludes that should the permit be granted at Domgay Hall Broiler Unit, the ammonia emissions from the unit would increase. The modelling predicts that the maximum annual ammonia concentrations at all the receptors considered except one, would remain below the threshold (20% of CLe (Critical Level) for SSSI, 4% of CLe for SAC, SPA and Ramsar site and 100% of CLe for Ancient Woodland (3 μg/m3)).

Following the initial screening, the Montgomery Canal SAC (1.34km west of the installation) was the only site above the 4% CLe. The applicant then carried out detailed modelling, which predicted a below 1% ammonia process contribution (PC) from Domgay Hall Broiler Unit on the Montgomery Canal SAC.

Our conclusions were that the proposal is not likely to have a significant effect on any of these sites.

HRA consultation:

OGN Form 1 and CRoW Appendix 4 have been completed and forwarded onto our internal Natural Resource Management (NRM) team for consultation and notification. These forms were also sent to Natural England for external consultation, due to the proximity of Domgay Hall Broiler Unit to the England-Wales border. Natural England assessed the impact of ammonia emissions on Midlands Meres and Mosses SAC and concluded that ammonia emissions are below the significance threshold for the site.

Full details of the assessment carried out of the potential effect of ammonia emissions from the proposed site on any SSSI and Natura 2000/Ramsar site referenced above are detailed in the forms. The forms are available from our public register. It was also noted that there were no known European Protected Species that would be adversely impacted by this installation.

## 6  Setting ELVs and other Permit conditions

### 6.1	Translating BAT into Permit conditions

Article 14(3) of the Industrial Emissions Directive (IED) states that BAT conclusions shall be the reference for setting the permit conditions to installations covered by the Directive. As a result of the Commission Implementing Decision (EU) 2017/302 of 15 February 2017 establishing best available techniques (BAT) conclusions, under Directive 2010/75/EU of the European Parliament and of the Council, for the intensive rearing of poultry or pigs, the format of our Permit for the intensive farming sector has been updated. Appendix 1 of the Permit sets out generic conditions which apply to all sites. Appendix 2 sets out site specific conditions based on the activities being carried out.

### 6.2	Monitoring

Monitoring should be carried out for the parameters listed in Appendix 1, Schedule 3 of the permit using the methods and to the frequencies specified in Table S3.1 for broiler chickens. These monitoring requirements have been introduced in order to demonstrate compliance with the best available techniques (BAT) conclusions for the intensive rearing of poultry or pigs, as set out in the Commission Implementing Decision (EU) 2017/302 of 15 February 2017.

No monitoring is required from the point source emissions on site.

### 6.3	Reporting

We have specified reporting requirements in Appendix 2, Schedule 4 of the Permit to ensure compliance with permit conditions and to monitor the efficiency of farming activities at the site in line with BAT. We made this decision in accordance with EPR 6.09 “How to Comply with your Permit for Intensive Farming”.

### 6.4 Pre-operational conditions

Based on the information in the application, we do not consider that we need to impose pre-operational conditions.

### 6.5 Improvement conditions

Based on the information on the application, we do not consider that we need to impose improvement conditions.

# ANNEX 1:	Consultation Reponses

### A)	Advertising and Consultation on the Application

The Application has been advertised and consulted upon in accordance with Natural Resources Wales Public Participation Statement.  The way in which this has been carried out along with the results of our consultation and how we have taken consultation responses into account in reaching our draft decision is summarised in this Annex.  Copies of all consultation responses have been placed on Natural Resources Wales public register.

### 1)	Consultation Responses from Statutory and Non-Statutory Bodies

| Response Received from Public Health Wales |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| The applicant needs to assess the potential human health impacts for ammonia, odour and fugitive emissions. The manure management plan should also consider impacts from dusts or bio-aerosols. | Permit conditions cover concerns |

### 2)	Consultation Responses from Members of the Public and Community Organisations

A number of the issues raised during the consultation process are outside Natural Resources Wales remit in reaching its permitting decisions.  Specifically questions were raised which fall within the jurisdiction of the planning system, both on the development of planning policy and the grant of planning permission.  Specific planning issues raised related to the location of the site, the location of the stack, traffic movements and emissions from off-site traffic movements.

Guidance on the interaction between planning and pollution control is given in PPS23 / Planning Policy Wales.  It says that the planning and pollution control systems are separate but complementary.  We are only able to take into account those issues, which fall within regulatory scope of the Environmental Permitting Regulations.

### a)	Representations from Local MP, Assembly Member (AM), Councillors and Parish / Town / Community Councils

| Response Received from |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| None | N/A |

### b)	Representations from Community and Other Organisations

| Response Received from |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| None | N/A |

### c)	Representations from Individual Members of the Public

| Response Received from |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| None | N/A |