[image: image2.png]Cyfoeth
Naturiol
Cymru

Natural
Resources
Wales





Notice of variation and consolidation with introductory note

The Environmental Permitting (England & Wales) Regulations 2016
Essity UK Limited

Scour valve discharge from 

Oakenholt Mill Reservoir

Oakenholt Mill Reservoir
Chester Road

Flint 

Flintshire

CH6 5PU

Variation application number

PAN-015487
Permit number
EPR/BB3696ZC
Scour valve discharge from

Oakenholt Mill Reservoir

Permit number EPR/BB3696ZC
Introductory note

This introductory note does not form a part of the notice.

The following notice gives notice of the variation and consolidation of an environmental permit.

The variation is to allow the following discharges as required for ongoing maintenance and safety of the dam to comply with Reservoir Act 1975 and Eel regulations to open up access up stream to support the EPS. NRW are to be notified of any intention to release raw water a minimum 14 days in advance.  All activities are to be carried out in accordance with written management plan detailed in Table 1.2 of this permit.

The request to discharge in an emergency is not included in this permit, for this is covered under emergency provision of EPR 2016.
Discharge Activates permitted.

1. A 6 hour Drawdown Test (with 14 day prior notice to NRW) as required to assist in Emergency Planning – justification to secure Public Safety. This equates to 6000m3.

2. Drawdown up to 3 m below Spill Level to assist data gathering by Geophysical Surveys – justification to secure Public Safety.

3. Drawdown to 1 m below Spill Level to assist any construction or any future maintenance works to the Spillways or any other part of the dam structure and maintenance of downstream water course (with 14 day prior notice to NRW) – justification Contractor Safety during a project to secure Public Safety.

4. Drawdown by Pump (not the Scour Valve) to 1 m below Spill Level to assist Hydraulic Testing during the construction of the Auxiliary Spillway (water will return to the Lead Brook Watercourse).

5. Exercising of the scour valve for a maximum of 20 minutes on up to 12 occasions per annum as required by the Supervising Engineer or All Reservoir Panel Engineer appointed under the Reservoirs Act 1975 discharging 222 l/s for 20 minutes.

6. An additional drawdown to 1 m below Spill Level for the anchoring of the Mussel Rope installation of an eel passage required by NRW to meet the requirements of Eel Regulations (England and Wales) 2009 as amended.
The schedules specify the changes made to the permit.
The status log of a permit sets out the permitting history, including any changes to the permit reference number.  It is not backdated before 6 April 2010. 
	Status log of the permit

	Description
	Date
	Comments

	Application received
	Duly made 21/02/2020
	Application for discharge of raw water.

	Permit determined 

EPR/BB3696ZC 
	27/05/2020
	Permit issued to Essity UK Limited.

	Application PAN-015487
(variation and consolidation)
	Duly made

27/10/2021
	Application to vary and update the permit to increase number of discharges of raw water.

	Variation determined EPR/BB3696ZC
	15/02/2022
	Varied permit issued to Essity UK Limited


End of introductory note

	Notice of variation and consolidation 


The Environmental Permitting (England and Wales) Regulations 2016
The Natural Resources Body for Wales (“Natural Resources Wales”) in exercise of its powers under regulation 20 of the Environmental Permitting (England and Wales) Regulations 2016 varies and consolidates
permit number

EPR/BB3696ZC
issued to:

Essity UK Limited (“the operator”)

whose registered office is

Southfields Road

Dunstable

Bedforshire

LU6 3EJ

company registration number 03226403
to operate a regulated facility at
Oakenholt Mill Reservoir 
Chester Road

Flint

Flintshire

CH6 5PU

to the extent set out in the schedules.
The notice shall take effect from 15/02/2022
	Name
	Date

	Melissa Broad


	15/02/2022


Authorised on behalf of Natural Resources Wales
Schedule 1 

Only conditions 2.2, 2.3, 3.2 and Table S1.1, S1.2, S3.1, S3.2 and S3.3 have been varied by the consolidated permit ERP/BB3696ZC as a result of the application made by the operator.

Schedule 2 – consolidated permit

Consolidated permit issued as a separate document. 
	Permit


The Environmental Permitting (England and Wales) Regulations 2016
Permit number 

EPR/BB3696ZC
This is the consolidated permit referred to in the variation and consolidation notice for application PAN-015487 authorising, 

Essity UK limited (“the operator”),

whose registered office is

Southfields Road

Dunstable

Bedforshire

LU6 3EJ

company registration number 03226403
to operate a water discharge activity at

Oakenholt Mill Reservoir
Chester Road

Flint

Flintshire

CH6 5PU

to the extent authorised by and subject to the conditions of this permit.

	Name
	Date

	Melissa Broad
	15/02/2022


Authorised on behalf of Natural Resources Wales
Conditions 

1 Management
1.1 General management

1.1.1 The operator shall manage and operate the activities:

(a) in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances and those drawn to the attention of the operator as a result of complaints; and

(b) using sufficient competent persons and resources.
1.1.2 Records demonstrating compliance with condition 1.1.1 shall be maintained. 

1.1.3 Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

2
Operations
2.1
Permitted activities

2.1.1
The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”). 

2.2
The site 

2.2.1
The activities shall not extend beyond the site, being the land shown edged in green and the discharges shall be made at the points marked on the site plan at schedule 7 to this permit and as listed in table S3.2 (discharge points).

2.3
Operating techniques

2.3.1
(a) 
The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

(b) 
If notified by Natural Resources Wales  that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan, and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

3
Emissions and monitoring
3.1
Emissions to water

3.1.1
There shall be no point source emissions to water except from the sources and emission points listed in schedule 3.

3.1.2
The limits given in schedule 3 shall not be exceeded.

3.2 Emissions of substances not controlled by emission limits

3.2.1 Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.
3.2.2
The operator shall:

(a)
if notified by Natural Resources Wales  that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

(b)
implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

3.3
Monitoring 

3.3.1
Permanent means of access shall be provided to enable sampling/monitoring to be carried out in relation to the emission points specified in schedule 3 tables S3.1, S3.2 and S3.3 unless otherwise agreed in writing by Natural Resources Wales.

4
Information

4.1
Records
4.1.1 All records required to be made by this permit shall:

(a) be legible;

(b) be made as soon as reasonably practicable;

(c) if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

(d) be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made.
4.1.2
The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales. 

4.2
Reporting
4.2.1
The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.
4.3
Notifications
4.3.1
Natural Resources Wales shall be notified without delay following the detection of:

(a) any malfunction, breakdown or failure of equipment or techniques, accident, or  emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution;

(b) the breach of a limit specified in the permit; or 

(c) any significant adverse environmental effects.  

4.3.2
Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.
4.3.3
Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

4.3.4
Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules: 

Where the operator is a registered company:
(a) any change in the operator’s trading name, registered name or registered office address; and

(b)
any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:
(a) any change in the operator’s name or address; and

(b) any steps taken with a view to the dissolution of the operator.

4.3.5
Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

(a)
Natural Resources Wales shall be notified at least 14 days before making the change; and


(b)
the notification shall contain a description of the proposed change in operation. 

4.4
Interpretation
4.4.1
In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

4.4.2
In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone. 

Schedule 1 - Operations 

	Table S1.1 Activities

	Activity reference
	Description of activity
	Limits of specified activity

	A1
	6hr draw down of raw water via Outlet 1
	N/A

	A2
	Drawdown of raw water up to 3 m below Spill Level via Outlet 1
	N/A

	A3
	Drawdown of raw water to 1 m below Spill level via Outlet 1
	N/A

	A4
	Drawdown of raw water by Pump (not the Scour Valve) to 1 m below Spill Level via Outlet 1
	N/A

	A5
	20 minutes Scour valve operation discharging raw water for a maximum 12 occasions per annum via Outlet 1
	N/A 


	Table S1.2 Operating techniques

	Activity reference
	Description of documentation
	Parts
	Date Received

	A1
	NRW Environmental Permit-Management Plan and Environmental Risk Assessment-Oakenholt Reservoir
	All
	16/11/2021

	A2
	
	
	

	A3
	
	
	

	A4
	
	
	

	A5
	
	
	


Schedule 2 - Waste types, raw materials and fuels

Wastes are not accepted as part of the permitted activities and there are no restrictions on raw materials or fuels under this schedule.

Schedule 3 – Emissions and monitoring

	Table S3.1 Point Source emissions to water (other than sewer) – emission limits and monitoring requirements

	Discharge source and discharge point ref. & location
	Parameter  
	Limit (including unit)
	Reference Period
	Limit of effective range
	Monitoring frequency
	Compliance Statistic

	A1 - 6hr draw down of raw water via Outlet 1
	Maximum daily discharge volume
	6000 m3/day


	Total daily volume
	N/A                                                                
	N/A 
	Maximum 

	
	Maximum rate of discharge
	222 litres per second
	Instantaneous (spot sample)
	N/A
	N/A
	Maximum

	
	Visible oil or grease
	No significant trace present
	Instantaneous (spot sample)
	N/A
	N/A
	No significant trace

	A2 - Drawdown of raw water up to 3 m below Spill Level via Outlet 1
	Maximum rate of discharge
	222 litres per second
	Instantaneous (spot sample)
	N/A
	N/A
	Maximum

	
	Visible oil or grease
	No significant trace present
	Instantaneous (spot sample)
	N/A
	N/A
	No significant trace

	A3 - Drawdown of raw water to 1 m below Spill level via Outlet 1
	Maximum rate of discharge
	222 litres per second
	Instantaneous (spot sample)
	N/A
	N/A
	Maximum

	
	Visible oil or grease
	No significant trace present
	Instantaneous (spot sample)
	N/A
	N/A
	No significant trace

	A4 -Drawdown of raw water by Pump (not the Scour Valve) to 1 m below Spill Level via Outlet 1
	Maximum rate of discharge
	222 litres per second
	Instantaneous (spot sample)
	N/A
	N/A
	Maximum

	
	Visible oil or grease
	No significant trace present
	Instantaneous (spot sample)
	N/A
	N/A
	No significant trace

	A5 - 20 minutes Scour valve operation discharging raw water for a maximum 12 occasions per annum via Outlet 1
	Maximum rate of discharge
	222 litres per second
	Instantaneous (spot sample)
	N/A
	N/A
	Maximum

	
	Maximum daily discharge volume 
	267 m3/day
	Total daily volume
	N/A
	N/A
	Maximum

	
	Visible oil or grease
	No significant trace present
	Instantaneous (spot sample)
	N/A
	N/A
	No significant trace


	Table S3.2 Discharge points

	Activity reference
	Effluent Name
	Discharge Point
	Discharge point NGR
	Receiving water/Environment

	A1
	Raw water 
	Outlet 1
	SJ 26303 71203
	Lead Brook

	A2
	
	
	
	

	A3
	
	
	
	

	A4
	
	
	
	

	A5
	
	
	
	


	Table S3.3 Monitoring points

	Activity reference
	Effluent(s) and discharge point(s)
	Monitoring type
	Monitoring point NGR
	Monitoring point reference

	A1
	Raw water via 
Outlet 1
	Effluent sample point
	SJ 26303 71203
	Outlet 1

	A2
	
	
	
	

	A3
	
	
	
	

	A4
	
	
	
	

	A5
	
	
	
	


.
Schedule 4 – Reporting

There is no reporting under this schedule.
Schedule 5 - Notification 

These pages outline the information that the operator must provide. 

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

Part A 
	Permit Number
	EPR/BB3696ZC

	Name of operator
	


Oakenholt Mill, Chester Road, Flint, Flintshire, CH6 5PU

	
	

	Time and date of the detection 
	


	(a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution

	To be notified within 24 hours of detection

	Date and time of the event
	

	Reference or description of the location of the event 
	

	Description of where any release into the environment took place
	

	Substances(s) potentially released
	

	Best estimate of the quantity or rate of  release of substances
	

	Measures taken, or intended to be taken, to stop any emission
	

	Description of the failure or accident.
	


	(b) Notification requirements for the breach of a limit

	To be notified within 24 hours of detection unless otherwise specified below

	Emission point reference/ source
	

	Parameter(s)
	

	Limit
	

	Measured value and uncertainty
	

	Date and time of monitoring
	

	Measures taken, or intended to be taken, to stop the emission
	


	Time periods for notification following detection of a breach of a limit

	Parameter
	Notification period

	
	

	
	

	
	


	(c) Notification requirements for the detection of any significant adverse environmental effect

	To be notified within 24 hours of detection

	Description of where the effect on the environment was detected
	

	Substances(s) detected
	

	Concentrations of substances detected
	

	Date of monitoring/sampling
	


Part B - to be submitted as soon as practicable
	Any more accurate information on the matters for notification under Part A.
	

	Measures taken, or intended to be taken, to prevent a recurrence of the incident
	

	Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission
	

	The dates of any unauthorised emissions from the facility in the preceding 24 months.
	


	Name*
	

	Post
	

	Signature
	

	Date
	


* authorised to sign on behalf of the operator
Schedule 6 - Interpretation 

"accident" means an accident that may result in pollution.

"annually" means once every year.
“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.
“emissions of substances not controlled by emission limits ” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission limit.
“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.
“Natural Resources Wales” means the Natural Resources Body for Wales established by article 3 of the Natural Resources Body for Wales (Establishment) Order 2012. The Natural Resources Body for Wales (Functions) Order 2013 transferred the relevant functions of the Countryside Council for Wales, and functions of the Environment Agency and the Forestry Commission in Wales to the Natural Resources Body for Wales
"quarter" means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.
“year” means calendar year ending 31 December.

Schedule 7 - Site plan

[image: image1.png]



© Crown Copyright and database right 2022. Ordnance Survey licence number 100019741.
END OF PERMIT

	Variation and consolidation application number  EPR/BB3696XC/V002
	Page i
-  -
- i -
- i -i
	


PAGE  
	Variation and consolidation application number  EPR/BB3696ZC/V002
	Page 3
-  -
- 3 -
- 3 -3
	



