# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Maes Mynan (Lloyds Quarry) operated by Breedon Trading Limited.

The application number is PAN-015222.

The permit number is EPR/CB3399FW.

The permit is for the regulation of dewatering water (comprising a mixture of rainwater and site drainage) from a settlement lagoon within the quarry to a watercourse. The quarry was used for gravel washing but at present is only used to store and move aggregates that do not require washing.  The dewatering of the quarry will be carried out after periods of heavy rain to reduce the risk of flooding at the site whereby the discharge will be pumped from the lagoon to an unnamed tributary of the River Wheeler. Settlement in the lagoon is expected to achieve a suspended solids limit of 50 mg/l. The applicant has proposed a maximum 1000m3/day discharge, with a maximum discharge rate of 34.7 l/second.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | No responses were received from web publicising. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the site has been carried out as part of the permitting process. We consider that the application will not affect the features of the site. We have not formally consulted on the application. The decision was taken in accordance with our guidance. Although there is a Local Wildlife Site (SINC): Maes Mynan Quarry Wood within screening distance, it is not hydrologically connected to the discharge and so there is no pathway for impact. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. There are no designated sites within 10km downstream of the discharge point and so there is no pathway for impact to any designated sites. |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. Measures meet that as specified in TGN 7.01. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. - Suspended solids (measured after drying at 105oC) – 50 mg/l - pH between 6 -9 The suspended solids limit has been set in line with current Water Quality Standards and to ensure good lagoon design/operation in accordance with NRW’s “Lagoon Guidance 2008”. The pH limit has been set to safeguard the receiving environment. | ü |
| Monitoring | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods and to the frequencies specified. These monitoring requirements have been imposed in order to comply with TGN 7.01 Section 3 Requirements. The operator will be required to carry out flow monitoring in accordance with these requirements. We made these decisions in accordance with the Environmental Permitting Regulations 2016 and TGN EPR 7.01 “How to comply with your permit.” Based on the information in the application we are satisfied that the operator’s techniques, personnel and equipment have either MCERTS certification or MCERTS accreditation as appropriate. | ü |
| Reporting | We have specified reporting in the permit. Reporting frequencies are in line with requirements set out in EPR 7.01. We made these decisions in accordance with the Environmental Permitting Regulations 2016 and TGN 7.01. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR 2016. | ü |