# Schedule 5- Notification

Part A

| Permit Number | PP3139GB |
| --- | --- |
| Name of Operator | Enovert (North) Limited |
| Location of Installation | Bangor Road Johnstown Wrexham LL14 6ET |
| Time and date of the detection | 22/06/2019 |

| b) Notification requirements for breach of a limit |  |
| --- | --- |
| To be notified within 24 hours of detection unless otherwise specified |  |
| Emission point reference/source | HAF SW2 Chloride (mg/l) 67mg/l 70mg/l 08/06/2019 |
| Parameter(s) |  |
| Limit |  |
| Measured value and uncertainty |  |
| Date and time of monitoring |  |
| Measures taken, or intended to be taken, to stop the emission | Surface Water Monitoring Point (SW2) will continue to be monitored monthly, in accordance with Environmental Permit PP3139GB. |

Part B

| Any more accurate information on the matters for notification under Part A. | After reviewing the Chloride concentration data since 2017 for SW2 in the previous SC5 (HAF239), it was highlighted that Chloride concentrations periodically exceeded the compliance limit of 67mg/l (See Table 1 below & Graph 1). The data since 2017 would seem to indicate historic breaches of the compliance limit mostly occur during the Winter into Spring seasons and possibly related to rainfall events. It was notable that the start of June 2019 was a particularly wet period. This fluctuations in Chloride concentration in SW2 may reflect the fluctuations in the Chloride concentration in the groundwater, as the groundwater is subsequently pumped from a nearby chamber into the SW lagoon, prior to discharge. A brief statistical analysis demonstrates the average concentration of Chloride in SW2 to be 69mg/l (see Graph 1) over the past two years, which is 2mg/l above the compliance limit. The Chloride concentration also demonstrates a Standard Deviation of +/-16mg/l. The latest result demonstrates a decline in concentration from May 2019 (81mg/l) although the result of 70mg/l is within the Standard Deviation (Graph 1) and 1mg/l above the average concentration of Chloride since 2017 (Graph1). |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident | Chloride concentrations will be continually monitored monthly and subsequent results reviewed to determine if any future trends in the Chloride concentration in surface water monitoring point SW2. |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment or harm which has been or may be caused by the emission |  |
| The dates of any unauthorised emissions from the installation in the preceding 24 months. | SC5 HAF239 – May 2019 SC5 HAF234 - March 2019 |

| Name* | Chris Ridgway |
| --- | --- |
| Post | Environmental Data Analyst |
| Date | 08/07/2019 |

* authorised to sign on behalf of Enovert (North) Ltd

| Sample Point | Date | Cl (mg/l) |
| --- | --- | --- |
| HAF SW 2 | 10/01/2017 | 65 |
| HAF SW 2 | 28/02/2017 | 65 |
| HAF SW 2 | 21/03/2017 | 66 |
| HAF SW 2 | 18/04/2017 | 77 |
| HAF SW 2 | 09/05/2017 | 65 |
| HAF SW 2 | 15/06/2017 | 60 |
| HAF SW 2 | 18/07/2017 | 58 |
| HAF SW 2 | 20/09/2017 | 56 |
| HAF SW 2 | 10/10/2017 | 54 |
| HAF SW 2 | 15/11/2017 | 61 |
| HAF SW 2 | 05/12/2017 | 34 |
| HAF SW 2 | 16/01/2018 | 99 |
| HAF SW 2 | 20/02/2018 | 82 |
| HAF SW 2 | 27/03/2018 | 99 |
| HAF SW 2 | 24/04/2018 | 96 |
| HAF SW 2 | 16/05/2018 | 63 |
| HAF SW 2 | 12/06/2018 | 66 |
| HAF SW 2 | 25/07/2018 | 83 |
| HAF SW 2 | 20/09/2018 | 89 |
| HAF SW 2 | 11/10/2018 | 54 |
| HAF SW 2 | 14/11/2018 | 60 |
| HAF SW 2 | 04/12/2018 | 47 |
| HAF SW 2 | 22/01/2019 | 51 |
| HAF SW 2 | 21/02/2019 | 73 |
| HAF SW 2 | 19/03/2019 | 83 |
| HAF SW 2 | 18/04/2019 | 67 |
| HAF SW 2 | 07/05/2019 | 81 |
| HAF SW 2 | 08/06/2019 | 70 |

Table 1: Chloride Concentration SW2 since 2017 including permit breaches.

Graph 1: Chloride Concentrations in SW2 at Hafod Quarry since January 2017 to present