# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Rhosgoch Holiday Park operated by Mr Jason Winter.

The application number is PAN-016127

The permit number is EPR/CB3496HE

The discharge is for an existing discharge of septic tank treated effluent installed in 1983. A maximum of 6.7 cubic metres per day will go to ground via an infiltration system. The system serves a 4-bedroom house, 9 x 2-bedroom and 6 x 1-bedroom static caravans.

### Owing to the proximity to the nearest surface water receptor an improvement condition that requires the installation of a new British Standard Package Treatment Plant and infiltration system has been added as a condition to this permit.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | The web publicising, responses (Annex 2) were taken into account in the decision. No Responses received The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| The facility |  |  |  |
| The regulated facility | ###### The extent/nature of the facilities taking place at the site required clarification. The regulated facility is a standalone groundwater activity in accordance with EPR RGN 2 Understanding the meaning of regulated facility. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the special site has been carried out as part of the permitting process. We consider that the application will not affect the features of the site | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN 7.01 and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Improvement conditions | Based on the information on the application, we consider that we need to impose improvement conditions. We have imposed improvement conditions to ensure that the environmental integrity of the local groundwater is maintained and to ensure that there is no pollution reaching the nearby watercourse. When undertaking the Environmental Risk Assessment the existing system identified an environmental risk. When applying the improvement condition the Environmental Risk Assessment shows a score that is acceptable. This requires; The operator shall install a Package Treatment Plant that complies with British Standard EN12566-3 and a drainage field that shall be designed in accordance with the British Standard BS 6297:2007+A1:2008 ‘Code of practice for the design and installation of drainage fields for use in wastewater treatment’. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a groundwater discharge activity permit under EPR2016. | ü |