Permit Number: 	EPR/XP3830UR			Operator: 		The First Milk Cheese Company Limited

Facility:		Haverfordwest Creamery	                      Form Number: 	Water1 / 04/12/2023

# Reporting of emissions to water (other than to sewer) and land for the period from XX/XX/XXXX to XX/XX/XXXX

| Emission Point | Substance / Parameter | Emission # Limit Value [5] | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| W2 | Maximum daily discharge volume | 2500 m3/day (total daily volume) | Total daily volume |  | Flow meter |  |  |
|  | Maximum rate of discharge | 120 m3/hr | Instantaneous (continuous) |  | Flow meter |  |  |
|  | Temperature | 35 ºC (absolute maximum) | Instantaneous (continuous) |  | Standard thermocouple |  |  |
|  | Biochemical oxygen demand | 40 mg/L (absolute maximum) | Instantaneous (spot sample) |  | BS EN 1899-1 |  |  |
|  |  | 20 mg/L (95th percentile) | Instantaneous (spot sample) |  |  |  |  |
|  | Chemical oxygen demand (COD) | 125 mg/L (daily average) | 24 hour flow proportional composite |  | BS 6068-2.34 |  |  |
|  | Total suspended solids | 50 mg/L (daily average) | 24 hour flow proportional composite |  | BS EN 872 |  |  |
|  | Ammonia | 10 mg/L (absolute maximum) | Instantaneous (spot sample) |  | BS EN ISO 11732 |  |  |
|  |  | 3.5 mg/L (95th %ile) | Instantaneous (spot sample) |  |  |  |  |
|  | Total nitrogen | 20 mg/L (daily average) | 24 hour flow proportional composite |  | BS EN 12260 or BS EN ISO 11905-1 |  |  |
|  | Total phosphorus | 4 mg/L (daily average) | 24 hour flow proportional composite |  | BS EN ISO 11732 |  |  |
|  | Iron | 10 mg/L (absolute maximum) | Instantaneous (spot sample) |  | BS EN ISO 11885 |  |  |
|  |  | 5 mg/L (95th %ile) | Instantaneous (spot sample) |  |  |  |  |
|  | Aluminium | 2 mg/L (absolute maximum) | Instantaneous (spot sample) |  | BS EN ISO 11885 |  |  |
|  |  | 1 mg/L (95th %ile) | Instantaneous (spot sample) |  |  |  |  |
|  | pH | 6 – 9 (minimum, maximum) | Instantaneous (continuous) |  | BS ISO 10523 |  |  |
|  | Chloride (Cl-) | No limit set | 24 hour flow proportional composite |  | Various EN standards available including EN ISO 10304-1 and EN ISO 15682 |  |  |
|  | Mercury and its compounds [6] | 228 g [6] | - |  | Compliance based in mass calculation |  |  |
|  | Cadmium and its compounds [6] | 96 g [6] | - |  | Compliance based on mass calculation |  |  |

Notes:

[1] The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

[2] Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

[3] For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

[4] The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

[5] For 95%ile of all measured values according to look up table

[6] This is an annual limit and only requires to be reported annually.

Signed ………………………………………………		Date………………………..

(Authorised to sign as representative of Operator)