# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for A465 section 5 and 6 Dowlais Top to Hirwaun – Trewaun Site, operated by FCC Construccion.

The application number is PAN-015813

The permit number is EPR/CB3497FE

The permit regulates the discharge of rainfall dependent site drainage and dewatering of excavations during road construction along the A465. Treated runoff and excavation waters will be discharged at a maximum rate of 13.8 l/s to a surface water culvert leading to the Afon Cynon.

Treatment consists of a 1848m3 settlement lagoon, followed by chemical settlement using flocculants and coagulants and pH correction. Chemical settlement will take place using an onsite Siltbuster treatment system which is located adjacent to a slurry pond to allow for desludging of the unit. The coagulant “PAC Brenntag” will be added to the unit at a maximum dose rate of 20mg/l at a flow rate of 40m3/hr. The flocculant “EM533 AquaTreat” will be added to the unit at a maximum dose rate of 2mg/l at a flow rate of 40m3/hr. If required, Sodium Hydroxide (Caustic Soda Liquor) will be added at a maximum dose rate of 50mg/l at a flow rate of 40m3/hr for pH correction purposes.

Numeric limits have been put in place for suspended solids (25 mg/l) and a neutral range of 6-9 has been imposed for pH. We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising. | The web publicising responses were taken into account in the decision. No responses were received. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN (EPR 7.01) and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. A pH range of 6-9 has been imposed to ensure that the discharged effluent remains neutral. A suspended solids limit of 25 mg/l has been imposed based on the treatment efficiencies of the chemical treatment facility on site as well as the lower limit of annual means solids for migratory passage and potential spawning grounds of salmonoid fish. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |