# Contents

1.0	Introduction and Scope	2

2.0	General	2

2.1	Permit details	2

2.2	Site location / plans	2

2.3	Operational hours	3

2.4	Permitted operations	3

2.5	Permitted waste types	3

2.6	Site management / competence	4

2.7	Document & Record Control	4

3.0	Site Infrastructure & Equipment	5

3.1 	Notice Board	5

3.2	Site security	5

3.3	Waste storage areas	5

3.4	Fuel storage	5

3.5	Maintenance	6

4.0	Site specific processes, activities and operations	6

4.1	Waste Acceptance Criteria	7

4.2       Production and Storage	7

4.3	Non-Conforming Waste	7

4.4       Testing	8

4.5       Transport & Delivery	9

4.6	Site checks	9

5.0	Emissions to Air, Water and Land	9

5.1	Point Source Emissions	9

5.2	Fugitive Emissions	9

5.2.1	Fugitive emissions to water	9

5.2.2	Fugitive emissions to air	9

5.2.3	Odour	10

5.2.4	Control of pests, birds and other scavengers	10

5.2.5	Noise and vibration	10

6.0	Incidents, Non Conformances & Complaints	10

7.0	Emergency Response & Accident Management	11

Appendix 1 – Site Plan	12

Appendix 2 – Accident Management Plan	13

Appendix 3 – Weekly Site Checklist	17

1.0	Introduction and Scope

Horan Construction has introduced this systems manual for the environmental, quality management and Factory Production Control of aggregates, produced from inert waste, which will be sold as construction products, or as constituents within products, e.g. unbound materials for its operations at the Clipper Road site.

The manual forms part of the wider Integrated Management System operated by Horan and draws upon some of the processes and procedures contained within it which are referenced throughout this document. However, due to the uniqueness of the operations at this site, this manual has been prepared to give additional information on the controls in place to safeguard the environment at this site.

A site plan is included in Appendix 1 which shows the site boundaries and the locations of waste processing activities.

# 2.0	General

## 2.1	Permit details

Horan Construction are applying for a Tier 2 bespoke permit for the treatment of waste to produce soil, soil substitutes and aggregate. The Site will only accept waste arising from their own excavations and shall only consist of inert materials including stone, rock, earth, concrete, tarmac etc. The total tonnage anticipated to be dealt with by the site is 10,000 T per annum.

## 2.2	Site location / plans

The site is located within the operational docks with no sensitive receptors in direct proximity to the site, and with other industrial type land uses (operational docks, car crushing plant) between the site and the nearest residential properties 960m to the south east off Pierhead Street. The Site address is:

Horan Clipper Road Site

Land off Clipper Road,

Cardiff Bay,

Cardiff,

CF10 4LX

## 2.3	Operational hours

## The Hours and days of operation are specified on the current planning application form as:

## Monday to Friday from 8:00 to 18:30; and

## Saturday from 8:00 to 18:30.

## There will be no works on Sundays or Bank Holidays.

## 2.4	Permitted operations

## R13: Storage of wastes pending the operations numbered R3 and R5.

## R3: recycling or reclamation of organic substances which are not used as solvents.

## R5: Recycling or reclamation of other inorganic materials.

## 2.5	Permitted waste types

Waste shall only be accepted if it is a type and quantity specified in the Permitted list of wastes, and if it conforms to the description in the documentation supplied by the producer and holder.

In addition to our Permit requirements and in order to apply the waste hierarchy as required by regulation 12 of the Waste (England and Wales) Regulations 2011 (as amended), we will only accept waste materials that are covered by the Wrap Quality Protocol (See table of accepted EWC Codes below) and is inert as described in Appendix A of the Wrap Quality Protocol. Very minor contamination of wood, plastic and metal is permitted provided that it does not exceed 1% by mass.

Any wastes that are not categorised in Appendix 2 (Permitted Wastes) should be considered contrary/non-conforming and dealt with appropriately.

Table 1 – Acceptable EWC codes under both the Site Permit and Quality Protocol

| European Waste Catalogue Code | Description | Restrictions |
| --- | --- | --- |
| 17 01 01 | Concrete including solid dewatered concrete process waste | Selected construction and demolition waste acceptable only with low content of other types of materials (like metals, plastics, organics, wood, rubber etc.) The origin of the waste must be known. Must not include concrete slurry. |
| 17 01 02 | Bricks |  |
| 17 01 03 | Tiles and ceramics |  |
| 17 01 07 | Mixtures of concrete, bricks, tiles and ceramics |  |
| 17 02 02 | Glass | Must not include fibreglass or glass fibre |
| 17 03 02 | Bituminous mixtures other than those mentioned in 17 03 01 | Allowed only if: - Bituminous mixtures from the repair and refurbishment of the asphalt layers of roads and other paved areas. - Must not include coal tar or tarred products. - Must not include freshly mixed bituminous mixtures. |
| 17 05 04 | Soil and stones other than those mentioned in 17 05 03 | Must not contain any contaminated soil or stone from contaminated sites. |
| 17 05 08 | Track ballast other than those mentioned in 17 05 07 | Allowed only if it does not contain soil and stones from contaminated sites. |
| 20 02 02 | Soils and stones | Must not contain contaminated stones from garden and parks waste. |

## 2.6	Site management / competence

Based on the guidance outlined in the Environment Agency’s ‘How to comply with your Permit’ (Version 6 2013), we have calculated that we are required to have a COTC holder on site for one day per operational week. Coverage will be provided by Adrian Drysdale who holds an Environmental Permitting Operators Competence (EPOC Certificate). A record of weekly operational hours and site attendance of the technically competent manager will be kept in the site diary.

Further to having sufficient COTC holders, we have also ensured that training needs are identified and relevant training plans are created for all staff (See IMS04 – Competence, Training and Awareness). A training program has been developed to inform all personnel and contractors of the importance of complying with the Environmental Policy, procedures and other requirements of the Environmental Management System. Employees are also made aware of the potential consequences of departure from procedures.

All employees are made aware of their role and responsibilities and will be provided with training appropriate to the activities performed. Competent personnel are assigned to specific tasks and selected on the basis of their education, training and experience. Training needs and requirements are continually monitored and evaluated to ensure that levels of competence remain appropriate for the tasks assigned.

Should Sub-contractors be used in any part of the process, they will operate under the requirements of this manual and steps will be taken to ensure that sub-contractors meet all specified requirements.

## 2.7	Document & Record Control

All documentation and records issued in conjunction with this Manual is subject to control and is administered and maintained in accordance with IMS05 – Document and Record Control. This procedure describes the approval and issue of documents, how changes are made, revision levels identified and obsolete documents retrieved. Current versions of relevant documents e.g. this Manual, Environmental Permit etc. and other data will be available at the Clipper Road site for reference by operational staff.

# 3.0	Site Infrastructure & Equipment

## 3.1 	Notice Board

In conformance with permitting regulations and the Management System, Horan Construction Limited, shall display a clear, all-weather, easily readable Site Notice Board at or near the entrance to the site.

The Site Notice Board shall contain the following information:

- Company Name

- Permit Holder’s Name

- Emergency Contact Name

- Permit Holder’s Telephone Number

- Statement that the site is permitted by Natural Resources Wales

- The Permit Number

- NRW National Numbers

- Any other NRW Telephone Number required

The Notice Board shall be inspected at least once per operational week. In the event of damage or defect, the board shall be repaired or replaced within three working days.

## 3.2	Site security

The Site is located within the Docks and therefore the entrance is manned 24/7. In addition to this sufficient fencing encompasses the site which will be locked when not in use.

## 3.3	Waste storage areas

The location of the operational and storage areas are shown on Drawing PL-02 in Appendix 1. All waste material will be stored on hard standing. Permitted waste types do not include sludges or liquids. No point source emissions to water are permitted.

Clearly labelled enclosed skip/containers will be provided for the deposit of rejected waste which cannot be removed from the site immediately. The location of these skips may be varied as operating conditions permit (i.e. to permit the loading of rejected wastes but clear labelling and management control will ensure its use as specified).

## 3.4	Fuel storage

Any fuel tanks which are located on site will be either contained by a bund capable of containing a minimum of 110% of the volume of fuel stored in the tank or consist of double skinned tanks (mobile fuel tanks). Checks are made of all bunds / fuel stores as part of the Weekly Checklist. Spill training and equipment will also be provided for all those responsible for conducting operations at the Site.

## 3.5	Maintenance

Poor maintenance is a common cause of both human harm and environmental incidents. With this in mind we have developed stringent procedures (IMS09 – Maintenance & Calibration) which include:

- A programme of Planned Preventative Maintenance (PPM) to ensure minimal risk to the Environment shall take place;

- The inspection and maintenance schedules recommended by plant and equipment manufacturers shall be adopted where practicable;

- Preventative maintenance on the Plant and Equipment, Reception Areas and Processing Areas shall take place (this includes daily defect checks and weekly site checks).

- A full list of equipment utilized at the site along with the maintenance and inspection Schedules will be maintained in accordance with IMS09.

# 4.0	Site specific processes, activities and operations

The following process flow diagram outlines the various stages and controls that are in place to ensure that all aggregates meet both legal requirements and the requirements of the Wrap Quality Protocol for the Production of Recycled Aggregates.

## 4.1	Waste Acceptance Criteria

Our Planning Permission allows for the acceptance of no more than 10,000 tonnes of inert waste per annum. In order to ensure that this limit is not exceeded, a record of each delivery will be recorded in the Site Diary and a running total of throughput is recorded. In addition to our Permit requirements and in order to apply the waste hierarchy as required by regulation 12 of the Waste (England and Wales) Regulations 2011 (as amended), we will only accept waste materials that are covered by the Wrap Quality Protocol (See Table 1 of accepted EWC Codes) and is inert as described in Appendix A of the Wrap Quality Protocol. Very minor contamination of wood, plastic and metal is permitted provided that it does not exceed 1% by mass.

If there is any uncertainty over whether the waste generated off site is likely to be contaminated or does not meet the ‘inert’ criteria, we will ensure that it undergoes chemical analysis and assessment to determine whether or not it is suitable for the intended use without significant risk of pollution.

On arrival at site a visual inspection of each load is also undertaken prior to tipping to ensure the contents comply with the acceptance criteria outlined in both the Site Permit and the Wrap Quality Protocol.

Loads which are contaminated or deemed as unacceptable are rejected from the process and a record of rejected loads is kept in the site diary. Where materials are rejected from the site there are processes in place for quarantine, re-loading, removal and disposal of the load where applicable. If the load is deemed acceptable, it is directed to the appropriate storage area and stockpiled for processing.

## 4.2       Production and Storage

As the Site will only handle waste generated by Horan Construction Limited and in relatively small quantities, it is envisaged that processing will only take place intermittently for 3 or 4 days per month. Processing of waste will only be undertaken within the site boundary (see Appendix 1) and when not in use, the crusher will be located on the concrete pad (see Appendix 1).

Prior to operating, an inspection of the Crusher (Nakayama – MC240) shall take place to ensure there is no damage which could compromise the effectiveness of the process and repairs are undertaken where necessary. Any large contaminants are removed either mechanically or manually before the material is put through the processing equipment.

Once the material has been processed it is placed into stockpiles and all product is inspected prior to loading for normal appearance. If materials appear different from normal they are reprocessed. When necessary material is re-crushed at a later date.

During the processing of input materials and the handling and storage of products, care will be taken to ensure appropriate segregation of waste material and product.

## 4.3	Non-Conforming Waste

Non-conforming wastes discovered at any stage in the process will be deposited in the skip / container provided for non-conforming wastes. Where necessary, particularly where the non-conformance waste discovered would be classed as a difficult, hazardous or clinical waste, Natural Resources Wales will be contacted to agree a course of action. Where necessary, a record of the actions taken will be recorded in accordance with IMS16 – Non-Conformance

## 4.4       Testing

The only product produced at the site is an ‘Unbound General Fill’ material from recycled aggregates for which there is no harmonized European Standard (hEN). Therefore, there is no requirement to provide CE marking for these products under the Construction Products Regulations 2013. However, we will undertake product testing in line with the requirements of the relevant Standards and Guidance outlined in Table 2 below and on request of the Customer if necessary.

In order to ensure that we are meeting the frequency requirements for testing, a log of production hours / days shall be kept in the Site Diary. Where external testing is required we will utilize the services of a UKAS accredited test laboratory. Records of all testing are retained indefinitely and are provided to customers on request. If the testing identifies any issues with the products, then the appropriate action is taken.

The affected material is either:

- Reprocessed

- Diverted to another application for which it is suitable

- Rejected, isolated and marked as Non-conforming

In all cases of non-conformity, records are maintained of the investigation and any necessary corrective action and preventive actions taken (recorded in accordance with IMS16).

Table 2 – Minimum Test Frequencies for unbound recycled aggregates

| Property | BS EN test method | Product Standards/Specifications | Minimum test frequency |
| --- | --- | --- | --- |
| Grading | BS EN 933-1 | BS EN 13242 Aggregates for unbound and hydraulically bound mixtures | 1 per week of production working days |
|  |  | MCHW series 500/600/800 references BS EN 13242 |  |
| Particle shape | BS EN 933-3 and BS EN933-4 | BS EN 13242 Aggregates for unbound and hydraulically bound mixtures | PD 6682-6 recommends ‘no requirement’ |
| Particle density | BS EN 1097-6 | BS EN 13242 Aggregates for unbound and hydraulically bound mixtures | 1 per month of production working days |
| Composition | BS EN 933-11 | BS EN 13242 Aggregates for unbound and hydraulically bound mixtures | 1 per month of production working days |

## 4.5       Transport & Delivery

Product will only be used by Horan Construction Limited and will not be sold to any third parties. When dispatching product provisions shall be made to ensure that loads are secure and within specified weight limits. A running total of loads will also be kept in order to demonstrate the quantity of material we have re-processed and re-used.

## 4.6	Site checks

To ensure that waste storage arrangements are being adhered with, weekly checks (if operating) of the site are conducted utilising the ‘Weekly Site Checklist’ (See Appendix 3). If issues are identified, then records are made and suitable actions are determined in accordance with IMS16 – Non-Conformance.

# 5.0	Emissions to Air, Water and Land

## 5.1	Point Source Emissions

There shall be no point source emissions to air, water or land from any of the activities undertaken at the Clipper Road Site.

## 5.2	Fugitive Emissions

### 5.2.1	Fugitive emissions to water

Permitted waste types do not include sludges or liquids but there is potential for contaminated rainwater run-off from wastes stored outside especially during heavy rain. However, only waste that is inert shall be accepted on site and therefore the risk of contamination from this material is very low.  Silt traps on the site will stop any direct emissions of run off from stockpiles/suspended solids.

The Site is over 450m from the Severn Estuary - designated SSSI, SAC, SPA and Ramsar site, there is very little risk of emission straight to this receptor.

### 5.2.2	Fugitive emissions to air

A series of dust mitigation measures will be implemented on site to ensure dust emissions are controlled as far as is practically possible. The measures include:

- Sheeting of vehicles delivering waste to the site (if necessary);

- Sheeting of vehicles transporting potentially dusty loads off site;

- Use of dust suppression equipment on stockpiles of processed and unprocessed material where necessary.

- Halting of processing operations in excessive winds.

- Drop heights ALWAYS minimised to prevent dust emissions.

- Site operatives will continuously monitor dust emissions whilst the site is in operation and will report back to Management for advice if required.

### 5.2.3	Odour

The nature of the waste accepted at the site presents a low risk of odour nuisance.  If odour issues were identified, the relevant waste will be dealt with in accordance with IMS16 – Non-Conformance if necessary.

Any wastes identified as giving rise to odour will be quarantined where possible and removed from site immediately where practicable. If a complaint is received regarding odour then the details shall be recorded and investigated in accordance with IMS14 – External Communications and Complaints.

### 5.2.4	Control of pests, birds and other scavengers

It is unlikely that vermin/scavengers will present a problem, due to the waste types handled at the site.  The site will be inspected routinely for the presence of vermin/scavengers and the results recorded on the Weekly Checklist. A recognised pest control contractor will be brought in within 48 hours if any problems are encountered.

### 5.2.5	Noise and vibration

# Horan is currently undertaking environmental noise surveys over a period covering operating hours to confirm ambient Lmax, Leq and background L90 noise levels at up to 3 positions, adjacent to the nearest noise sensitive receiver locations. However, it is anticipated that the emissions from the activities shall be free from noise and vibration levels likely to cause pollution outside the site. For much of the operating hours the site will be relatively quiet, however there will be intermittent occurrences of noise when waste arrives to site, when processing takes place, and when materials leave site. Therefore, the following controls have been introduced:

- All vehicles, equipment and plant will be maintained with a clear intention to control noise and vibration levels.

- No treatment activities will take place outside permitted working hours (no evenings or weekends).

- All vehicles, equipment and plant will be switched off when not in use.

- There will be regular noise monitoring for susceptible plant and equipment.

- Where appropriate, abatement measures will be introduced.

# 6.0	Incidents, Non Conformances & Complaints

In the event of an incident or non-conformance, continued environmental performance and Permit compliance are essential. Therefore, as part of our IMS we have developed robust procedures for ensuring that appropriate actions to correct an immediate problem, as well as actions to prevent recurrence, are put in place when a non-conformance has been identified.

Moreover, our procedures require that a further review of other processes, in order to prevent similar occurrences, is part of this process. This process, including those responsible and having the authority for handling and investigating non-conformance, is documented in IMS16 - Non-conformance. This procedure is to be followed and a Corrective Action Request raised, for all accidents and near misses, non-conformances identified as part of an audit or daily check, customer complaints or general staff observations.

# 7.0	Emergency Response & Accident Management

As part of our IMS we have undertaken environmental risk assessments on all our operations (IMS01 – Aspects, Impacts and Risk). This has enabled us to determine which of our activities pose the greatest risk to the environment. As a result, we have developed an Accident Management Plan (see Appendix 2) which outlines the controls and mitigation measures that we have in place for the various activities that we undertake at Clipper Road.

# Appendix 1 – Site Plan

# Appendix 2 – Accident Management Plan

| Activity | Potential Impact(s) | Risk before controls | Control Measures | Risk after controls | Mitigation Measures | Associated Documents |
| --- | --- | --- | --- | --- | --- | --- |
| ARRIVAL |  |  |  |  |  |  |
| Transport of waste to and from site | Water pollution from oil leak | M | - Regular maintenance / servicing of all vehicles - Daily vehicle checks | L | - Spill kits | - Spill procedure - Weekly site checklist - Daily plant/vehicle checklist - Vehicle/equipment maintenance schedules |
|  | Dust emissions and transfer of mud onto public highway | M | - Sheeting / netting of all vehicles - No overfilling of vehicles - General housekeeping of site | L | - Dust suppression measures |  |
|  | Air pollution from vehicle use | M | - Regular maintenance / servicing of all vehicles | L | - Stop operating vehicle until it has deemed fit |  |
|  | Noise | L | - Operation confined to permitted hours - Regular maintenance / servicing of all vehicles | L | - Stop operating vehicle until it is deemed fit |  |
| STORAGE |  |  |  |  |  |  |
| Acceptance & storage of inert waste | Water and land contamination from leachate | L | - Storage on hardstanding - Stringent waste acceptance criteria - WAC testing if necessary - Regular site inspections | L | - Spill kits - Silt traps | - Spill procedure - Weekly site checklist |
|  | Odour | L | - Storage within Permit time and quantity limits - Inspections of each load | L | - Immediate removal of offensive wastes |  |
|  | Dust emissions | L | - Drop heights kept to a minimum - Regular site inspections | L | - Dust suppression systems |  |
|  | Attraction of flies and vermin | L | - Storage within Permit time and quantity limits - Acceptance of inert waste only | L | - Use of external pest control contractors - Immediate removal of offensive waste |  |
| Acceptance & Storage of waste outside terms of Permit | Potential for water, land and air pollution. Risk to human health | L | - Waste acceptance criteria - Visual inspection before and after tipping - Information and instruction for site users | L | - Quarantine area / container - Inform the Natural Resources wales if necessary - Disposal through a licensed contractor | - Waste acceptance procedure |
|  | PROCESSING |  |  |  |  |  |
| Use of plant & equipment for sorting / processing waste | Water and land pollution from leaks / spills (re-fuelling) | H | - Daily vehicle/plant checks - Preventive maintenance regime | L | - Spill kits - Trained spill responders - Silt traps | - Spill procedure - Daily plant/vehicle checklist - Vehicle/equipment maintenance schedules - Risk Assessments |
|  | Land contamination from puncture / tipping of vessels | M | - Mobile fuel bowser is double bunded - Storage locations of bowsers kept away from site traffic | L | - Spill kits - Trained spill responders |  |
|  | Loss of processing capacity through equipment failure | M | - Daily vehicle/plant checks - Preventative maintenance regime | L | - Financial reserves to replace integral equipment - Temporary lease of equipment - Insurance |  |
|  | Dust emissions & litter | M | - No processing during adverse weather conditions (windy) - Drop heights kept to a minimum - Regular site inspections | L | - Dust suppression systems - Immediate clean-up of litter |  |
|  | Odour | L | - Treatment within Permit time and quantity limits - Regular site inspections | L | - Immediate removal of offensive waste |  |
|  | Noise & Vibration | L | - Noise / Vibration assessments - Safe systems of work - Regular maintenance checks of equipment - Adhere to operating hours | L | - Personal Protective Equipment - Noise abatement measures - Information, Instruction, Training and Supervision |  |
| EMERGENCY |  |  |  |  |  |  |
| Spillage | Water and land pollution | H | - Regular site inspections - Supervision of fuel deliveries / re-fuelling - Plant and equipment re-fuelled in designated areas | L | - Drip trays / bunds - Spill kits - Trained spill responders - Disposal through licensed contractor - Silt Traps | - Spill procedure - Re-fuelling procedure |
| Fire | Air, land and water pollution | L | - Acceptance of inert materials only - Separation of combustible materials and ignition sources - No smoking policy - Good housekeeping - Daily vehicle/plant checks - Preventative maintenance regime - COSHH Assessments - Emergency drills | L | - Contact emergency services - Fire extinguishers - Fire plan - Insurance provision | - Fire plan - Weekly site checklist - Vehicle/equipment maintenance schedules |
| Flood | Land and water pollution | L | - Regular site inspections - Safe location for storage of hazardous materials | L | - Contact emergency services | - Weekly site checklist - Vehicle/equipment maintenance schedules |
| Vandalism | Land and water pollution | L | - Security guards monitoring access to Docks - Tanks and valves locked when not in use out of hours - Locked gates and perimeter fences | L | - Contact emergency services - Insurance | - Weekly site checklist |
| Major Accident | Serious injury / loss of life / major environmental pollution | H | - Risk assessments - Safe systems of work - Information, Instruction, Training and Supervision - Mentoring programmes - Regular equipment maintenance checks - Signs and signals - Personal Protective Equipment | L | - Accident / Incident reports and corrective actions - RIDDOR policy - Employers Liability Insurance - Information, Instruction, Training and Supervision | - Risk Assessments / Method Statements - RIDDOR Policy |
| Inclement Weather (heavy rainfall, snow/ice etc.) affecting our site and disposal outlets | Loss of processing capacity / loss of outlets for short durations | M | - Bulletins received and regular checks with the Met Office - Hold stocks of salt / grit for use on site - Regular maintenance of vehicles / machinery | L | - Site Closure | - Risk Assessments |
| Market Collapse | Loss of outlets / Cash flow issues | M | - Due diligence checks - Regular review of market trends - CPD - continuing competence, attendance at industry events, association registration, legal/market updates, industry knowledge. - Continual market and product research/new and future-proof. | L | - Use of alternative outlets | - Subscriptions - Risk Assessments |
| Loss of human resource | Loss of Technical competence | L | - Safe systems of work - Competitive salaries / benefits package - Equal opportunities policy - Information, Instruction, Training and Supervision | L | - Access to external CoTC holders - Sub-contractors / Agency workers - Overtime for existing employees | - Risk Assessments - Method Statements - Training Records |

# Appendix 3 – Weekly Site Checklist

| # WEEKLY (IF OPERATIONAL) ENVIRONMENTAL CHECKLIST |  |  |  |  |
| --- | --- | --- | --- | --- |
| # Date of Inspection: | # Inspection completed by: | # Location: |  |  |
| # Issue/Location Checked |  | # Yes | # No | # Comments/ Action Required/ Completion Date |
| Are there any signs of liquid spillages on site (e.g. diesel, chemicals, leachate)? |  |  |  |  |
| Are there any bunds/silt traps or drip trays that require pumping out / dispsoal? |  |  |  |  |
| Are drums and other storage containers stored on hardstanding? |  |  |  |  |
| Are storage areas locked and secure? |  |  |  |  |
| Are designated waste storage areas being used correctly? (e.g. hazardous and non-hazardous wastes not mixed) |  |  |  |  |
| Are flammable and non-flammable substances stored separately? |  |  |  |  |
| Are spill kits fully stocked? |  |  |  |  |
| Are there any skips on site that require emptying? |  |  |  |  |
| Are bins provided for oily wastes (i.e. used spill granules, used spill kits) being used appropriately? |  |  |  |  |
| How is the drainage on / near site? |  |  |  |  |
| Are dust/emission levels on site acceptable? |  |  |  |  |
| Are there any odours from any activities on site likely to cause nuisance beyond the site boundary? |  |  |  |  |
| Are noise levels on site likely to cause nuisance beyond the site boundary? |  |  |  |  |
| Are levels of housekeeping of an acceptable standard both internally and externally? |  |  |  |  |
| Are roadways clear of obstructions? |  |  |  |  |
| Are operational areas and site and access roads free of mud/debris? |  |  |  |  |
| Are defect sheets completed prior to operating the crusher? |  |  |  |  |
| Are all movements of waste / material being recorded? |  |  |  |  |
| Other Comments: Please use this box to record any other observations (positive or negative) not included on the list above |  |  |  |  |