Please note – this document is purely to assist you in making your decision and may not necessarily cover all information you require.

| Deployment Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-017258 |
| Permit Reference Number: | EPR/BB3498HA |
| Permit Holder: | Prichard Remediation Limited |
| Site Address: | Ex Virginia Park Golf Club, Virginia Park, Heol Bro Wen, Caerphilly, CF83 3SW |
| Grid Reference: | ST 15974 87798 |

| Administrative Checks- By admin officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: SE.waste.queries@cyfoethnaturiolcymru.gov.uk Environment team: ET.NewportCaerphilly&BlaenauGwent@cyfoethnaturiolcymru.gov.uk NLO: wasteregulation.southcentral@naturalresourceswales.gov.uk Land/groundwater remediation only: Geoscience: geoscience@cyfoethnaturiolcymru.gov.uk Local Authority Contaminated Land: crm@caerphilly.gov.uk |
| Date due: | 21/03/2022 |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- By determining officer |  |  |
| --- | --- | --- |
|  | Action completed or document provided? (Yes, No or n/a) | Comments |
| Correct fee? | Y | See permit compliance rating and multiplier |
| Original fee received and cleared | Y | Amount: 2,541.00 Date cleared on MyNRW: 03/03/2022 Transaction number: 2098698 Reference: BS/030322/10 TOM PRIC CON LTD VIRPAR0203 |
| MyMap report completed? | Y |  |
| Location map | Y | VPSP_V1.0- Virginia Park Remediation Site Plans |
| Site Specific Risk Assessment | Y | CSM Risk Assessment 230222. |

| # About your permit | Y N n/a | Comments |
| --- | --- | --- |
| # What permit is being deployed under? | # Bespoke permit for treatment using mobile plant | # This is incorrect as the permit is a Bespoke permit for land and groundwater remediation # Not included in RFI as it adds no value |
| # Has the permit number been given? | # Y |  |
| # Is the base permit still live? | # Y | Checked DMS/PALS |
| # Has there been any variations | # Y | # V002 to add treatments |
| # Has name and address of permit holder been provided? | # Y |  |
| # Has contact information been provided? | # Y |  |

| # Deployment details | Y N n/a | Comments |
| --- | --- | --- |
| # Have we been consulted on, and agreed to, the Conceptual Site Model/Risk Assessment/Remediation Strategy? | # Y | # Stated yes |
| # If yes, have references and contact information been provided? | # Y | # Alex Bowder- |
| # If yes, has there been any changes? | # N | # Stated No |
| # Document uploaded? | # N | # Emails have been saved onto the DMS by Alex Bowder # 02/02/22 from Prichards- attachments and queries # 09/02/22- NRW response # 24/02/22 from Prichards- CSM # 01/03/22- NRW response |
| # Does the applicant want this application to be assessed without having remediation targets pre-agreed by us? | # Y | # Stated yes |
| # Has there been any other pre-application advice? | # N | # Stated No, but there have been legal discussions before and after the refused deployment PAN-014297 # Not included in RFI as have the documents internally |
| # If yes, have references and contact information been provided? | # n/a |  |

| # About the site | Y N n/a | Comments |
| --- | --- | --- |
| # The site name, address, NGR given? | # Y |  |
| # Does the site address and 12 digit grid reference match? | # Y |  |
| # Is the site located within the boundary of another Environmental Permit? | # N |  |
| # If yes, please provide details below: | # n/a |  |
| # Site plan provided? | # Y | # VPSP_V1.0 |
| # Has there been an approved deployment in the same area within the last 12 months? | # N | # But we refused a deployment PAN-014297 |
| # Does it include the following? - Operating site boundary (clearly outlined in red) - Security and access arrangements (including additional details where alternative security, other than that specified in the licence is to be provided [for example containerised unit and buried pipework] - or where others are responsible for site security of a larger redevelopment scheme and you are relying on that security for your plant). - Areas of waste soils and contaminated material, substances or products for remediation by the mobile plant - Location/siting of principle plant and equipment - Process treatment and storage areas including any buildings, bays and fixed tanks, quarantine areas, areas of hardstanding, areas of impermeable pavement and sealed drainage – where it is not possible to clearly draw these, include text which confirms that all potentially polluting materials will be deposited, stored, treated or otherwise handled in an area of the site where an engineered containment and drainage system has been provided - Drainage systems for the operating site, including all interceptors, sumps and discharge points - Proposed location of boundary monitoring points and pollution control units (e.g. for noise, dust and VOCs emissions) - Potential receptors (for example, housing)– identify all land uses immediately surrounding the site on the site plan including any areas of public open space - Protected sites – only SACs, SPAs, Ramsar or SSSI sites within 1km of the operating site need to be identified on the site plan | # 1. Y # 2. Y # 3. Y # 4. query # 5. query # 6. query # 7. query # 8. query # 9. n/a | # 1 VPSP_V1.0 section 2.0 # 2. VPSP_V1.0 section 3 # 3. VPSP_V1.0 section 4. areas 1-3 are the where the waste is located (phase 1a, 1b and 2). # 4. VPSP_V1.0 section 4 states figure 4 shows plant locations but the area key does not list any areas as showing the location/siting of principal plant and equipment. SEE RFI # NOT RESOLVED 25/04/22 # RESOLVED IN RESPONSE 04/05/22 # 5. VPSP_V1.0 section 4. areas 1-3 are the where the waste is located (phase 1a, 1b and 2). # 4-7 are the treatment and storage. Is 8 the quarantine area? - SEE RFI # INFO PROVIDED IN RESPONSE 25/04/22 6. VPSP_V1.0 section 5. needs to show Drainage systems for the operating site, including all interceptors, sumps and discharge points but only shows areas where the “water storage locations” aka lagoons are SEE RFI # INFO PROVIDED IN RESPONSE 25/04/22 7. VPSP_V1.0 section 4- dust only. This should also include noise, odour and groundwater monitoring points - SEE RFI # INFO PROVIDED IN RESPONSE 25/04/22 8. VPSP_V1.0 sections 7 and 8 Section 8.13 refers to mobile plant permit SR2010 No11. This activity is being applied for under a bespoke permit for Bespoke permit for land and groundwater remediation. SEE RFI # RESOLVED IN RESPONSE 25/04/22 Reference to rule set has been removed, although section still lists location criteria from the rule set. We have not questioned this further as despite these limits not being listed within the permit held, they are considered good practice. 9. none in 1km |

| # Waste types and quantities | Y N n/a | Comments |
| --- | --- | --- |
| # Has the waste types, quantities etc and treatment types been provided? | # Y | # MPP2 form states 19 13 01* # 19 13 02 # Solid wastes from soil remediation. # These are the accepted waste codes- See letter from NRW to Blackfords LLP on 20/08/21. # Contaminants given as Removal of oversize fractions not suitable for engineering and Removal of small fractions of potentially hazardous waste # EMS section 2.5 and DMP section 5.5 lists different waste codes 17 05 04 (Soil and stones other than those mentioned in 17 05 03), 19 13 02 (Solid wastes from soil remediation other than those mentioned in 19 13 01) and 20 02 02 (Soil and stones) - SEE RFI- RESOLVED IN RESPONSE 25/04/22 # EMS section 4.1 is titles inert/non hazardous wastes. this is a very general section and not specific to the wastes included within this deployment which include a hazardous waste code- SEE RFI- RESOLVED IN RESPONSE 25/04/22 # Total of 40,020 cubic meters of material is to be treated. Information is needed on how these amounts were calculated, and how the amount of material being processed/treated is being tracked- SEE RFI- RESOLVED IN RESPONSE 25/04/22 |
| # Are the waste types and quantities from specified in the operator’s permit? | # n/a | # Permit does not contain list of waste codes |
| # Has the document been uploaded? | # N |  |

| # Specified activities to be carried out on site | Y N n/a | Comments |
| --- | --- | --- |
| # Has details on suitability of treatment methods been provided? | # N | # Reference made to an approved Remediation Strategy- we do not have this, nor do we know who approved it- SEE RFI- RESOLVED IN RESPONSE 25/04/22 # States info is in the EMS, CEMP, and emission monitoring documents- this info is not included- SEE RFI - RESOLVED IN RESPONSE 25/04/22 |
| # Has the operator told us which treatment technology they will use? | # Y | # Screening, sorting raking, separating # EMS also lists Shredders / Crushers- SEE RFI- RESOLVED IN RESPONSE 25/04/22 |
| # Are all of the proposed treatment technologies listed in the operator’s permit? | # Y | # Only sorting and separation directly, however screening and raking, could be types of sorting and separation. # EMS section 4.2 also included shredders and crushers- these are not permitted under the permit held- SEE RFI - RESOLVED IN RESPONSE 25/04/22 |
| # Is the treatment technology fit for purpose? | # Y |  |
| # Has details on residual materials/waste been provided? | # N | # States that residual waste will be isolated/ quarantined but doesn’t say how they will be identified or stored- SEE RFI - RESOLVED IN RESPONSE 25/04/22 |
| # Are we satisfied that these will be handled appropriately? | # N | # Do not have adequate info to assess. See above RFI # RESOLVED IN RESPONSE 25/04/22 |
| # Has details on quarantine facilities been provided? | # Y | # Details and quantities given, but info on containment measures has not been provided- SEE RFI- RESOLVED IN RESPONSE 25/04/22 |
| # Are we satisfied the maximum capacities of the quarantine facilities are suitable to prevent or minimise pollution? | # N | # Do not have adequate info to assess. See above RFI # RESOLVED IN RESPONSE 25/04/22 |
| # Have the locations of the quarantine facilities been included on the site plan? | # Y | # Although not explicitly- SEE RFI # RESOLVED IN RESPONSE 25/04/22 |
| # Has the document been uploaded? | # Y | # PR_EMS_V1.3 010322 |

| # Activities involving the import of waste | Y N n/a | Comments |
| --- | --- | --- |
| # Will the activity involve the import of wastes? | # N |  |
| # Does the site form part of a Cluster project? * | # N |  |
| # Has waste acceptance information been provided? | # Y | # See EMS section 2.1 |
| # Has the document been uploaded? | # Y | # PR_EMS_V1.3 010322.pdf # CEMP Virginia Park v5.pdf |

| # Duration of this deployment | Y N n/a | Comments |
| --- | --- | --- |
| # Is the duration of the deployment 12 months or less? | # Y |  |

| # Management supervision | Y N n/a | Comments |
| --- | --- | --- |
| # Has the technically competent manager details been provided? | # Y | # Mr George Harvey |
| # Has the nominated competent person details been provided? | # Y | # Mr George Harvey |
| # Has the site supervision plan been provided? | # Y | Not clear who is carryout out the site supervision or what the minimum site time is (minimum attendance is 1 hr/week)- SEE RFI - RESOLVED IN RESPONSE 25/04/22 # States EMS details the visits and inspections to be undertaken- it is not clear where this info is- SEE RFI -RESOLVED IN RESPONSE 25/04/22 |
| # Is the site supervision plan appropriate? | # N | # Do not have adequate info to assess. See above RFI # RESOLVED IN RESPONSE 25/04/22 |
| # Has the document been uploaded? | # Y | # G. Harvey MROC13 Registration given, but EMS Section 1.1 stated Mr Harvey has completed the qualification, so we need the certificate to confirm- SEE RFI -RESOLVED IN RESPONSE 25/04/22 |

| # Conceptual site model and risk assessment | Y N n/a | Comments |
| --- | --- | --- |
| # Has the conceptual site model (CSM)/risk assessment been provided? | # Y | # Included in RFI - wording from the GRA used - hazardous wastes not assessed - odour not sufficiently addressed & OMP is required - stated pests not likely- asked to provide more info - flooding information incorrect - errors in risk magnitude (line 30) - states risk from contaminated run off not addressed - included wording on SSSIs and European site where there are none - not identified any actual receptors - missing all parts of the process as set out in the form guidance # NOT ALL ISSUES RESOLVED- SEE 2ND RFI # RESOLVED IN RESPONSE 04/05/22 |
| # Has the document been uploaded? | # Y | # CSM Risk Assessment 230222.xlsx |

| # Pollution control | Y N n/a | Comments |
| --- | --- | --- |
| # Have the pollution control measures been provided? | # N | # No Odour Management Plan has been provided- as the source of the waste is a former landfill an OMP is required- SEE RFI - RESOLVED IN RESPONSE 25/04/22 # Queries on both the DMP and NMP- SEE RFI- no updated NMP was provided. not all requested changes have been made to DMP # RESOLVED IN RESPONSE 04/05/22 # Infrastructure issues- states we agreed hardstanding was ok, but I have no evidence of this- SEE RFI- info provided, we can accept hardstanding is reasonable- RESOLVED IN RESPONSE 25/04/22 # Note- Email of 10/02 provided however this does not approve the treatment of waste on hardstanding # Surface water and groundwater queries adequately resolved. |
| # Has the document been uploaded? | # Y | # DMP VP 010322 v1.pdf # NMP VP 010322 v1.pdf |

| # Emission monitoring plans | Y N n/a | Comments |
| --- | --- | --- |
| # Has the site specific monitoring plan been provided? # The following may be applicable: - Groundwater - Surface water - Soil gases - Air emissions - Noise - Vibrations - Odour - Volatile Organic Compounds | # N | # - Groundwater # - Air emissions # - Noise # Not # - Surface water # - Soil gases # - Vibrations # - Odour # - Volatile Organic Compounds # RFI for missing EMP (odour) and justification for the rest- SEE RFI # OMP and justifications provided- RESOLVED IN RESPONSE 25/04/22 # Queries on groundwater EMP, dust and noise- see above |
| # Have the indicator parameters been provided? | # N | # See above # Quoted Noise and Dust management and groundwater scheme of monitoring. |
| # Has the document been uploaded? | # Y | # 12476 Virginia Park Groundwater # Scheme of Monitoring.pdf # Discharge of Pre Commencement Conditions by LPA.pdf |

| # Record Keeping - Commissioning, operating and maintenance | Y N n/a | Comments |
| --- | --- | --- |
| # Details of record keeping given? | # Y | # See EMS and site inspection checklist # Does not include the info required- details of commissioning, operating and maintenance procedures – SEE RFI- RESOLVED IN RESPONSE 25/04/22 |
| # Document uploaded? | # Y | # Site inspection checklist VP.pdf |

| # Supporting documents | Y N n/a |
| --- | --- |
| Have relevant documents been ticked and supplied? - Site plan showing all required features - Conceptual Site Model and Risk Assessment - Pollution Control - Monitoring Plans - Record Keeping details | # Yes, to all: - 12476 Virginia Park Groundwater Scheme of monitoring - CEMP Virginia Park v5 - CSM Risk Assessment 230222. - Discharge of Pre Commencement Conditions by LPA. - DMP VP 010322 v1. - G. Harvey MROC13 Registration.jpg Download - NMP VP 010322 v1. - PR_EMS_V1.3 010322 - Site inspection checklist VP. - VPSP_V1.0l |

| # Confidentiality and national security | Y N n/a | Comments |
| --- | --- | --- |
| # Has confidentially been claimed? | # N |  |
| # Has national security been claimed? | # N |  |

| # Declaration | Y N n/a | Comments |
| --- | --- | --- |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | # n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | # n/a | # bespoke permit |
| # Has a relevant person or approved person signed the declaration? | # Y |  |

| Site specific risk assessment | Y N n/a | Comments |
| --- | --- | --- |
| If there are potential sensitive receptors has a site specific risk assessment been submitted? | Y | CSM/Risk Assessment |
| Does the site specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards | N | See RFI for CSM/Risk assessment |
| - Pathways |  |  |
| - Receptors that can be affected by that source of pollution |  |  |
| - Measures that will minimise or mitigate the impact will be put in place |  |  |

| SSSI/ Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 1 km of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a “likely significant effect” been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as “information only” | n/a |  |

| Natura 2000 or Ramsar site/ Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 1 km of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | No |

Comments

Consultation responses: Response received from: - NRW area team- query re drainage added to RFI- RESOLVED IN RESPONSE 25/04/22 - NRW geoscience team- query re updating of documents and treating of contaminated water added to the RFI- RESOLVED IN RESPONSE 25/04/22, confirmed with Geoscience team 04/05/22 - Local authority contaminated land team- highlighted issues with residual waste- these had also been identified in NRWs assessment and added to RFI. Also query on the sheeting of materials leaving the site- added to RFI RESOLVED IN RESPONSE 25/04/22

MyMap and location assessment: See: - Appendix 2 – Location Checks - PAN-017258 Risk Screening Report (DMS) - PAN-017258 MyMap Report (DMS) # Our searches have found the following within 1km of the proposed site: - Ancient woodland to the North and South East of the site - River Rhymney Local Wildlife Site - Nant yr Aber River Local Wildlife Site - Coed y Maerdy, East of Caerphilly Local Wildlife Site - Caerphilly/ Machen Disused Railway, East of Trethomas Local Wildlife Site - Caerphilly Castle scheduled ancient monument - Watercourse to the south of the site- Porset Brook flowing to Rhymney River - Himalyan Balsam # And the following within the proposed site boundary: - Watercourses and ponds - Section 7 Priority habitats - Aquifer Designation (Bedrock)- GROVESEND FORMATION Secondary A - Aquifer Designation (Superficial)- TILL, DEVENSIAN Secondary 0 (undifferentiated) - Himalyan Balsam

Request for information: - 1st email sent 14/04/22- LINK - response received. See DMS for documents received on date 18/04/22, 19/04/22 and 25/04/22 - 2nd email sent 03/05/22- LINK - response received 04/05/22- See DMS for documents received on this date

Documents (crossed out documents were superseded): Application documents - MPP2 form - 12476 Virginia Park Groundwater Scheme of monitoring - CEMP Virginia Park v5 - CSM Risk Assessment 230222. - Discharge of Pre Commencement Conditions by LPA. - DMP VP 010322 v1. - G. Harvey MROC13 Registration.jpg Download - NMP VP 010322 v1. - PR_EMS_V1.3 010322 - Site inspection checklist VP. - VPSP_V1.0 1st RFI response - email- response - email- groundwater - 12476 Virginia Park Interim Groundwater Report - Feb 2022 - Construction Environmental Management Plan - CSM Risk Assessment - Discharge of Pre Commencement Conditions by LPA - DMP VP 140422 v1.1 - G. Harvey MROC13 Certificates - NRW correspondence 10 March 2022 - NRW Correspondence 23 December 2020 - NRW Correspondence 30 December 2020 - OMP 140422 - PR_EMS_V1.3 140422-Final - Site inspection checklist VP - Tracking spreadsheet_DT April 2021 - VP Remediation Strategy - VPSP_V1.1 2nd RFI response - email response - CSM_RA_V2.2_03052022 - PR_EMS_V1.4 - VP_CEMP_V7.1_03052022 - VP_DMP_V1.2 - VP_NMP_V1.1 - VPSP_V1.2

Ops handover: - This determination makes no comment on the excavation or deposition of the waste - reference has been made to dewatering (PAN-017258 RFI Response NRW Correspondence 30 December 2020) but this was not included in this application, therefore we have made no queries about this under this deployment application. - the local authority has told us that crushing of waste may be taking place, however this was not applied for, and therefore has not been assessed - the treatment of this waste under this deployment (sorting/separation, screening/raking) should not affect groundwater

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Permitting Officer: Louise Bailey Date: 04/05/2022 |  |
| ## Team leader or ToP approval: Huw Davies Date: 04/05/2022 |  |

| Appendix 1 – Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1890 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 1910 minutes |

| Appendix 2 – Location Checks |  |  |
| --- | --- | --- |
| Designations | Present? | Comments |
| Special protection areas (SPA) Potential special protection areas (pSPA) | N |  |
| Special areas of conservation (SAC) Candidate special areas of conservation (cSAC) | N |  |
| Ramsar sites | N |  |
| Site of Special Scientific Interest (SSSI) | N |  |
| Ancient woodland | Y | To the North and South East of the site |
| Areas of Outstanding Natural Beauty (AONB) | N |  |
| Heritage Coast | N |  |
| Local nature reserves | N |  |
| Local wildlife sites | Y | River Rhymney Nant yr Aber River Coed y Maerdy, East of Caerphilly Caerphilly/ Machen Disused Railway, East of Trethomas |
| National nature reserves | N |  |
| National parks | N |  |
| Scheduled ancient monuments | Y | Caerphilly Castle |
| World Heritage Sites | N |  |
| Controlled waters such as rivers and lakes | Y | Watercourse on site (on old golf course) and surrounding edge of site Water course to the south of the site- Porset Brook flowing to Rhymney River |
| European Protected Species | N |  |
| Nationally Protected Species | N |  |
| Section 7 Priority species | N |  |
| Section 7 Priority habitats | Y | Purple moor grass and rush pastures on site- now destroyed |
| Other | Y | Site on Aquifer Designation (Bedrock)- GROVESEND FORMATION Secondary A Site on Aquifer Designation (Superficial)- TILL, DEVENSIAN Secondary 0 (undifferentiated) Within 1km of the site: Himalyan Balsam |