Permit Number: 	PAN-005141			                      Operator: 		Newbridge Energy Limited

Facility:		Blazers Fuels Ruthin			Form Number: 	Air1 / 11/05/2022

# Reporting of emissions to air for the period from DD/MM/YYYY to DD/MM/YYYY

| Emission Point | Substance / Parameter | Emission Limit Value* | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 NGR: SJ 11634 59010 | Dark smoke | No visible dark smoke | Daily when in operation |  | Methodology as approved in accordance with pre-operational measure PO1 |  |  |
|  | Oxides of nitrogen (NO and NO2 expressed as NO2) | 475 mg/Nm3 | Periodic (average over one hour) |  | MCERTS BS EN 14792 |  |  |
|  | Dust | 50 mg/Nm3 |  |  | MCERTS BS EN 13284-1 |  |  |
|  | Carbon Monoxide | 225 mg/Nm3 |  |  | MCERTS BS EN 15058 |  |  |
|  | TVOC | 30 mg/Nm3 |  |  | MCERTS BS EN 12619 |  |  |
| A2 NGR: SJ 11602 59022 | Dark smoke | No visible dark smoke | Daily when in operation |  | Ringlemann Chart Shade 1 |  |  |
|  | Oxides of nitrogen (NO and NO2 expressed as NO2) | 300 mg/Nm3 | Periodic (average over one hour) |  | MCERTS BS EN 14792 |  |  |
|  | Dust | 30 mg/Nm3 |  |  | MCERTS BS EN 13284-1 |  |  |
|  | Carbon Monoxide | 225 mg/Nm3 |  |  | MCERTS BS EN 15058 |  |  |
|  | TVOC | 30 mg/Nm3 |  |  | MCERTS BS EN 12619 |  |  |

* At reference conditions of a temperature of 273.15 K, a pressure of 101.3 kPa and after correction for the water vapour content of the waste gases at a standardised O2 content of 6 % for solid fuels

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date………………………..

(Authorised to sign as representative of Operator)